Thirty-Seventh Meeting of the

Informal South Pacific ATS Co-ordinating Group

(ISPACG/37)

Thirtieth Meeting of the FANS Interoperability Team (FIT/30)

Papeete, Tahiti

26 April 2023

Agenda Item 9: Information Papers

Collins SATVOICE Implementation

Presented by Jumper Leggio

SUMMARY

1.	INTRODUCTION

  - Collins Aerospace has been implementing a closed-loop Voice over IP network to support implementation of SatVoice one-stage dialing in U.S. Airspace for Air Traffic Services

  - The network is intentionally diverse from the Collins datalink infrastructure to ensure diversity from FANS

  - The system uses the Session Initiation Protocol (SIP) for call setup and telephony signaling, a widely used telephony protocol but configured to include octal and priority call elements

2.	DISCUSSION

  - Initial results – lab testing

    - Audio latency reduced about 10% from 0.89 sec to 0.80 sec

        - Observed during testing w/Inmarsat.  Iridium testing is forthcoming.

    - GtA call setup time reduced from 20 sec to < 1 sec

        - PSTN setup was observed from 2019 Pacific Voice Trials

2.2   (Show one-stage dialling-current state diagram)

2.3  (Show one-stage dialling – future state diagram)

2.4.   Phase I Trials

a) FEDEX & UNITED over INMARSAT callback checks

- Iridium equipped aircraft to be added in coming weeks

b) Call back checks only, no ATC

c) Test short codes map to VoIP network

- Live short codes continue routing through PSTN

d) Objective is to exercise the system and collect hundreds or thousands of calls

e) Single server at Collins core, single network connections to core notes

f)  Success criteria to move to Phase II

- No unresolved trouble tickets

- Number of one-stage calls (AtG, GtA, Iridium/Inmarsat)

2.5  Phase II Trial (Under Development)

a)  PSTN setup was observed from 2019 Pacific Voice Trials

    - Callback check in conjunction with SELCAL check and HF frequency assignment

    - Use test short code or move operation short code to SIP

    - Success criteria to move to go-live

      - Collins has redundant SIP services to Annapolis core.

      - All core notes have redundant network connections.

      - No unresolved trouble tickets.

      - Number of calls

3. 	CONCLUSION

  - The meeting is invited to note the information provided.

-- END --