PBCS Performance Analysis NZZO – June 2020

In June ADS-C data points were 18,859 and CPDLC 435. The current data points available for analysis remain low compared with pre-pandemic traffic levels. This month’s analysis reviewed both the June, and the consolidated April/May/June data and evaluated performance where data points exceeded 100.

PBCS Performance Summary

Previous analysis update:

Boeing CRA advised 8th July reference PR ACNZ_2020_04 raised 16th March, that the Air Canada B788 SATCOM AMI (ORT) is correctly configured and they are continuing to investigate. This fleet did not operate in NZZO in June.

Air New Zealand B77W – PR ACNZ_2020_07 raised 8 June to investigate delays. The delays may be caused by de-activated MTSAT satellite remaining in ORT.

Air New Zealand B789 – PR ACNZ_2020_06 raised 8 June to investigate delays. The delays may be caused by de-activated MTSAT satellite remaining in ORT.

Tasman Cargo B763 – VHEXZ. Performance returned to normal in June. Aircraft was not correctly registered with CSP. June RSP180 performance as follows:

| Operator | Aircraft Type | Tail No | ADS-C downlink Message counts | ASP <= 90 sec | ASP <= 180 sec |
| --- | --- | --- | --- | --- | --- |
| TMN | B763 | VHEXZ | 499 | 98.00% | 99.40% |

June 2020 Analysis

Air New Zealand B77W

| Operator | Aircraft Type | Tail No | ADS-C downlink Message counts | ASP <= 90 sec | ASP <= 180 sec |
| --- | --- | --- | --- | --- | --- |
| ANZ | B77W | ZKOKM | 753 | 99.20% | 99.60% |
| ANZ | B77W | ZKOKN | 842 | 97.98% | 98.46% |
| ANZ | B77W | ZKOKO | 491 | 97.15% | 98.37% |
| ANZ | B77W | ZKOKP | 872 | 97.94% | 98.74% |
| ANZ | B77W | ZKOKQ | 410 | 99.02% | 99.51% |
| ANZ | B77W | ZKOKR | 151 | 97.35% | 98.68% |
| ANZ | B77W | ZKOKS | 261 | 100.00% | 100.00% |

Slight degradation below RSP180 99.0% at 180 seconds for some tails continued through June. Airline has advised that delays may be related to the de-activated MTSAT satellite remaining in the aircraft ORT. Airline is working this and CRA continues to investigate PR ACNZ_2020_07.

No significant operational impact. FANS PR under investigation by CRA.

Air New Zealand B789 – ZKNZG

| Operator | Aircraft Type | Tail No | ADS-C downlink Message counts | ASP <= 90 sec | ASP <= 180 sec |
| --- | --- | --- | --- | --- | --- |
| ANZ | B789 | ZKNZE | 149 | 98.66% | 100.00% |
| ANZ | B789 | ZKNZF | 299 | 99.67% | 100.00% |
| ANZ | B789 | ZKNZG | 151 | 98.68% | 98.68% |
| ANZ | B789 | ZKNZH | 163 | 99.39% | 100.00% |
| ANZ | B789 | ZKNZJ | 101 | 100.00% | 100.00% |
| ANZ | B789 | ZKNZK | 117 | 100.00% | 100.00% |
| ANZ | B789 | ZKNZL | 392 | 100.00% | 100.00% |
| ANZ | B789 | ZKNZM | 382 | 100.00% | 100.00% |
| ANZ | B789 | ZKNZN | 252 | 100.00% | 100.00% |
| ANZ | B789 | ZKNZQ | 500 | 100.00% | 100.00% |
| ANZ | B789 | ZKNZR | 356 | 100.00% | 100.00% |

Slight degradation below RSP180 99.0% at 180 seconds for NZG in June while NZE reported last month for same slight degradation is now OK. Airline advised that delays may be related to de-activated MTSAT satellite remaining in aircraft ORT. Airline is working this and CRA continues to investigate PR ACNZ_2020_06.

No significant operational impact. FANS PR under investigation by CRA.

Air New Zealand A20N – ZKNHD

| Operator | Aircraft Type | Tail No | ADS-C downlink Message counts | ASP <= 90 sec | ASP <= 180 sec |
| --- | --- | --- | --- | --- | --- |
| ANZ | A20N | ZKNHD | 321 | 95.33% | 98.75% |

| Date | RGS | REP_TYPE | Latitude | Longitude | AC_time | OCS_time | Downlink_time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20200609 | IGW1 | P | -19.4163 | -170.189 | 02:29:28 | 02:32:44 | 196 |
| 20200615 | IGW1 | P | -29.0388 | 167.9434 | 03:28:08 | 03:28:13 | 5 |
| 20200615 | IGW1 | W | -29.0262 | 167.9083 | 03:38:46 | 03:43:31 | 285 |
| 20200615 | IGW1 | P | -29.1084 | 167.7379 | 03:41:13 | 03:44:25 | 192 |
| 20200615 | IGW1 | P | -29.1462 | 167.617 | 03:42:29 | 03:46:31 | 242 |
| 20200615 | IGW1 | A | -29.4993 | 166.4683 | 03:52:30 | 03:52:56 | 26 |

Slight degradation below 99.0% at 180 seconds was caused by 3 delayed reports on 15th June departing YSNF towards NFFF at 29S167E. Initial delayed report was a waypoint change followed within 2 minutes by two periodic reports. Suspect that NFFF would have been establishing datalink which may explain delays. Aircraft was meeting RSP180 performance in previous analysis. This occurrence identical to that seen with ZKNHB on 25th April and reported in the April 2020 analysis.

No operational impact. No further action.

Cathay Pacific B77W – fleet performance

CPA B77W fleet performance shows below RSP180 requirements in the 3-month consolidated data from April-June. This fleet is operating on Iridium SATCOM via IG1 RGS.

| Operator/ Aircraft Type | ADS-C downlink Message counts | ASP <= 90 sec | ASP <= 180 sec |
| --- | --- | --- | --- |
| CPA/B77W | 264 | 92.80% | 97.73% |

Fleet did not operate in May and the available data points for individual tails in April and June are not sufficient for an accurate assessment.  While tails BKPV and BKQG show below RSP180 requirements, with only 22 data points this is not considered valid.

| Operator | Aircraft Type | Tail No | ADS-C downlink Message counts | ASP <= 90 sec | ASP <= 180 sec |
| --- | --- | --- | --- | --- | --- |
| CPA | B77W | BKPE | 33 | 100.00% | 100.00% |
| CPA | B77W | BKPO | 40 | 95.00% | 100.00% |
| CPA | B77W | BKPT | 22 | 95.45% | 100.00% |
| CPA | B77W | BKPV | 22 | 77.27% | 95.45% |
| CPA | B77W | BKPX | 24 | 95.83% | 100.00% |
| CPA | B77W | BKQG | 22 | 72.73% | 81.82% |
| CPA | B77W | BKQK | 10 | 100.00% | 100.00% |

The actual data points for BKPV and BKQG are shown below.

BKPV June data points

| Date | RGS | REP_TYPE | Latitude | Longitude | AC_time | OCS_time | Downlink_time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20200607 | IG1 | P | -28.5718 | 160.5041 | 21:23:54 | 21:24:44 | 50 |
| 20200607 | IG1 | P | -30.1762 | 162.5807 | 21:38:50 | 21:39:19 | 29 |
| 20200607 | IG1 | W | -30.4898 | 163.0054 | 21:41:49 | 21:42:18 | 29 |
| 20200607 | IG1 | P | -31.5483 | 164.9314 | 21:53:47 | 21:54:01 | 14 |
| 20200607 | IG1 | W | -32.4759 | 166.7086 | 22:04:35 | 22:05:28 | 53 |
| 20200607 | IG1 | P | -32.8241 | 167.3978 | 22:08:43 | 22:08:56 | 13 |
| 20200607 | IG1 | P | -34.0499 | 169.9343 | 22:23:39 | 22:24:49 | 70 |
| 20200607 | IG1 | P | -34.111 | 170.0661 | 2:24:25 | 22:25:59 | 94 |
| 20200607 | AKL | P | -34.3062 | 170.4886 | 2:26:52 | 22:27:04 | 12 |
| 20200607 | AKL | W | -34.8517 | 171.6823 | 2:33:44 | 22:33:47 | 3 |
| 20200607 | AKL | W | -35.0793 | 171.9801 | 2:35:53 | 22:35:55 | 2 |
| 20200607 | IG1 | P | -35.3701 | 172.3516 | 2:38:36 | 22:38:48 | 12 |
| 20200609 | HLZ | P | -36.752 | 174.2107 | 1:49:53 | 01:50:01 | 8 |
| 20200609 | AKL | P | -35.4328 | 172.4421 | 2:04:50 | 02:04:53 | 3 |
| 20200609 | AKL | W | -34.8426 | 171.6803 | 2:11:17 | 02:11:28 | 11 |
| 20200609 | IG1 | P | -34.0492 | 170.7047 | 2:19:46 | 02:22:53 | 187 |
| 20200609 | IG1 | P | -32.6419 | 169.0405 | 2:34:42 | 02:34:52 | 10 |
| 20200609 | IG1 | P | -31.305 | 167.5293 | 2:49:38 | 02:49:56 | 18 |
| 20200609 | IG1 | P | -30.0007 | 166.114 | 3:04:35 | 03:06:15 | 100 |
| 20200609 | IG1 | P | -29.9683 | 166.0796 | 3:04:57 | 03:07:48 | 171 |
| 20200609 | IG1 | W | -29.1436 | 165.2127 | 3:14:16 | 03:14:32 | 16 |
| 20200609 | IG1 | P | -28.7025 | 164.7179 | 03:19:31 | 03:21:51 | 140 |

Only delay exceeding 180 seconds associated with a VDL-SATCOM transition on 9th June.

BKQG June data points

| Date | RGS | REP_TYPE | Latitude | Longitude | AC_time | OCS_time | Downlink_time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20200614 | IG1 | P | -30.6743 | 159.8566 | 21:28:34 | 21:28:55 | 21 |
| 20200614 | IG1 | P | -31.9031 | 162.3649 | 21:43:30 | 21:44:22 | 52 |
| 20200614 | IG1 | W | -32.204 | 163.0057 | 21:47:18 | 21:47:52 | 34 |
| 20200614 | IG1 | P | -33.0481 | 164.8655 | 21:58:26 | 21:58:43 | 17 |
| 20200614 | IG1 | P | -34.0825 | 167.2862 | 22:13:23 | 22:13:35 | 12 |
| 20200614 | IG1 | A | -34.9425 | 169.4394 | 22:26:45 | 22:26:57 | 12 |
| 20200614 | IG1 | P | -34.9836 | 169.5457 | 22:27:25 | 22:28:15 | 50 |
| 20200614 | IG1 | A | -35.0133 | 169.6236 | 22:27:54 | 22:30:08 | 134 |
| 20200614 | IG1 | P | -35.0388 | 169.6902 | 22:28:19 | 22:37:10 | 531 |
| 20200614 | IG1 | A | -35.2716 | 170.3025 | 22:32:10 | 22:38:56 | 406 |
| 20200614 | IG1 | PW | -35.315 | 170.418 | 22:32:54 | 22:40:48 | 474 |
| 20200616 | HLZ | P | -36.7755 | 174.5137 | 01:56:17 | 01:56:20 | 3 |
| 20200616 | HLZ | W | -36.6995 | 174.3142 | 01:57:55 | 01:57:58 | 3 |
| 20200616 | AKL | W | -36.1807 | 173.7814 | 02:03:29 | 02:03:32 | 3 |
| 20200616 | AKL | P | -35.5804 | 172.8256 | 02:11:13 | 02:11:16 | 3 |
| 20200616 | AKL | W | -34.8418 | 171.6813 | 02:20:25 | 02:20:34 | 9 |
| 20200616 | AKL | P | -34.3228 | 171.0381 | 02:26:09 | 02:26:15 | 6 |
| 20200616 | IG1 | P | -32.9648 | 169.4152 | 02:41:06 | 02:41:15 | 9 |
| 20200616 | IG1 | P | -31.6741 | 167.9401 | 02:56:02 | 02:56:14 | 12 |
| 20200616 | IG1 | P | -30.449 | 166.5943 | 03:10:58 | 03:11:17 | 19 |
| 20200616 | IG1 | P | -29.7074 | 165.8031 | 03:20:02 | 03:25:57 | 355 |
| 20200616 | IG1 | PW | -29.2204 | 165.2926 | 03:25:54 | 03:28:11 | 137 |

Four delays between 2227-2232 on 14 June, and two delays between 0320-0325 on 16 June.

Intention is to continue monitoring over next few months and also see if additional data points are available from Oakland and Nadi for this fleet.

Minimal operational impact. Continue monitoring and access Fiji and Oakland data for further analysis.