Planning Team Meeting

(PT/33)

Queenstown, NEW ZEALAND

18 – 22 March 2019

Agenda Item 5: Working Papers

Proposed Doc7030 amendment for ADS-B ITP and ADS-C CDP and RNP2 20NM Lateral separation

Presented by Airways New Zealand

SUMMARY

This paper proposes amendments to Doc7030 Regional Supplementary Procedures to incorporate ADS-B In-Trail Procedure (ITP), ADS-C Climb Descend Procedure (CDP) and the RNP2/GNSS 20NM lateral separation standard.

1.	INTRODUCTION

1.1	As discussed at previous meetings ICAO regional office secretariat have advised all separation standards in use over the high seas were required to be documented in the Doc7030 Regional Supplementary Procedures.

2.	DISCUSSION

2.1	Airways New Zealand took an action to develop amendments to the Doc 7030 to incorporate separation standards that were either in use or proposed for use in the region. The intention is to agree these changes as a group and then individual ISPACG states can propose the amendments to ICAO.

2.2	This paper proposes text in Attachment A and B to amend the PAC and MID/ASIA Regional Supplementary procedures to incorporate the 15NM ADS-B ITP and ADS-C CDP longitudinal standards	, and the 20NM RNP2/GNSS climb through separation in PANS-ATM.

2.3 	The paper also proposes consequential changes to capture editorial modifications in the 2018 Doc 4444 amendment.

2.4	We note that the 30NM/27NM RNP4 standard is captured in the current Doc7030 that was amended for PBCS implementation.

3.	ACTION BY THE MEETING

3.1	The meeting is invited to:

a)	Discuss the proposed amendment and agree a joint draft.

b)	Present the agreed draft to ISPACG for agreement.

Attachment A: PT WP NZ1 – PAC – Supplementary –Procedures-Doc-7030_final 20190403

Attachment B: PT WP NZ1 – MID-ASIA – Supplementary –Procedures-Doc-7030_final 20190403

ATTACHMENT – (A)

DRAFT

PROPOSAL FOR AMENDMENT OF THE 
REGIONAL SUPPLEMENTARY PROCEDURES 
PACIFIC (PAC) REGION (Doc. 7030/5)

- Regional Supplementary Procedures:

PAC

- Proposed by:

ISPACG

- Proposed amendment:

Editorial Note: Amendments are arranged to show deleted text using strikeout (text to be deleted), and added text with grey shading (text to be inserted).

Glossary

…

CDP	Climb and descent procedure

ITP 	In-trail procedure

…

Chapter 6.	AIR TRAFFIC SERVICES

…

6.2	SEPARATION

6.2.1	Lateral

…

6.2.1.3 		The minimum lateral separation shall be 42.6 km (23 NM) between aircraft on designated controlled oceanic routes or areas within the Anchorage Oeanic, Auckland Oceanic, Nadi, Oakland Oceanic and Tahiti FIRs. This minimum is applied in accordance with 5.4.1.2.1.6 b) and Table 5-2 of PANS-ATM and the following:

…

…

6.2.1.4 	The minimum lateral separation shall be 37 km (20 NM) between aircraft on designated controlled oceanic routes or areas within the Anchorage Oceanic, Auckland Oceanic, Nadi, Oakland Oceanic and Tahiti FIRs. This minimum is applied in accordance with 5.4.1.2.1.6 and Table 5-2 of PANS-ATM and provided the following conditions are met:

a) 	navigation – RNP 2 per para. 4.1.2 or GNSS;

…

6.2.2	Longitudinal

(P-ATM – Chapters 5 and 13)

...

6.2.2.3 		The minimum longitudinal separation using ADS-B in-trail procedure (ITP) within the Anchorage Oceanic, Auckland Oceanic, Nadi, Oakland Oceanic and Tahiti FIRs shall be 28 km (15 NM). This minimum is applied in accordance with 5.4.2.7 of PANS-ATM.

6.2.2.4 		The minimum longitudinal separation using ADS-C climb and descend procedure (CDP) within the Anchorage Oceanic, Auckland Oceanic, Nadi, Oakland Oceanic and Tahiti FIRs shall be 28 km (15 NM). This minimum is applied in accordance with 5.4.2.8 of PANS-ATM.

-END-

ATTACMENT (B)

DRAFT

PROPOSAL FOR AMENDMENT OF THE 
REGIONAL SUPPLEMENTARY PROCEDURES 
PACIFIC (PAC) REGION (Doc. 7030/5)

- Regional Supplementary Procedures:

PAC

- Proposed by:

ISPACG

- Proposed amendment:

Editorial Note: Amendments are arranged to show deleted text using strikeout (text to be deleted), and added text with grey shading (text to be inserted).

Glossary

…

CDP	Climb and descent procedure

ITP 	In-trail procedure

…

Chapter 6.	AIR TRAFFIC SERVICES

…

6.2	SEPARATION

6.2.1	Lateral

…

6.2.1.4 		The minimum lateral separation shall be 42.6 km (23 NM) between aircraft on designated controlled oceanic routes or areas within the Auckland Oceanic, Brisbane, Fukuoka, Honiara, Melbourne, Nauru, New Zealand and Port Moresby FIRs. This minimum is applied in accordance with 5.4.1.2.1.6 b) and Table 5-2 of PANS-ATM and provided the following conditions are met:

…

6.2.1.5 	The minimum lateral separation shall be 37 km (20 NM) between aircraft on designated controlled oceanic routes or areas within the Auckland Oceanic, Brisbane, Fukuoka, Honiara, Melbourne, Nauru, New Zealand and Port Moresby FIRs. This minimum is applied in accordance with 5.4.1.2.1.6 and Table 5-2 of PANS-ATM and provided the following conditions are met:

a) 	navigation – RNP 2 per para. 4.1.2 or GNSS;

…

6.2.2	Longitudinal

(P-ATM – Chapters 5 and 13)

...

6.2.2.6		The minimum longitudinal separation using ADS-B in-trail procedure (ITP) within the Auckland Oceanic, Brisbane, Fukuoka, Honiara, Melbourne, Nauru, New Zealand, and Port Moresby FIRs shall be 28 km (15 NM). This minimum is applied in accordance with 5.4.2.7 o PANS-ATM.

6.2.2.7		The minimum longitudinal separation minima using ADS-C climb and descent procedure (CDP) within the Auckland Oceanic, Brisbane, Fukuoka, Honiara, Melbourne, Nauru, New Zealand, and Port Moresby FIRs shall be 27.8 km (15 NM). This minimum is applied in accordance with 5.4.2.8 of PANS-ATM.

-END-