FANS Interoperability Team Meeting

(FIT/26)

Queenstown, NEW ZEALAND

18 – 22 March 2019

Agenda Item 5: Working Papers

CPDLC and ADS-C Data Link Performance Monitoring for the Brisbane FIR and Melbourne FIR

Presented by Airservices Australia

SUMMARY

This paper presents observed performance of the Controller Pilot Data Link Communication (CPDLC) and Automatic Dependent Surveillance – Contract (ADS-C) from the Brisbane FIR and Melbourne FIR as specified in the Global Operational Data Link Document (GOLD). Data was for the six months from July to December 2018.

1.	INTRODUCTION

  - This paper provides observed performance from the operational data link system in the Brisbane FIR and Melbourne FIR. The purpose of this paper is to present the most recent observed performance for the period 1 July 2018 to 31 December 2018 (reporting period) according to the GOLD Appendix D specifications.

  - The performance data observed from the Controller Pilot Data Link Communications (CPDLC) and Automatic Dependent Surveillance - Contract (ADS-C) systems are measured against the appropriate Required Communication Performance (RCP) and Required Surveillance Performance (RSP) specification to demonstrate that safety objectives that rely on the communications infrastructure can be met by the aircraft and ground systems.

  - This paper presents the data link performance by media type and by operator. These results are generated using an automated process that transforms the raw data into the format specified by the GOLD, and produces a report to highlight the key results and any potential poor performance.

2.	DISCUSSION

2.1	Performance Monitoring for the Brisbane FIR: ADS-C Performance

2.1.1	The RSP180 requirement is for 95% of downlinks to be received within 90 seconds and for 99.9% of downlinks to be received within 180 seconds. For the reporting period, 98.94% were received within 90 seconds and 99.68% were received within 180 seconds.

2.1.2	Of the 31 aircraft types monitored (with more than 100 downlinks), all except GLF4 (377 downlinks) meet the 95% RSP180 criteria. Seven meet the 99.9% RSP180 criteria while a further 21 are 99.0% or better for the 99.9% RSP180 criteria.

2.1.3	The 95% RSP180 criteria was met for all satellite paths and all paths were 99.0% or better for the 99.9% criteria. This is an improvement in performance, particularly for transactions via Iridium satellite paths, compared to the period January to June 2018.

2.1.4	131 out of 133 operator/aircraft type pairs with at least 100 ADS-C messages meet the 95% RSP180 criteria while 122 pairs are 99.0% or better for the 99.9% RSP180 criteria.

2.2	Performance Monitoring for the Brisbane FIR: CPDLC Performance

2.2.1	The RCP240 requirement is for 95% of CPDLC transactions to be completed within 180 seconds and for 99.9% of transactions to be completed within 210 seconds. For the reporting period, 99.20% of transactions were completed within 180 seconds and 99.49% of transactions were completed within 210 seconds.

2.2.2	All 16 aircraft types monitored meet the 95% RCP240 criteria. Only four types meet the 99.9% RCP240 criteria while a further 11 are 99.0% or better for the 99.9% RCP240 criteria.

2.2.3	The 95% RCP240 criteria is met for all satellite paths except for IG1 (Iridium). The 99.9% criteria is 99.0% or better for all paths except for IG1.

2.2.4	69 out of 71 operator/aircraft pairs with at least 100 transactions during the period meet the 95% RCP240 criteria while 58 pairs are 99.0% or better for the 99.9% RCP240 ACP criteria.

2.3	Performance Monitoring for the Melbourne FIR: ADS-C Performance

2.3.1	For the reporting period, 99.05% of downlinks were received within 90 seconds and 99.69% were received within 180 seconds.

2.3.2	Of the 31 aircraft types monitored (with more than 100 downlinks), all meet the 95% RCP240 criteria. Eight meet the 99.9% RCP240 criteria while a further 20 are 99.0% or better for the 99.9% RCP240 criteria.

2.3.3	The 95% RSP180 criteria was met for all satellite paths except for IG1 and IGW1 (both Iridium). The 99.9% criteria is 99.0% or better for all paths except for IG1, IGW1 and EUA1.

2.3.3	121 out of 123 operator/aircraft type pairs with at least 100 ADS-C messages meet the 95% RSP180 criteria while 115 pairs are 99.0% or better for the 99.9% RSP180 criteria.

2.4	Performance Monitoring for the Melbourne FIR: CPDLC Performance

2.4.1	For the reporting period, 99.63% of transactions were completed within 180 seconds and 99.76% of transactions were completed within 210 seconds.

2.4.2	All 15 aircraft types monitored meet the 95% RCP240 criteria. Three types meets the 99.9% RCP240 criteria while a further 10 are 99.0% or better for the 99.9% RCP240 criteria. Two types (B77L and GLEX) are lower than 99.0% for 99.9% RCP240 criteria.

2.4.3	The 95% RCP240 criteria is met for all satellite paths except for IG1 (Iridium) The 99.9% criteria is 99.0% or better for all paths except for IG1 and XXF (Inmarsat).

2.4.4	All 47 operator/aircraft pairs with at least 100 transactions during the period meet the 95% RCP240 criteria while 43 pairs are 99.0% or better for the 99.9% RCP240 criteria.

3.	ACTION BY THE MEETING

3.1	The meeting is invited to:

a)	note the content of the report

Appendix A: Performance Data for Brisbane FIR

ADS-C: Latency for Brisbane FIR

Table 1: RSP180 report by media type for Brisbane FIR

| Criteria | YBBB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RSP180 |  |  |  |  |  |
| Period | Jul-Dec 2018 |  |  | Jan-Jun 2018 |  |  |
|  | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec |
| By Media Type |  |  |  |  |  |  |
| All | 1 237 106 | 98.94 | 99.68 | 1 254 416 | 99.11 | 99.74 |
| VHF | 702 130 | 99.59 | 99.87 | 676 333 | 99.8 | 99.93 |
| SAT | 531 106 | 98.26 | 99.51 | 574 136 | 98.44 | 99.56 |
| HF | 3 870 | 73.06 | 88.32 | 3 947 | 79.7 | 91.99 |

Table 2: RSP180 report by Aircraft Operator/Type

| FIR | YBBB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Criteria | RSP180 |  |  |  |  |  |
| Period | Jul-Dec 2018 |  |  | Jan-Jun 2018 |  |  |
|  | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec |
| CPA/B773 | 24 906 | 95.17 | 98.87 | 19 975 | 96.6 | 98.91 |
| LAN/B787 | 4 132 | 96.71 | 98.81 | 4 182 | 97.2 | 99.21 |
| MAS/B737NX | 485 | 97.11 | 98.76 | 574 | 92.51 | 96.86 |
| MMD/FA8X | 207 | 96.62 | 97.58 |  |  |  |
| PAC/B767 | 1 666 | 97.42 | 98.98 | 2 061 | 96.17 | 98.25 |
| PAL/A330 | 11 680 | 98.18 | 98.93 | 13 128 | 99.56 | 99.83 |
| -/GL5T | 811 | 97.29 | 98.89 | 838 | 97.61 | 99.28 |
| -/GLEX | 4 952 | 97.62 | 98.83 | 6 259 | 98.58 | 99.55 |
| -/GLF4 | 334 | 93.11 | 97.01 | 1 006 | 95.63 | 98.91 |
| -/GLF5 | 3 394 | 96.52 | 98.53 | 3 462 | 96.59 | 98.76 |

CPDLC: ACP for Brisbane FIR

Table 3: RCP240 report by media type for Brisbane FIR

| FIR |  | YBBB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Criteria |  | RCP240 |  |  |  |  |  |  |  |  |  |
| Period |  | Jul-Dec 2018 |  |  |  |  | Jan-Jun 2018 |  |  |  |  |
|  |  | Message Counts | ACP % <= 180sec | ACTP % <= 120sec | ACP % <= 210sec | ACTP % <= 150sec | Message Counts | ACP % <= 180sec | ACTP % <= 120sec | ACP % <= 210sec | ACTP % <= 150sec |
| By Media Type |  |  |  |  |  |  |  |  |  |  |  |
| All |  | 37 659 | 99.2 | 99.09 | 99.49 | 99.4 | 38 135 | 99.13 | 99.05 | 99.43 | 99.34 |
| VHF |  | 20 410 | 99.62 | 99.6 | 99.75 | 99.73 | 18 056 | 99.64 | 99.71 | 99.71 | 99.75 |
| SAT |  | 13 041 | 99.0 | 98.86 | 99.46 | 99.26 | 15 626 | 98.98 | 98.85 | 99.46 | 99.25 |
| HF |  | 6 | 50.0 | 33.33 | 50.0 | 50.0 | 4 | 75.0 | 50.0 | 75.0 | 50.0 |
| By Ground Earth Station (GES) |  |  |  |  |  |  |  |  |  |  |  |
| APK1 | SAT | 3 605 | 98.56 | 98.36 | 99.22 | 98.83 | 2 516 | 98.77 | 98.73 | 99.44 | 99.32 |
| IOR2 | SAT | 264 | 98.86 | 98.48 | 99.24 | 98.86 | 931 | 99.57 | 99.46 | 100.0 | 99.68 |
| ROK | VHF | 453 | 99.34 | 98.9 | 99.34 | 98.9 | 420 | 99.05 | 99.05 | 99.76 | 99.05 |

Table 4: RCP240 report by Aircraft Operator/Type (exceptions only)

| FIR | YBBB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Criteria | RCP240 |  |  |  |  |  |
| Period | Jul-Dec 2018 |  |  | Jan-Jun 2018 |  |  |
|  | Message Counts | ACP % <= 180sec | ACP% % <= 210sec | Message Counts | ACP % <= 180sec | ACP % <= 210sec |
| ACA/B787 | 190 | 96.32 | 97.89 | 171 | 99.42 | 99.42 |
| ANA/B787 | 167 | 98.2 | 98.8 | 150 | 98.67 | 98.67 |
| CAL/A350 | 416 | 98.08 | 98.56 | 502 | 97.61 | 98.41 |
| CBJ/A330 | 143 | 91.61 | 92.31 | 201 | 91.54 | 92.54 |
| CCA/B787 | 199 | 98.49 | 98.49 | 139 | 95.68 | 96.4 |
| CPA/B773 | 899 | 98.78 | 98.89 | 1 041 | 98.56 | 99.04 |
| CSN/A330 | 2 468 | 98.26 | 98.5 | 3 147 | 98.92 | 99.3 |
| CSN/B787 | 919 | 98.69 | 98.91 | 621 | 98.87 | 99.19 |
| PAL/A330 | 402 | 98.26 | 98.76 | 413 | 99.03 | 99.27 |
| SIA/A350 | 810 | 98.52 | 98.64 | 919 | 98.59 | 98.69 |
| SIA/A380 | 555 | 98.38 | 98.74 | 457 | 98.69 | 99.12 |
| SLK/B38M | 171 | 94.74 | 96.49 | 172 | 95.35 | 98.26 |
| THA/B772 | 176 | 98.3 | 98.86 | 96 | 100.0 | 100.0 |

Appendix B: Performance Data for Melbourne FIR

ADS-C: Latency for Melbourne FIR

Table 5: RSP180 report by media type for Melbourne FIR (exceptions only)

| Criteria |  | YMMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RSP180 |  |  |  |  |  |
| Period |  | Jul-Dec 2018 |  |  | Jan-Jun 2018 |  |  |
|  |  | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec |
| By Media Type |  |  |  |  |  |  |  |
| All |  | 998 862 | 99.05 | 99.69 | 1 026 332 | 99.03 | 99.69 |
| SAT |  | 535 991 | 98.77 | 99.59 | 610 131 | 98.64 | 99.55 |
| VHF |  | 461 828 | 99.45 | 99.85 | 415 181 | 99.67 | 99.92 |
| HF |  | 1 043 | 69.8 | 84.85 | 1 020 | 72.65 | 86.76 |
| By Remote Ground Station (RGS)/Ground Earth Station (GES) |  |  |  |  |  |  |  |
| EUA1 | SAT | 1 102 | 97.19 | 98.55 | 777 | 98.46 | 99.61 |
| IG1 | SAT | 525 | 94.67 | 98.29 | 445 | 97.75 | 99.1 |
| XXF | SAT | 2 664 | 93.58 | 97.41 | 1 066 | 96.06 | 98.31 |
| CGK | VHF | 182 | 98.35 | 98.35 | 557 | 99.46 | 99.64 |

Table 6: RSP180 report by Aircraft Operator/Type for Melbourne FIR (exceptions only)

| FIR | YMMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Criteria | RSP180 |  |  |  |  |  |
| Period | Jul-Dec 2018 |  |  | Jan-Jun 2018 |  |  |
|  | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec | Message Counts | 95% % <= 90sec | 99.90% % <= 180sec |
| ALK/A330 | 14 384 | 98.35 | 98.57 | 13 252 | 99.92 | 99.94 |
| CPA/B773 | 9 912 | 94.58 | 97.27 | 10 037 | 97.53 | 98.81 |
| QQE/GLF6 | 102 | 97.06 | 98.04 |  |  |  |
| QTR/B772 | 12 705 | 92.07 | 96.62 | 12 556 | 92.44 | 96.91 |
| THA/B773 | 193 | 95.85 | 98.96 | 290 | 99.66 | 100.0 |
| -/FA7X | 244 | 97.95 | 98.36 | 287 | 97.21 | 97.91 |
| -/GLF4 | 202 | 95.05 | 97.03 | 541 | 96.67 | 98.52 |
| -/GLF6 | 1 100 | 98.18 | 98.82 | 1 154 | 98.61 | 99.57 |

CPDLC: ACP by Melbourne FIR

Table 7: RCP240 report by media type

| FIR | YMMM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Criteria | RCP240 |  |  |  |  |  |  |  |  |  |
| Period | Jul-Dec 2018 |  |  |  |  | Jan-Jun 2018 |  |  |  |  |
|  | Message Counts | ACP % <= 180sec | ACTP % <= 120sec | ACP % <= 210sec | ACTP % <= 150sec | Message Counts | ACP % <= 180sec | ACTP % <= 120sec | ACP % <= 210sec | ACTP % <= 150sec |
| By Media Type |  |  |  |  |  |  |  |  |  |  |
| All | 43 271 | 99.63 | 99.64 | 99.76 | 99.74 | 44 010 | 99.51 | 99.56 | 99.68 | 99.7 |
| SAT | 25 929 | 99.6 | 99.62 | 99.74 | 99.71 | 28 187 | 99.35 | 99.44 | 99.58 | 99.61 |
| VHF | 15 822 | 99.79 | 99.85 | 99.87 | 99.9 | 14 366 | 99.91 | 99.96 | 99.94 | 99.98 |
| HF | 2 | 50.0 | 0.0 | 50.0 | 0.0 | 6 | 66.67 | 66.67 | 83.33 | 83.33 |

Table 8: RCP240 report by Aircraft Operator/Type (exceptions only)

| FIR | YMMM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Criteria | RCP240 |  |  |  |  |  |
| Period | Jul-Dec 2018 |  |  | Jan-Jun 2018 |  |  |
|  | Message Counts | ACP % <= 180sec | ACP% % <= 210sec | Message Counts | ACP % <= 180sec | ACP % <= 210sec |
| CPA/B773 | 158 | 98.1 | 98.73 | 136 | 99.26 | 100.0 |
| QTR/B772 | 757 | 96.7 | 97.89 | 712 | 93.96 | 95.93 |
| VOZ/A330 | 154 | 98.7 | 98.7 | 191 | 99.48 | 100.0 |
| NAN/GLEX | 112 | 95.54 | 95.54 | 79 | 100.0 | 100.0 |