Thirty-First Meeting of the

Informal South Pacific ATS Co-ordinating Group

(ISPACG/31)

Hawaii, USA

6 to 9 March 2017

Agenda Item 4, Review Open Action Items, AI 25-2, Speed Variation Concern

Un-Announced Speed Changes in the Oakland Oceanic Flight

Information Region (KZAK FIR)

Presented by the United States (U.S.) Federal Aviation Administration (FAA)

SUMMARY

This paper provides an update on implementation of operator notification procedures for unannounced speed changes within the Pacific Oceanic Flight Information Regions

1.	INTRODUCTION

  - Aircraft speed changes that are not requested or announced to air traffic control (ATC) can place an unacceptable risk on separation minima.  Future Air Navigation System (FANS) 1/A equipped aircraft may be separated by as little as 30 nautical miles (NM) laterally and longitudinally.  When aircraft make a speed change and do not advise ATC, longitudinal separation can erode quickly.  Data shows that aircraft are frequently making Mach speed changes of Mach .04 or greater and not advising ATC.  A Mach speed change of M.04 equates to a change of around 26 knots.  In the Oakland FIR, a 10 minute reporting rate is used for all Automatic Dependent Surveillance – Contract (ADS-C) aircraft applying an Required Navigation Performance (RNP)4 30 NM separation standard.  If an aircraft makes a speed change of M.04/26 knots, ATC separation can erode by 4.33 NM before the next ADS-C report can make ATC aware of the speed change.  A 4.33 NM erosion is roughly 14 percent of the separation minima, which creates a significant safety risk.

  - In order to manage speed changes in the North Atlantic (NAT), all turbojet aircraft in the NAT are assigned a fixed Mach speed in the oceanic clearance that is issued to aircraft before entry into the NAT area.  This practice is governed by the following provision in the NAT section of the Regional Supplementary Procedures (SUPPs, Doc 7030):

6.1.1.7 The ATC-approved true Mach number shall be included in each clearance given to subsonic turbo-jet aircraft operating within Bodo Oceanic, Gander Oceanic, New York Oceanic, Reykjavik, Santa Maria Oceanic and Shanwick Oceanic control areas.

  - Iceland recently presented a paper at the NAT Air Traffic Management (ATMG) meeting on the Speed Distribution of Aircraft that have an Assigned Mach Speed (see Attachment 1).

    - The paper analyzed the speed information in ADS-C periodic reports and compared them to aircraft assigned fixed Mach speeds.  The paper indicated, “Aircraft manufacturers have stated that the reported Mach speed in ADS-C reports is instantaneous speed and not the target speed of the aircraft.  It has been stated that because the reported Mach speed is instantaneous, it could not be relied on for separation and fix-time calculations since it will show fluctuations from the target speed and may show significant fluctuations in some cases, such as when the aircraft encounters turbulence (target speed = the speed that the aircraft is striving to maintain).”

    - The data set in the Iceland paper was from 12,234 flights and 92,284 periodic ADS-C reports.  The data showed that 95% of the ADS-C reports were within, plus or minus .01 Mach, from the ATC cleared static Mach speed.  Additionally, 99.99% of the ADS-C reports were within, plus or minus .02 Mach, from the ATC cleared static Mach speed.

    - The data from the Iceland paper shows that Mach speed assignments can be very effective in managing aircraft speeds.  It also shows that 95% of ADS-C reports fall within .01 Mach of their cleared Mach speed assignment, which means that Mach speed fluctuation in ADS-C reports falls within .01 Mach most of the time.

  - While fixed Mach speed assignments can be effective in managing aircraft speeds, they are not as efficient because they cause extra fuel burn.  Most aircraft in the Pacific utilize Cost Index or Econ mode to manage their speed.  Cost index or Econ modes leads to a very gradual slowing of the aircraft speed as the weight of the aircraft is reduced.  These gradual speed changes do not place a risk to safety.  It is the abrupt large speed changes without advising ATC that occur when an aircraft encounters turbulence, slows to meet an airport curfew, or other pilot commanded large speed change events, that pose a risk to safety.

  - The FAA has presented data on the risks of unannounced speed changes to International Civil Aviation Organization (ICAO), Informal South Pacific Air Traffic Control Coordinating Group (IPACG), and ISPACG meetings.

2.	DISCUSSION

2.1  	Over the entire 2016 year, Oakland Air Route Traffic Control Center (ARTCC) analyzed the first 10 days of every month to study how many unannounced speed changes were taking place in the KZAK FIR.  The total number of unannounced speed changes for 2016 was 1289.

.

Below is an example of one airline and the information that is forwarded to the airline and Flight Standards.

| A (Code) | 13 | WX | Total |  |
| --- | --- | --- | --- | --- |
| 1227 | 12/3/2016 |  | 1 | 0.05 |
| 1227 | 12/6/2016 | 1 | 1 | 0.04 |
| 1292 | 12/4/2016 |  | 2 | 0.05 |
| 15 | 12/6/2016 |  | 1 | 0.04 |
| 1580 | 12/5/2016 |  | 2 | 0.05 |
| 200 | 12/9/2016 |  | 1 | 0.04 |
| 2147 | 12/10/2016 |  | 1 | 0.04 |
| 218 | 12/4/2016 |  | 1 | 0.04 |
| 534 | 12/2/2016 |  | 1 | 0.04 |
| 87 | 12/9/2016 |  | 1 | 0.04 |
| 98 | 12/6/2016 |  | 1 | 0.05 |

Below is the de-identified data for 2016.

2.2  	Oakland ARTCC has issued a Notice to Airman (NOTAM) that addresses this issue.

A1613/16 - ATTN ALL AIRCREWS -NEW PROCEDURAL REQUIREMENT FOR FLIGHTS OPERATING IN OAKLAND OCEANIC CONTROL AREA (KZAK) IN ORDER TO SUPPORT COST INDEX OR ECON SPEEDS AND MAINTAIN ATC SEPARATION SPACING AIRCREWS ARE REQUIRED TO USE THE FOLLOWING PROCEDURES IN THE KZAK FIR.  A PILOT MUST INFORM ATS EACH TIME THE CRUISING MACH NUMBER VARIES OR IS EXPECTED TO VARY BY A VALUE EQUAL TO OR GREATER THAN 0.02 MACH FROM: (1) THE MACH NUMBER AT FIR ENTRY; OR

(2) ANY SUBSEQUENT SPEED CHANGE NOTIFIED TO ATC IN FLIGHT

IF YOU HAVE ANY QUESTIONS CONTACT DUSTIN BYERLY (510)745-3543.

This NOTAM has been in place for the entire 2016 and the number of events continues to rise.  It was the hope that having such a NOTAM in place would lessen the number of events.

2.3  	ICAO Annex 2 has changed the verbiage in 3.6.2.2 that more closely aligns with the current KZAK NOTAM.

Deviation from Mach number/true airspeed: if the sustained Mach number/true airspeed at cruising level varies by plus or minus Mach 0.02 or more, or plus or minus 19 km/h (10 kt) true airspeed or more from the current flight plan, the appropriate air traffic services unit shall be so informed

2.4  	Oakland ARTCC will continue to try to work with the operators to reduce the number of un-cleared speed changes and gain more compliance with the procedure.

3.	ACTION BY THE MEETING

3.1	 The meeting is invited to note the information provided, and

3.2	Work together with ANSPs to eliminate unannounced speed changes.

3.3	Recommend to the regulatory bodies of each Pacific ANSP to publish a SAFO or equivalent that highlights to operators the important safety implications of unannounced speed changes.