Thirtieth Meeting of the

Informal South Pacific ATS Co-ordinating Group

(ISPACG/30)

Surfers Paradise, Australia

14 to 17 March 2016

Agenda Item [5.10]

ATS Interfacility Data Communications (AIDC) Performance for Brisbane

Presented by Airservices Australia

SUMMARY

This information paper provides a review of AIDC performance by Airservices Australia between

1.	INTRODUCTION

1.1		With the increasing use of AIDC to automate coordination between ATS Units, there is increasing importance to periodically review performance associated with this coordination medium.

1.2	AIDC monitoring also provides the ability to identify possible AIDC interoperability issues between two ATS Units.

2.	DISCUSSION

2.1	An analysis was recently conducted of 7 days (1st Mar – 7th May 2016) of AIDC messages transmitted and received by YBBB and YMMM. A summary of performance data and message usage statistics is included in Attachment 1 to this working paper Paper.

2.2	A Eurocat limitation involving the size of Field 18 that had previously been identified still exists. This limitation results in an LRM being sent in response to an ABI containing ‘excessive’ Field 18 information.

2.4	LRM analysis

2.4.1	The normal response to a syntactically correct AIDC message is a Logical Acknowledgement Message (LAM). If the received message is syntactically incorrect, the response should be a Logical Rejection Message (LRM).

2.4.2	An LRM analysis can indicate interoperability problems between two ATS Units. Several reasons for LRMs identified from our AIDC logs are included in the Table below:

| Description | NZZO | NFFF | Summary |
| --- | --- | --- | --- |
| Field18 limitation | 1 | 10 | Caused by a Eurocat limitation involving the size of Field 18 |
| Invalid message sequence | 2 | 16 | These generally related to hemstitch flights, typically flights transiting from NFFF to ANAU to NFFF to YBBB. The number of these LRMs is primarily due to UPR operations |
| CRC error |  | 1 | Possibly caused by a message corruption, or a CRC miscalculation |

2.4.3	As well as the LRMs identified above, a relatively large number (23) of LRMs were also generated for “in house” internal messaging between YMMM and YBBB. This is due to a known software fault.

2.5	AIDC Performance

2.5.1	The AIDC performance between South Pacific ATSUs is considered to be generally acceptable. For the most part, one way transit times were between 0.2 and 4 seconds. Refer to the following Tables:

|  | ABI |  | EST |  | ACP |  | TOC |  | AOC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # | (sec) | # | (sec) | # | (sec) | # | (sec) | # | (sec) |
| YBBB  YMMM | 6019 | 0.5 | 3365 | 0.2 | 3470 | 0.3 | 3417 | 0.2 | 3447 | 0.2 |
| YBBB  NZZO | 608 | 3.8 | 535 | 2.3 | 580 | 2.3 | 543 | 1.9 | 577 | 2.1 |
| YBBB  NFFF | 319 | 1.2 | 268 | 0.8 | 226 | 0.9 | 267 | 0.7 | 236 | 0.8 |
| YBBB KZAK | 6 | 1.7 | 6 | 1 | 12 | 0.7 | 8 | 1.0 |  |  |
| YMMM  YBBB | 7408 | 0.5 | 3461 | 0.3 | 3332 | 0.2 | 3497 | 0.2 | 3332 | 0.2 |
| NZZO  YBBB | 659 | 2.5 | 578 | 1.1 | 583 | 1.7 | 582 | 1.1 | 533 | 1.1 |
| NFFF  YBBB | 281 | 0 | 234 | 0 | 276 | 0.2 | 239 | 0 | 261 | 0 |
| KZAK  YBBB | 15 | 0 | 13 | 0 | 5 | 0.0 |  |  | 2 | 0 |

Table 1. Summary of AIDC transit times

2.5.2	In reviewing all the data (including AIDC exchanges with WAAF), there were a small number of messages where transit time exceeded 45 seconds. Within the South Pacific there was only one such message. The cause of this delay is unknown, but it only represents a very small proportion of the total messages exchanged.

| Msg | MRN | From | To | Msg sent | Ack sent | ACK | Tx time (sec) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ABI | YBBB027020 | YBBB | NZZO | 160306010539.00 | 160306010646.00 | LAM | 67 |

Table 3. List of delayed AIDC messages (South Pacific only)

2.5.3	Unfortunately, messaging with WAAF is still subject to intermittent delays (refer Attachment 2). There were 24 occasions where the transit time exceeded 45 seconds (ranging between 56 and 482 seconds), with an average of 220 seconds. It is understood that the fault lies in the network in Makassar, but the date for rectification is not known. A listing of delayed messages is included at Attachment 2.

2.5.4	The cause of the issue reported at ISPACG PT18 concerning ‘zero’ (or ‘negative’) transit times has been determined. Investigation indicates that it is a Eurocat problem caused by the (incorrect) rounding of times that are used in the AIDC message header. While this means that performance data may be incorrect by up to one second, it does not appear to cause any untoward problems, nor will the error increase (i.e. it is not a UTC synchronization issue as originally thought)

2.6	“Success rates” of AIDC messages

2.6.1	For every AIDC message transmitted a response of either a LAM or LRM is expected. The following tables show the percentage of LAM and LRM responses transmitted/received between YBBB and neighbouring ATSUs. Note that in accordance with the definition in the AIDC Guidance material, the success rate is expressed as the percentage of messages that are delivered to the destination ATSU (i.e. rather than the number of messages for which a LAM is received)

|  | Msg sent to | NZZO |  | NFFF |  | KZAK |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAM | LRM | LAM | LRM | LAM | LRM |
| Msg sent by YBBB | ABI | 99.67% | 0.00% | 97.49% | 2.51% | 90.91% | 9.09% |
|  | EST | 99.63% | 0.00% | 97.01% | 2.99% | 90.91% | 9.09% |
|  | ACP | 100.00% | 0.00% | 100.00% | 0.00% | 100.00% | 0.00% |
|  | CDN | 100.00% | 0.00% |  |  |  |  |
|  | PAC |  |  |  |  |  |  |
|  | CPL | 100.00% | 0.00% | 100.00% | 0.00% |  |  |
|  | TOC | 98.16% | 0.00% | 97.75% | 0.00% | 92.31% | 0.00% |
|  | AOC | 99.83% | 0.00% | 98.31% | 0.00% |  |  |

Table 4. Responses to AIDC messages sent by YBBB

|  | Msg sent by | NZZO |  | NFFF |  | KZAK |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAM | LRM | LAM | LRM | LAM | LRM |
| Msg sent to YBBB | ABI | 99.70% | 0.15% | 95.73% | 3.91% | 100.00% | 0.00% |
|  | EST | 100.00% | 0.00% | 100.00% | 0.00% | 100.00% | 0.00% |
|  | ACP | 100.00% | 0.00% | 100.00% | 0.00% | 100.00% | 0.00% |
|  | CDN |  |  |  |  | 100.00% | 0.00% |
|  | PAC | 100.00% | 0.00% |  |  |  |  |
|  | CPL |  |  |  |  |  |  |
|  | TOC | 99.83% | 0.00% | 100.00% | 0.00% |  |  |
|  | AOC | 99.81% | 0.00% | 100.00% | 0.00% | 100.00% | 0.00% |

Table 4. Responses to AIDC messages received by YBBB

2.6.2	From the above tables, it can be seen that while the success rate is high, it is not always 100%. For example, ABI and EST messages sent to NZZO had a success rate of 99.67% and 99.63% respectively. The cause of the “missing messages” is not known, and no investigation into this is currently in place.

3.	ACTION BY THE MEETING

3.1	The meeting is invited to:

a)	Note the information in this paper

- Consider the need to conduct regular analysis of AIDC performance and interoperability

- Discuss the questions posed concerning AIDC success rate

References

Guidance Material for End-to-End Safety and Performance Monitoring of Air Traffic Service (ATS) Data link systems in the Asia Pacific Region, Version 4.0 — Feb 2011

Delayed AIDC messages (>45 seconds)

| Msg | MRN | From | To | Msg sent | Ack sent | ACK | Tx time (sec) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TOC | YBBB188573 | YBBB | WAAF | 160301025100.00 | 160301025207.00 | LAM | 67 |
| AOC | YBBB198515 | YBBB | WAAF | 160302071514.00 | 160302071610.00 | LAM | 56 |
| EST | WAAF044648 | WAAF | YBBB | 160302134633.00 | 160302135435.00 | LAM | 482 |
| TOC | WAAF044651 | WAAF | YBBB | 160302135300.00 | 160302135440.00 | LAM | 100 |
| ABI | YBBB000118 | YBBB | YMMM | 160302141016.00 | 160302141611.00 | LAM | 355 |
| EST | WAAF000001 | WAAF | YBBB | 160302203846.00 | 160302204236.00 | LAM | 230 |
| TOC | WAAF000002 | WAAF | YBBB | 160302203846.00 | 160302204236.00 | LAM | 230 |
| EST | WAAF000003 | WAAF | YBBB | 160302203847.00 | 160302204237.00 | LAM | 230 |
| EST | WAAF000005 | WAAF | YBBB | 160302203848.00 | 160302204238.00 | LAM | 230 |
| EST | WAAF000007 | WAAF | YBBB | 160302203849.00 | 160302204238.00 | LAM | 229 |
| TOC | WAAF000006 | WAAF | YBBB | 160302203848.00 | 160302204239.00 | LAM | 231 |
| TOC | WAAF000004 | WAAF | YBBB | 160302203847.00 | 160302204239.00 | LAM | 232 |
| TOC | WAAF000010 | WAAF | YBBB | 160302203850.00 | 160302204240.00 | LAM | 230 |
| EST | WAAF000009 | WAAF | YBBB | 160302203850.00 | 160302204241.00 | LAM | 231 |
| EST | WAAF000011 | WAAF | YBBB | 160302203851.00 | 160302204241.00 | LAM | 230 |
| TOC | WAAF000008 | WAAF | YBBB | 160302203849.00 | 160302204242.00 | LAM | 233 |
| TOC | WAAF000012 | WAAF | YBBB | 160302203852.00 | 160302204242.00 | LAM | 230 |
| TOC | WAAF000014 | WAAF | YBBB | 160302203852.00 | 160302204243.00 | LAM | 231 |
| EST | WAAF000013 | WAAF | YBBB | 160302203852.00 | 160302204243.00 | LAM | 231 |
| EST | WAAF000015 | WAAF | YBBB | 160302203853.00 | 160302204244.00 | LAM | 231 |
| TOC | WAAF000016 | WAAF | YBBB | 160302203853.00 | 160302204246.00 | LAM | 233 |
| EST | WAAF000017 | WAAF | YBBB | 160302203853.00 | 160302204247.00 | LAM | 234 |
| ABI | YBBB027020 | YBBB | NZZO | 160306010539.00 | 160306010646.00 | LAM | 67 |
| TOC | YBBB031978 | YBBB | WAAF | 160306132056.00 | 160306132437.00 | LAM | 221 |
| ACP | WAAF007063 | WAAF | YBBB | 160307041506.00 | 160307041843.00 | LAM | 217 |
| ACP | WAAF007065 | WAAF | YBBB | 160307041506.00 | 160307041845.00 | LAM | 219 |