Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France, 2-5 November 2010)

Agenda Item 4:	Provide Status on CPWG/9 Actions

ANCHORAGE UPDATE ON ELIMINATION OF RESTRICTIONS

(Action Item # CP08-12)

(Presented by Federal Aviation Administration)

SUMMARY This working paper presents information regarding the ongoing work among Anchorage Air Route Traffic Control Center (ZAN), users, and neighboring Air Navigation Service Providers to reduce NOTAMed restrictions wherever possible.

- Introduction

# Since routes were first created in the early 1990s to link the Russian Trans East (RTE) with Alaska, efforts have been made to efficiently use Anchorage Air Route Traffic Control Center (ZAN) airspace.

# As airspace is evaluated for radar and frequency coverage, as well as conflictions with designated airspace and other routes, many decisions need to made that balance service with safety.  The easy thing to do would be provide a structure of fixed routes for aircraft to fly.  Unfortunately this provides limited service to customers who are trying to be efficient and fiscally prudent.

- Discussion

2.1	Since CPWG/9 several improvements have been made to allow for more efficient westbound transitions at LISKI and PILUN.  Most notably Russia agreed to give and receive transfers in feet at these fixes as opposed to meters.  Though altitudes are still limited west of the boundary, the ability to use feet all the way through ZAN airspace allows for more efficient altitude use up to the boundary, and allows aircraft to transition to LISKI and PILUN along more diverse routes.  Aircraft can now transition westbound to LISKI from TAYTA (on 141W) and south as opposed to VOLOB and south as before.  Traffic to PILUN continues to fly anywhere from TAYTA and north.  NCA32 is now open eastbound and westbound, and 157W crossing restrictions westbound have been eliminated.

2.2	For RTE fixes many improvements have been made for eastbound and westbound traffic.  Westbound aircraft can now access B932 BAMOK by transitioning from NOLTI to EPLOS.  This can save as much as 1000 pounds of fuel from MARLO in comparison with previous restrictions.  In response to user requests, eastbound restrictions have been reduced as much as radar and frequencies will allow.  A new fix SANGL is being created east of KUTAL (B233) to allow User Preferred Routings (UPRs) to start hundreds of miles west of GAL.  Aircraft eastbound from VALDA (H222) can now fly UPRs starting at ICEEE, ERNIK traffic (B240) can transition at IDROD, BESAT (B583) restrictions have been reduced to MUNRI, and eastbound aircraft at BAMOK (B932) need only stay on route until LESAD as opposed to proceeding all the way to MCG.

2.3	Operational advantages have also been gained through the reduction of intervals at which aircraft are track loaded to Russian fixes.  Whereas all RTE fixes from FRENK southwest to BAMOK were previously loaded at 20 minute intervals, now each of these has been reduced.  The original test from February 2010 removing guaranteed “windows” and replacing them with a track load of 10 minutes at each fix proved to be a little ambitious.  When several fixes were overloaded causing excessive controller workload and reduced efficiencies to users, a compromise was reached.  RTE fixes VALDA and BESAT are now loaded at 15 minute intervals for each altitude, and all other RTE fixes are loaded at 10 minute intervals.  This has proven to be a good way to expand efficiencies while creating acceptable spacing at popular fixes.  This procedure has been in effect since June and has not yet experienced the heavy RTE traffic months.  ZAN will continue to study the benefits and feasibility of this procedure throughout the winter months and should have a more comprehensive evaluation to present at CPWG/11.

2.4	The attached map depicts the reduced restrictions and improved efficiencies.  When Russia implements RVSM in November 2011, ZAN looks forward to again exploring the topic of reduced restrictions.

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this Working Paper; and

  - provide recommendations to air navigation service providers for increasing the efficiency of airspace.