Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France, 2-5 November 2010)

Agenda Item 4:	Provide Status on CPWG/9 Actions

DEVELOPMENT OF AN ARCTIC AREA ATM OPERATIONAL CONTINGENCY PLAN

(Action Item #CP02-27)

(Presented by the Federal Aviation Administration)

SUMMARY This working paper presents the current draft version of the Arctic Area Air Traffic Management Operational Contingency Plan for consideration by the meeting.

1.	Introduction

# During the CPWG/9 meeting, the Federal Aviation Administration (FAA) noted that the development of a plan to deal with catastrophic failures was mandated by the International Civil Aviation Organization (ICAO) in Annex 11.

# FAA presented a draft table of contents for an Arctic area ATM contingency plan that was based on the North Atlantic (NAT) Contingency Plan, which was first published on 22 October 2009.  Ultimately, it would be expected that material developed for the Arctic area would be incorporated into other regional contingency plans.

# The meeting was reminded that this action item had remained on the CPWG work program without progress since CPWG/2.  The FAA position was that the CPWG needed to agree to develop an Air Traffic Management (ATM) Operational Contingency Plan for the Arctic area, or remove it from the work program.

# The ANSPs agreed that the contingency plan needed to be progressed.  NAV CANADA planned to have their portion of the plan completed before CPWG/10.  State ATM and the FAA agreed to begin work.  Each ANSP was asked to submit completed portions of the plan to the FAA for compilation and presentation to CPWG/10.

- Discussion

- Based on the information already available in the NAT Contingency Plan and other input received since the CPWG/9 meeting, a draft Arctic Area ATM Operational Contingency Plan is provided at Appendix A.

2.2	Information for the plan is still needed from NAV CANADA and the Russian Federation.  The chapters for Bodo and Reykjavik Flight Information Regions (FIRs) have been copied directly from the NAT Contingency Plan, and further review and update may be necessary for application to the Arctic airspace.

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this paper and the draft Arctic Area ATM Operational Contingency Plan included in Appendix A;

  - provide additional relevant contingency material and suggested changes to draft plan; and

  - establish a goal date for completion of the plan.

AIR TRAFFIC MANAGEMENT OPERATIONAL CONTINGENCY PLAN

FOR THE ARCTIC AREA

First Edition:  [date]

Published by the Cross Polar Trans East Air Traffic Management Providers’ Working Group (CPWG)

EXCLUSION OF LIABILITY

A printed or electronic copy of this document, plus any associated documentation, is provided to the recipient as is and without any warranties as to its description, condition, quality, fitness for purpose or functionality and for use by the recipient solely for guidance only.  Any implied conditions terms or warranties as to the description, condition, quality, fitness for purpose or functionality of the software and associated documentation are hereby excluded.

The information published in this document is made available without warranty of any kind; the air navigation service providers and members of the CPWG  accept no responsibility or liability whether direct or indirect, as to the currency, accuracy or quality of the information, nor for any consequence of its use.

TABLE OF CONTENTS

EXCLUSION OF LIABILITY ..................................................................................................................i

PART 1 - CONTINGENCY SITUATIONS AFFECTING INDIVIDUAL ATC FACILITIES

Chapter X: DETAILED PROCEDURES – [FACILITY]

1.1  FIR FOR WHICH THE CONTINGENCY PLAN APPLIES

1.2  FIRs WITH SUPPORTING PROCEDURES

1.3  NOTIFICATION PROCEDURES

General Provisions

Levels of service

In a limited service situation

In a no service situation

Air Traffic Flow Management

Responsibilities of Adjacent ANSPs

1.4  LIMITED SERVICE - PROCEDURES

Dispersal of traffic

Communications

Notification

Roles and responsibilities of adjacent facilities

Separation Minima

Contingency Tracks

Air Traffic Management

Disruption of ground/air communication capability

Effect on flights

Disruption of ability to provide control services

Separation standards

Contingency tracks

Air Traffic Flow Management

Responsibilities of adjacent ANSPs

1.5  NO SERVICE - PROCEDURES

Dispersal of traffic

Communications

Notification

Loss of ground/air communication capability

Effect on flights

Loss of ability to provide control services

Flight planning

Separation standards

Contingency tracks

Air Traffic Flow Management

Communications

Radar service

Responsibilities of adjacent ANSPs

1.6  ROLES AND RESPONSIBILITIES OF ADJACENT OAC’S AND ACC’S

Separation

1.7  FLIGHT CREW AND OPERATOR PROCEDURES

For flights within the XXX OCA – General

For flights within the XXX OCA – Westbound

For flights within the XXX OCA – Eastbound

For flights approaching the XXX OCA when the contingency is activated

Not in Receipt of an Oceanic Clearance

In receipt of an acknowledged Oceanic Clearance outside XXX FIR

In receipt of an acknowledged Oceanic Clearance within XXX FIR

Entering from another OCA

1.8  XXX OACC – CONTINGENCY ROUTE STRUCTURE

For activation within XXX OCA

For activation within adjacent OCA/FIR

A FIR

B FIR

C FIR

D OCA/FIR

Day Tracks

Night Tracks

Contingency Tracks

WESTBOUND TRACKS

EASTBOUND TRACKS

1.8  LONG TERM CONTINGENCY ARRANGEMENTS

Level 1:  Emergency Services

Level 2:  Single Stand Operation

Level 3:  Capacity Limited, Normal Control Service

Level 4: Normal Control Service

Level 5: Total Restoration of Services by Gander Oceanic

Appendix A - Procedures in event of XXX evacuation

Appendix B - Contact Details - XXX OACC

PART II -   CONTINGENCY SITUATIONS AFFECTING MULTIPLE FIRS

SCOPE OF THE PLAN

VOLCANIC ASH CONTINGENCY PLAN

AIR TRAFFIC CONTROL PROCEDURES

FOREWORD

This document is for guidance only.  Regulatory material relating to Arctic aircraft operations is contained in relevant ICAO Annexes, PANS/ATM (Doc 4444), Regional Supplementary Procedures (Doc 7030), State AIPs and current NOTAMs, which should be read in conjunction with the material contained in this Document.

The airspace generally referred to as the Arctic area is comprised of the flight information regions (FIRs) in the vicinity of the North Pole.  A number of States, with their Oceanic Control facilities geographically dispersed, provide air traffic control services in this vast airspace with inherent communication, navigation and surveillance (CNS) limitations.

The Air Traffic Management Operational Contingency Plan for the Arctic area is primarily for the information of operators and pilots planning and conducting operations in the Arctic airspace.  The intent is to provide a description of the arrangements in place to deal with a range of contingency situations.

This document will be made available electronically.

PART I - ATM CONTINGENCY PLAN FOR FLIGHTS OPERATING WITHIN THE

ARCTIC FLIGHT INFORMATION REGIONS AND OCEANIC CONTROL AREAS

- Objective

- The Air Traffic Management (ATM) Contingency Plan contains details of the arrangements in place to ensure, as far as possible, the continued safety of air navigation in the event of partial or total disruption of Air Traffic Services within the Arctic area.  This document is produced in accordance with the requirement of the International Civil Aviation Organization (ICAO) Annex 11 – Air Traffic Services, Chapter 2, paragraph 2.29.

- This plan details both common procedures throughout the Arctic area and the procedures specific to the individual air navigation service providers (ANSPs) within the Arctic area.  The plan is presented in two parts:

Part 1 – Contingency Situations Affecting Air Traffic Control (ATC) Facilities

ATC services within the Arctic area are provided by a number of States from a number of geographical locations.  This plan details the contingency arrangements at each of these facilities.  It is considered unlikely that any physical contingency at one particular facility will affect another directly, hence in Part 1 of this document the procedures for each Flight Information Region (FIR) or Control Area (CTA) are considered independently.

Part 2 – Contingency Situations Affecting Multiple FIRs

This part of the plan considers events which are likely to affect more than one facility within the Arctic area.  In particular, these include the contingency arrangements in place to deal with the airspace contaminated by volcanic ash.

- States and FIRs affected

- This document contains contingency procedures for those ANSPs providing an ATC service within the Arctic area.

- The States, FIRs and ACCs affected by this contingency plan and for which procedures are promulgated are:

Canada - Edmonton FIR

Denmark/Greenland - Sondrestrom FIR

Iceland 	- Reykjavik Oceanic FIR/CTA

Norway - Bodo Oceanic FIR

Russian Federation

- Magadan FIR

- Murmansk FIR

United States - Anchorage Arctic FIR

- Scope of the Plan

- This plan addresses contingency situations which may result in a degradation of the ATC service provided (limited service) as well as situations where there is a total loss of the ability to provide ATC services (no service).  It also provides information on:

- common procedures adopted by ATC facilities in the event of contingency situations; and

- detailed procedures adopted by individual ATC facilities in the event of a contingency situation.

- Where available, information is provided outlining the steps taken by ANSPs to deal with a long term unavailability of an ATC facility.  In particular, the procedures detailed by each ATC facility will, insofar as possible, include the following for the FIRs for which the contingency plan applies:

- FIRs with supporting procedures;

- notification procedures;

- contingency route structures for activation within the FIR, as well as for activation within an adjacent FIR;

- Long term contingency arrangements

- Contact details

- The plan also provides arrangements for implementation of:

- limited service in the case of:

  - disruption of ground/air communication capability, and/or

  - disruption of ability to provide control services; and

- no service in the case of:

- loss of ground/air communication capability, and/or

- loss of ability to provide control services.

- Implementation of the plan

- In the event of the need to adopt contingency procedures, ANSPs will notify all affected agencies and operators appropriately.  In limited service situations the ANSP will determine the level of notification necessary and take action as required to cascade the information.  In no service situations it is likely that the ATC facility involved will be subject to evacuation.  In this instance, the ANSP will issue NOTAMs and broadcast on appropriate frequencies that contingency procedures have been initiated.

- The notification process employed by individual ANSPs is detailed in their respective entries in this plan, however the general format will be as follows:

- Issue a NOTAM advising operators of the evacuation.  The following is an example of the type of information which may be promulgated:

“Due to emergency evacuation of [name of facility], all ATC services are terminated.  Flights within [name] FIR should continue as cleared and contact the next ATC agency as soon as possible.  Flights not in receipt of an oceanic clearance should land at an appropriate airfield or request clearance to avoid [name] FIR.  Flights should monitor [list frequencies].”

- Broadcast an evacuation message on appropriate frequencies:

“Emergency evacuation of [name of facility] is in progress. No air traffic control service will be provided by [name of facility].  Use extreme caution and monitor [list frequencies], emergency frequencies, and air to air frequencies.  Contact the next air traffic control unit as soon as possible.”

- Traffic Information Broadcast by Aircraft (TIBA) procedures

- The following Traffic Information Broadcast by Aircraft (TIBA) communications procedures have been developed in accordance with ICAO Annex 11 – Air Traffic Services, Attachment C.  These procedures should be applied by pilots when executing an altitude change to comply with a previously issued oceanic clearance:

- At least 3 minutes prior to the commencement of a climb or descent the flight should broadcast on the last assigned frequency, 121.5, 243.0 and 123.45 the following:

“ALL STATIONS [callsign][direction] DIRECT FROM [landfall fix] TO [oceanic entry point].  LEAVING FLIGHT LEVEL [number] FOR FLIGHT LEVEL [number] AT [distance] [direction] FROM [oceanic entry point] AT [time].”

- When the level change begins, the flight should make the following broadcast:

“ALL STATIONS [callsign] [direction] FROM [landfall fix] TO [oceanic entry point].  LEAVING FLIGHT LEVEL [number] NOW FOR FLIGHT LEVEL [number].”

- When level, the flight should make the following broadcast:

“ALL STATIONS [callsign] MAINTAINING FLIGHT LEVEL [number].”

PART II - CONTINGENCY SITUATIONS AFFECTING ATC FACILITIES

CHAPTER 1:  DETAILED CONTINGENCY PROCEDURES

ANCHORAGE ARCTIC FIR

FIR FOR WHICH THE CONTINGENCY PLAN APPLIES

Anchorage Arctic FIR

FIRS WITH SUPPORTING PROCEDURES

Nil

GENERAL PROVISIONS

Traffic flow in the Anchorage Arctic FIR is largely east to west.  North America to Asia traffic finds the generally clear skies and light and variable winds an attractive and reliable route to avoid North Pacific jet streams.  There is also west to east traffic and occasional south to north traffic from Anchorage to Europe.  Eastbound and westbound traffic flows between established fixes in the Magadan and Murmansk FIRs to restricted points of latitude in the Edmonton FIR.  In general Anchorage Arctic FIR acts as a link between these two neighboring facilities, and very few route or altitude changes occur in the Anchorage Arctic FIR.

Communications in this area are provided through a number of resources.  Anchorage Air Route Traffic Control Center (ZAN) has very high frequency (VHF) and ultra high frequency (UHF) locations along the Arctic coast of Alaska.  Remote communication sites are located at Barter Island (BTI), Deadhorse (SCC), Barrow (BRW) and Point Lay (PIZ).  Primary reporting along 141W, the ZAN/Edmonton boundary, is done through Gander high frequency (HF) Radio.  Primary reporting along 169W, the ZAN/Magadan boundary is done through Magadan HF Radio.  Service is augmented by San Francisco ARINC through HF, VHF and Long Distance Operational Control (LDOC) capabilities.  Iceland Radio also has capabilities in the Anchorage Arctic FIR.

Surveillance is achieved through radar to approximately 72N.  Radar sites are located at BRW, SCC and BTI.  Anchorage also has automatic dependent surveillance-contract (ADS-C) capabilities up to approximately 83N.  Though Arctic ADS aircraft are not currently displayed on ZAN radar scopes, when the Ocean 21 automation system is implemented in the Arctic portion of ZAN, these reports will be displayed.  This transition is expected in the latter half of 2011.

LEVELS OF SERVICE

Limited Service:  A limited service could result from:

-	Staffing Shortage

-	Minor equipment outage

-	Failure in a support facility

No Service:  A loss of all service could result from:

-	Major equipment outage

-	Loss of ZAN facility

-	Evacuation of facility

Limited Service

Limited Communications

Due to redundancies in ZAN communication the loss of a single communication service provider would not create a significant reduction in communication services in the Anchorage Arctic FIR.  In the event of lost VHF capability there is sufficient HF coverage to provide regular service.  Murmansk Area Control Centre (ACC), Edmonton ACC, and Magadan ACC would be advised and control messages could be passed by Gander and San Francisco HF Radio.

A loss of either San Francisco or Gander Radio would also not cause a disruption in basic services in Anchorage Arctic FIR.  The two facilities’ coverage is largely redundant in the Anchorage Arctic area.  Murmansk ACC, Edmonton ACC, and Magadan ACC would be advised to use alternate HF frequencies where required.  Any required HF broadcast required through Gander Radio could be supplemented by Iceland Radio.  Direct satellite voice communication is available up to 83N through the INMARSAT network, and aircraft with Iridium have coverage up to the North Pole.

Limited Air Traffic Service

In the event of limited air traffic services, ZAN supervisors would communicate with neighboring facilities as to the nature of the limitation, and provide any restrictions that need to be imposed.  As much notice as possible would be proffered to airspace users to allow them time to make informed flight planning decisions.  All normal routes and restrictions would remain in place. The ZAN Traffic Management Unit (TMU) would coordinate with all affected facilities as to flow rates and slot times if necessary.

No Service

Loss of Ground-to-Air Communications

In the event ZAN is unable to communicate with traffic in the Anchorage Arctic FIR, aircraft would be expected to remain on their current routes and altitudes until communications with another air traffic service provider can be established.  Aircraft would be expected to maintain their filed airspeed or last assigned mach number.  Edmonton ACC, Murmansk ACC, and Magadan ACC would be advised of the situation, and each would be asked for locations that they will have communication on various routes.  Each facility would be advised to broadcast to aircraft that communications with ZAN is not possible at this time, and the place and frequency to contact the next facility.  Boundary estimates would continue to be passed by ZAN to the required facilities via the normal or backup communications channels available.

No restriction of service or airspace would be expected from a loss of communication with aircraft transiting the Anchorage Arctic FIR.

Loss of ATC Service (ATC ZERO)

In the event of complete loss of ZAN’s ability to provide air traffic services in the Anchorage Arctic FIR, immediate notification would be made to Murmansk ACC, Magadan ACC, and Edmonton ACC.  Flights already airborne would be allowed to continue through Anchorage Arctic FIR.  Murmansk ACC would be advised to pass DEKMO estimates directly to Edmonton ACC.  Magadan ACC would be advised to request 141W crossing estimates from aircraft bound for Canadian airspace and pass these estimates and altitudes to Edmonton ACC.  Edmonton ACC would be advised to request Russian entry fix estimates and pass them to Magadan.  All aircraft not yet airborne will be held on the ground until they could flight plan around the affected airspace or an appropriate air traffic flow management (ATFM) program could be established.

Loss of Interfacility Communications

All communication between support facilities would be done by commercial telephone.  All requisite numbers are included in the “Contact Details” section at Appendix A.

RESUMPTION OF SERVICE

All affected facilities would be advised when ZAN was able to resume full or partial ATC services.  If ZAN was forced to resume service from a remote location, partial service would be expected for an undetermined period of time.

CONTACT DETAILS – ANCHORAGE CENTER

| Facility | Telephone Number | AFTN |
| --- | --- | --- |
| Anchorage Center (ZAN) |  | PAZAZQZX |
| Operations Manager in Charge | 1-907- 269-1103 |  |
| North Area Supervisor | 1-907-269-1915 |  |
| Magadan ACC |  | UHMMZRZX |
| Operations Supervisor | 011-7-413-260-6719 Or 011-7-413-260-7180 |  |
| HF | 2884, 5550, 8837, 11390 |  |
| Murmansk ACC |  | ULMMZOZX |
| Operations Supervisor | 011-7-815-228-1314 |  |
| HF | 8950, 11390 |  |
| Edmonton Centre |  | CZEGZQZX |
| Shift Manager | 1-780-890-8397 |  |
| North-High Supervisor | 1-780-890-4712 |  |
| Gander Radio |  | CYQXYSYX |
| Shift Manager | 1-709-651-5222 |  |
| HF | 2971, 4675, 8891, 11279 |  |
| San Francisco ARINC |  | KSFOXAAG |
| Team Leader | 1-925-294-8400 |  |
| Arctic HF | 6640, 11342, |  |
| Satellite Voice | 1-410-266-4430 |  |

CHAPTER 2:  DETAILED CONTINGENCY PROCEDURES

BODO OCEANIC FIR

FIR FOR WHICH THE CONTINGENCY PLAN APPLIES

Bodo Oceanic FIR

FIRs WITH SUPPORTING PROCEDURES

Reykjavik FIR

Norway FIR, Stavanger AoR

Norway FIR, Oslo AoR

Sweden FIR, Stockholm AoR

Fin FIR, Rovaniemi AoR

Murmansk FIR

LIMITED SERVICE

The Regional Rules and Regulation for Bodo Oceanic Area Control Centre (OAC) address the issues of limited service provision in the Nat Region. In the event that Bodo OAC/ATCC must be evacuated, the specifics of section …. (No service) will immediately be activated.

Once the Bodo Area Control Centre has been sterilized of oceanic traffic, the rebuilding of service provision will begin.

Until full service can be re-established Bodo OAC will delegate control of aircraft within Bodo Oceanic Control Area to Stavanger ATCC, Bodo ATCC Domestic sectors, Stockholm ACC and Reykjavik ACC.

Level 1:   Emergency Services

Control services to EMERGENCY and HUMANITARIAN Flights along with limited Airspace Reservations (no aircraft joining or departing)

Level 2:  Domestic Sector Operation

Emergency and Humanitarian flights along with limited Airspace Reservations (no aircraft joining or departing) would take priority. Control service provided through minimum staff with limited equipment. This would result in metered flow through the Bodo OCA of commercial, general aviation, military and state aircraft.

Level 3:  Capacity Limited, Normal Control Service

Emergency and Humanitarian flights, along with limited Airspace Reservations (no aircraft joining or departing) would take priority. Control service with accompanying clearance delivery communication would be offered through an increased number of operating positions. Flow restrictions and metering would be established to reduce congestion.

Level 4:   Normal Control Service

Provide control service using Bodo Domestic Sectors and Stavanger ATCC. ALL required VHF and HF communication will be available. The Bodo ATCC, Domestic Sectors will provide the full range of services required by westbound aircraft, and coordinate the traffic with adjacent centre.

Level 5:  Total Restoration of Services by Bodo OAC

Full Oceanic en route and planning service restored. Control of  Bodo OCA is returned back to Bodo OAC. Normal communication capabilities.

Dispersal of Air Traffic

Aircraft already within the Bodo OCA, will be given priority for the limited services available.  Aircraft intending to enter Bodo OCA will, if necessary, be restricted to meet the limited service capability.  Random westbound routing may be restricted.

Communications

Communication services will be maintained to the possible extent using available equipment supplemented with the assistance of adjacent facilities. Aircraft unable to contact Bodo Radio on HF Frequency shall call one of the following stations:

Iceland Radio

Shannon Aeradio

Notification

Bodo OAC will notify all adjacent units and co-ordinate necessary traffic restrictions.

Responsibilities of adjacent OACs and ATCCs

Upon notification by Bodo OAC the adjacent facilities shall be responsible to implement procedures necessary to meet Bodo OCA restrictions.

For Westbound traffic, Bodo OAC will issue clearances to 0o Longitude only.  Reykjavik OAC will assume responsibility west of 0o Longitude.  Eastbound traffic will be accepted as normal.

Separation Minima

Bodo OAC will be responsible for ensuring the coordination and implementation of any additional separation standard.

Same direction longitudinal separation may be increased if (e.g. add 3 minutes).  Lateral separation will not be increased.  Flight profile changes in the Bodo OCA may be limited.

Contingency Tracks

Bodo OAC shall publish contingency tracks within the Bodo OCA and ensure that the available limited Air Traffic Services are not overloaded.

Air Traffic Flow Management (ATFM) Requirements

Bodo OAC will, in conjunction with the EUROCONTROL Central Flow Management Unit (CFMU), initiate ATFM measures as required.

NO SERVICE

Bodo ATCC includes Bodo Domestic Control, Bodo Oceanic Control and Bodo HF. Should Bodo ATCC be evacuated, the potential exist for a major disruption to Air Traffic Control service within Bodo AoR (Norway FIR from 62N  to Russian Border boundary) and Bodo OFIR/OCA.

As soon as possible after evacuation Contingency Message will be forward to all concerned agencies.

Dispersal of Air Traffic

Where possible, aircraft already within the Bodo OCA will be notified that no services are available.  Oceanic traffic intending to operate through Norwegian domestic airspace will require further clearance to do so.

Aircraft that elect to continue flight through Bodo OCA will operate on published tracks and at published flight levels.  Aircraft that already are on random track will require specific co-ordination and approval from all concerned ATS units until the contingency tracks become active. The lowest flight level available for transiting flights will be FL280.  Traffic to and from Svalbard/ Longyear will use flight levels appropriate to direction of flight until exiting Bodo OCA.  The highest available flight level will be FL270.

Communications

Bodo Radio and adjacent facilities will extend HF monitoring and assist with flight information services to aircraft within or about to enter Bodo OCA.

If unable to establish radio contact with adjacent facilities, flights may use SATCOM voice and satellite telephone to provide position reports.

Notification

Bodo OAC will attempt to notify adjacent units of the loss of service. If adjacent units are unable to establish contact with Bodo OAC, the phone numbers listed in the appendix can be used. Adjacent facilities are also listed.

Responsibilities of adjacent OACs and ATCCs

Adjacent OACs/ATCCs should implement ATFM measures as required.  In addition, they may co-ordinate and publish routes to minimize the impact of the loss of service.  Norwegian domestic ATCC will ensure that the necessary oceanic separation minima are established for traffic entering Bodo OCA from their area.

Reykjavik OAC will be required to:

Clear eastbound traffic in accordance with the contingency tracks and provide necessary separation; and

Organize a method of passing and receiving estimates with the Norwegian domestic ATCC.

Separation Minima

Longitudinal separation for all traffic entering Bodo OCA from Norwegian domestic airspace shall be increased by 10 minutes.

Contingency Tracks

The contingency tracks, FL280 or above, will be laterally separated and will use flight levels appropriate to direction of flight.  Before leaving Bodo OCA, aircraft operating on contingency tracks shall request a clearance from the appropriate adjacent unit. Change of flight level will not be permitted while on the contingency tracks.

Air Traffic Flow Management

Bodo OAC will, in conjunction with the CFMU, initiate ATFM measures as required.

FLIGHT CREW AND OPERATOR PROCEDURES

Within Bodo OCA

The procedures outlined below are to be used as guidance for pilots in the immediate aftermath of sudden withdrawal of the ATC service as described above.

On receipt of the contingency message pilots are requested to broadcast the information to other flights on VHF frequency 127,725 or 121,5.

Flights should establish communication with the next agency at the earliest opportunity stating current position, cleared Flight Level, next position and estimate and subsequent position. This also applies to flights using automatic position reports.

If unable to establish radio contact, flights may use SATCOM voice or satellite telephone to provide position reports.

| Oceanic Centre | Reykjavik | Santa Maria | New York | Ballygreen (Shanwick Aeradio) | Bodo |
| --- | --- | --- | --- | --- | --- |
| Telephone Number | 00 354 569 4141 | 00 351 296 820 438 00 351 296 886 042 (satellite link) | 001 631 468 1413 | 00 353 471 199 | 00 47 75521283 |
| SATCOM INMARSAT Short Code | 425101 | 426305 | 436623 | 425002 | 425702 |

Flights may request their flight dispatch offices to forward position reports, if sending position reports to multiple ATS Units or if otherwise unable to forward position reports.

Flights operating with a received and acknowledged oceanic clearance will be expected to continue in accordance with the last clearance issued unless otherwise advised by ATC.

Flights involved in level change should complete the manoeuvre as soon as possible in accordance with the clearance.

Flights making automatic position reports are required to make voice position reports whilst within the Bodo OCA  unless advised otherwise.

Communications with the next ATSU should be established at the earliest opportunity.  Where no contact with the next agency can be established, Shanwick radio should be contacted on HF for advice.

For flights approaching the Bodo OCA when the contingency is activated

Not in Receipt of an Oceanic Clearance

In the event that Bodo OACC must be evacuated, only aircraft with received and acknowledged oceanic clearances shall be permitted to transit Bodo OCA.

If aircraft are unable to obtain or acknowledge an oceanic clearance, flights must plan to re-route around the Bodo OCA or to land at an appropriate aerodrome. Request the appropriate re-clearance on the current frequency.

In receipt of an acknowledged Oceanic Clearance

Aircraft operating with a received and acknowledged ocean clearance should proceed in accordance with the clearance. Flights should not request changes in altitude, speed or route except for reasons of flight safety or to comply with the oceanic clearance.

Entering from another OCA

Flights within Reykjavik Oceanic Airspace, can anticipate a large re-route to avoid the Bodo OCA.

Reykjavik will issue advice on procedures to be followed.

Bodo  OCA Contingency Tracks,  FL280 or above

Latitude at 0oL

Domestic border/Landfall

80N/ TRO

72N/AND

70N/ BDO

69N/OGPAR

68N/BNN

67N/TRM

66N/ABADA

65N/VIG

64N/FLS

BODO OACC – CONTINGENCY ROUTE STRUCTURE

For activation within Bodo FIR

CHAPTER 3:  DETAILED CONTINGENCY PROCEDURES

EDMONTON FIR

CHAPTER 4:  DETAILED CONTINGENCY PROCEDURES

MAGADAN FIR

CHAPTER 5:  DETAILED CONTINGENCY PROCEDURES

MURMANSK FIR

CHAPTER 6:  DETAILED CONTINGENCY PROCEDURES

REYKJAVIK OCEANIC FIR/CTA

FIR FOR WHICH THE CONTINGENCY PLAN APPLIES

Reykjavik Oceanic FIR/CTA

FIRs WITH SUPPORTING PROCEDURES

None

NOTIFICATION PROCEDURES

In a limited service situation notification of any service limitations and traffic management measures will be promulgated to operators and adjacent ANSPs by NOTAM normally not later than 12 hours prior to activation or as soon as practicable in case of an unexpected service interruption.

In a no service situation the OACC is likely to have been evacuated.  As soon as possible after evacuation a contingency message will be sent by NOTAM and Iceland radio will advice aircraft within Reykjavik FIR/CTA. Adjacent centres will be advised by phone.

LIMITED SERVICE - PROCEDURES

Disruption of ground/air communication capability

Iceland Radio and Shanwick Radio provide joint communications for the communications area  comprising Reykjavik/Sondrestrom and Shanwick Oceanic Areas resulting in a virtual radio station for the North Atlantic from 45N to the North Pole.

Radio Operators work flights in either area, updating both Reykjavik and Shanwick Control Centres.

Joint Operations between Iceland Radio and Shanwick Radio increases the ability to provide a =normal‘ service with assistance from adjacent aeronautical stations.

Iceland radio provides communication services using HF and general purpose VHF. Reykjavik OACC provides DCPC VHF communications in the South sector and most of the East sector. Reykjavik OACC and Iceland radio are located in separate buildings located several kilometers apart. Disruption at one facility is therefore unlikely to affect the other facility and each will therefore serve as a backup for the other in cases of limited disruption of ground/air communication capability.

In case of failure of Iceland radio HF services, the HF service will be delegated to the radio stations in neighboring areas; Shanwick Radio, Gander Radio and Bodo Radio.

Disruption of ability to provide control services

Reykjavik will determine, co-ordinate and promulgate any necessary restrictions to meet the service limitation. Traffic in possession of a valid oceanic clearance will have priority over any other traffic. En route reclearance of such traffic will not be permitted except in emergency.

Traffic without a valid oceanic clearance may be subject to tactical traffic management measurements to meet the requirements of the service limitation.

Flight planning

Flight plans shall be filed and addressed to Reykjavik Oceanic Area Control Centre as well as to the appropriate adjacent ATS Units and IFPS, where applicable, in accordance with normal procedures.

Separation standards

Reykjavik will be responsible for ensuring the co-ordination and implementation of any additional separation requirements. In case of contingency track activation, there shall be at least 20 minutes separation between aircraft upon entry on the same contingency track and level.

Contingency tracks

Dependant on the nature of the service limitation, Reykjavik may promulgate and activate contingency tracks for use in addition to the NAT OTS. The contingency route structure detailed in this section will in most cases be implemented.

Air Traffic Flow Management

Reykjavik will co-ordinate any necessary traffic management measures where necessary with the Central flow Management Unit (CFMU). Such measures may include, but are not limited to, temporary capacity restrictions and tactical re-routing measures.

Reykjavik will co-ordinate these restrictions where necessary with adjacent ANSPs where they may affect the flow of traffic through these units airspace.

Communications

Aircraft shall not communicate directly with Reykjavik Oceanic Control on DCPC VHF except when instructed to do so or if in emergency. Position reporting within Reykjavik FIR/CTA will be with Iceland Radio or via ADS/CPDLC/FMS in accordance with normal procedures. Aircraft unable to contact Iceland Radio on HF Frequency shall call one of the following stations:

Shanwick Radio

Bodö Radio

Gander Radio

Montreal radio

Aircraft shall maintain continuous listening watch on the assigned frequencies.

Radar service

Radar service will be provided at ATS discretion. Aircraft are required to maintain their assigned discrete SSR Code while within Reykjavik FIR/CTA.

Responsibilities of adjacent ANSPs

The action required of adjacent ANSPs will vary dependant on the nature of the service limitation.  Where such action is not contained within the inter-centre Letters of Agreement (LOAs) the requirement will be promulgated within the initial failure and restrictions message.

NO SERVICE - PROCEDURES

Loss of ground/air communication capability

Iceland Radio and Shanwick Radio provide joint communications for the communications area  comprising Reykjavik/Sondrestrom and Shanwick Oceanic Areas resulting in a virtual radio station for the North Atlantic from 45N to the North Pole.

Radio Operators work flights in either area, updating both Reykjavik and Shanwick Control Centres.

Joint Operations between Iceland Radio and Shanwick Radio increases the ability to provide a =normal‘ service with assistance from adjacent aeronautical stations.

Iceland radio provides communication services using HF and general purpose VHF. Reykjavik OACC provides DCPC VHF communications in the South sector and most of the East sector. Reykjavik OACC and Iceland radio are located in separate buildings located several kilometers apart. Disruption at one facility is therefore unlikely to affect the other facility and each will therefore serve as a backup for the other in cases of limited disruption of ground/air communication capability.

In case of failure of Iceland radio HF services, the HF service will be delegated to the radio stations in neighboring areas; Shanwick Radio, Gander Radio and Bodo Radio.

Loss of ability to provide control services

Should Reykjavik OACC be evacuated the potential exists for a major disruption to Air Traffic Control service within the Reykjavik OCA.

The HF and general purpose VHF radio communications facilities for the Reykjavik Oceanic Centre are remotely located at the Iceland radio facilities in another part of Reykjavik city, and will therefore unlikely be affected.

In the event that Reykjavik ATCC is evacuated, the operations will be moved to Iceland radio and the provision of Air Traffic Services (ATS) within the Reykjavik FIR/OCA will be continued at that location as far as practicable.

As soon as possible after evacuation a contingency message will be sent by NOTAM and Iceland radio will advice aircraft within Reykjavik FIR/CTA. Adjacent centers will be advised by phone.

Contact information that may be used in the event of an emergency evacuation is provided.

Flight planning

Flight plans shall be filed and addressed to Reykjavik Oceanic Area Control as well as to the appropriate adjacent ATS Units and IFPS, where applicable, in accordance with normal procedures.

Separation standards

Reykjavik will be responsible for ensuring the co-ordination and implementation of any additional separation requirements. In case of contingency track activation, there shall be at least 20 minutes separation between aircraft upon entry on the same contingency track and level.

Contingency tracks

The contingency route structure detailed in this section will be implemented.

Air Traffic Flow Management

Reykjavik will co-ordinate any necessary traffic management measures where necessary with the Central flow Management Unit (CFMU). Such measures may include, but are not limited to, complete closure of the airspace, temporary capacity restrictions and tactical re-routing measures.

Reykjavik will co-ordinate these restrictions where necessary with adjacent ANSPs where they may affect the flow of traffic through these units airspace.

Communications

HF congestion is likely. Communications should be kept to a necessary minimum. Unnecessary routing, flight level and speed changes will not be issued.

Communications and Position reporting within Reykjavik FIR/CTA will be with Iceland Radio or via ADS/CPDLC/FMC. Aircraft unable to contact Iceland Radio on general purpose VHF or HF Frequency shall call one of the following stations:

Shanwick Radio

Bodö Radio

Gander Radio

Montreal radio

Aircraft shall maintain continuous listening watch on the assigned frequencies.

Radar service

Radar service will not be provided. Aircraft are nevertheless required to maintain their assigned discrete SSR Code while within Reykjavik FIR/CTA.

Responsibilities of adjacent ANSPs

Other ATSUs will provide guidance as far as possible in the circumstances.

FLIGHT CREW AND OPERATOR PROCEDURES

For flights within the Reykjavik OCA

The procedures outlined below are to be used as guidance for pilots in the immediate aftermath of a sudden withdrawal of the ATC service as described above.

On receipt of the contingency message pilots are requested to broadcast to other flights on 121.5 and 123.45. A listening watch on these frequencies must be maintained.

Reykjavik OACC will endeavor to provide a limited ATC service through Iceland radio as soon as possible after evacuation commences.

Flights operating with a received and acknowledged oceanic clearance will be expected to continue in accordance with the last clearance issued unless otherwise advised by ATC. Aircrew shall use extreme caution and use all available means to detect any conflicting traffic.

Flights should remain in/establish communications with Iceland radio. Flights unable to contact Iceland radio should establish communication with the next agency at the earliest opportunity stating current position, cleared flight level, next position and estimate and subsequent position. This also applies to flights using automatic position reports (ADS/CPDLC/FMC) as these reports may not have been received by the next agency.

When flights making automatic position reports are notified of a Reykjavik evacuation they must revert to voice position reporting until clear of Reykjavik OCA, or notified otherwise. Pilots of FANS1/A equipped flights should note that they may be asked to log-on to the next agency while within the Reykjavik OCA, they should not initiate this action until instructed to do so.

If unable to establish radio contact, flights may use SATCOM voice or satellite telephone to provide position reports.

| Oceanic Centre | Gander | Santa Maria | New York | Ballygirreen (Shanwick Aeradio) |
| --- | --- | --- | --- | --- |
| Telephone Number | 001 709 651 5207 | 00 351 296 820 438 00 351 296 886 042 (satellite link) | 001 631 468 1413 | 00 353 471 199 |
| SATCOM INMARSAT Short Code | 431613 | 426305 | 436623 | 425002 |

Flights may request their flight dispatch offices to forward position reports, if sending position reports to multiple ATS Units or if otherwise unable to forward position reports.

For flights approaching the Reykjavik OCA when the contingency is activated

Not in Receipt of an Oceanic Clearance

In the event that Reykjavik OACC must be evacuated, only aircraft with received and acknowledged oceanic clearances are permitted to transit Reykjavik OCA.

If unable to obtain or acknowledge an oceanic clearance, flights shall re-route around the Reykjavik OCA or land at an appropriate airfield. The adjacent areas will issue advice on procedures to be followed.

In receipt of an acknowledged Oceanic Clearance

Aircraft operating with a received and acknowledged oceanic clearance can, at pilot‘s discretion, continue, but must expect a limited ATC service within the Reykjavik OCA. Aircrew shall use extreme caution and use all available means to detect any conflicting traffic. Due to the remote location of the HF service provider communications will be available through Iceland radio.

However, due to the uncertainty surrounding the contingency situation pilots are strongly advised to comply with the procedures detailed above for flights not in receipt of an oceanic clearance even if they are in receipt of an acknowledged Oceanic clearance.

REYKJAVIK OACC – CONTINGENCY ROUTE STRUCTURE

For activation within Reykjavik OCA

In a limited service contingency situation Reykjavik OACC may promulgate contingency tracks in addition to the published OTS. A set of routes, titled ICECON Tracks, have been established for this purpose. Promulgation of the tracks will be via AFTN.

It is mandatory to flight plan on the ICECON tracks during the periods detailed below. The contingency tracks must be flight planned as if they were random route tracks (detailing each waypoint in the flight plan).

IT IS ESSENTIAL FOR AVIATION SAFETY THAT ALL PILOTS UNDERSTAND AND COMPLY WITH THE PROVISIONS OF THIS CONTINGENCY PLAN.

Flight level changes for en-route aircraft should not be expected within Reykjavik FIR/CTA.

Random flights at directional levels will be accepted at FL 290 and below as well as FL 410 and above, however, flow restrictions may be imposed.

Radar service will be provided at ATS discretion.

Ambulance and SAR flights will be dealt with on individual bases.

Day Tracks

The following DAY TRACKS will be effective on entry into Reykjavik FIR/CTA from 0930 to 1800 except A, B and C, which will be activated as part of the NAT OTS.

A  BARKU - RATSU - 63N020W - 64N030W - 64N040W - 63N050W - 61N060W – MIBNO - RODBO

Westbound FL340/350/360

Eastbound FL380

B  ATSIX - 62N020W - 63N030W - 63N040W - 62N050W - 60N060W – PEPKI - LOPVI

Westbound FL340/350/360/370/380/390

Eastbound NIL

C BALIX - 61N020W - 62N030W - 62N040W -61N050W – MOATT - LOMTA

Westbound FL340/350/360/370/380/390

Eastbound NIL

ICECON 8 BESGA - MATIK - 62N010W - 64N020W - 66N030W - 67N040W - 67N050W - DARUB

Westbound FL340/350/360

Eastbound FL390

ICECON 10 GONUT - 63N010W - 65N020W - 67N030W - 69N040W - 70N050W - 70N060W - ADSAM

Westbound FL340/350/360

Eastbound FL370/380/390

ICECON 14 LIRKI - 66N005W - 71N010W - 7630N020W - 81N040W - ALERT

Westbound FL340/350/360

Eastbound NIL

ICECON 16 73N00W - 79N010W – 82N020W - PELRI

Westbound FL340/350/360

Eastbound FL310

ICECON 18 80N00W - 85N020W - OVBES

Westbound FL340/350/360

Eastbound FL310

ICECON 20 EXITA - 78N020W - 7830N040W - THT - LENIM

Westbound FL320/330

Eastbound FL370/380

ICECON 22 IPTON - 63N010W - 63N020W - 64N030W - 64N040W - 63N050W - 61N060W – MIBNO – RODBO

Westbound FL330

Eastbound NIL

ICECON 24 GUNPA - 62N010W - 62N020W - 63N030W - 63N040W - 62N050W - 60N060W – PEPKI – LOPVI

Westbound FL330

Eastbound NIL

ICECON 24A GUNPA - 62N010W - 62N020W - 62N030W - 62N040W - 61N050W – MOATT – LOMTA

Westbound FL330

Eastbound NIL

Contingency day tracks effective on entry into Reykjavik FIR/CTA from 0930 to 1800 except A, B and C, which will be activated as part of the NAT OTS.  Refer to the text above for flight level allocation on the tracks.

Night Tracks

NIGHT TRACKS will be effective on entry into Reykjavik FIR/CTA from 2300 to 0600 except ICECON 11 and 13 which will become effective from 0100 until 0600 at 30W.

ICECON 7 ADSAM - 70N060W - 70N050W - 69N040W - 67N030W - 65N020W - 63N010W - GONUT

Eastbound FL340/350

Westbound FL330

ICECON 9 DARUB - 67N050W - 66N040W - 65N030W - 64N020W - 62N010W – MATIK - BESGA

Eastbound FL340/350

Westbound FL330

ICECON 11 62N040W - 63N030W - KEF - 64N020W - 63N010W -IPTON

Eastbound FL360/370/380

Westbound NIL

ICECON 13 61N040W - 62N030W - 62N020W - 62N010W - GUNPA

Eastbound FL360/370/380

Westbound NIL

ICECON 13A 61N040W - 62N030W - 62N020W - RATSU - BARKU

Eastbound FL360/370/380

Westbound NIL

ICECON 15 ALERT- 81N040W - 7630N020W - 71N010W - 66N005W - LIRKI

Eastbound FL350/390

Westbound FL340

ICECON 17 PELRI - 82N020W - 79N010W – 73N000W

Eastbound FL350/360/370

Westbound FL310/340

ICECON 19 OVBES - 85N020W – 80N000W

Eastbound FL350/360/370

Westbound FL310/340

ICECON 21 LENIM - THT - 7830N040W - 78N020W - EXITA

Eastbound FL330/380

Westbound FL320

Eastbound traffic will not be permitted to route from Shanwick or Scottish airspace into Reykjavik airspace unless at FL 270 and below or FL 390 and above.

Contingency night tracks effective on entry into Reykjavik FIR/CTA from 2300 to 0600 except ICECON 11 and 13 which will become effective from 0100 until 0600 at 30W.  Refer to the text above for flight level allocation on the tracks.

Tracks available 24 hours

Iceland - inbound and outbound

INBOUND

RATSU - ALDAN - KEF

Westbound FL320

GUNPA - 63N010W - ING - KEF

Westbound FL 310

61N040W - 62N030W - DROMI

Eastbound FL 290/310

BIAR – IPTON 64N010W ES AKI

BIEG – IPTON 64n010W ES

Westbound FL300

OUTBOUND

KEF - BREKI - RATSU- BARKU

Eastbound FL 310 - To be level by BREKI

Radial 098 until FL 320 - 63N010W - GUNPA

KEF - EMBLA - 63N030W – 62N040W

Westbound FL 320

BIAR – AKI ES 64N010W IPTON

BIEG – ES 64N010W IPTON

Eastbound FL290

Faeroes Islands - inbound and outbound

INBOUND

VALDI - BUREM

Westbound FL 280

OUTBOUND

G11 - GONUT

Eastbound FL 290

Söndreström - inbound and outbound

INBOUND

ICECON 12 IPTON - 64N010W - 66N020W - 67N030W - 67N040W - 67N050W - SF

Westbound FL 310

EPMAN - SF

Eastbound FL 300

6000N05630W - KU - SF

Northbound FL 320

SAVIS - TOMAS - UP - DISGU - SF

Southbound FL 320

OUTBOUND

ICECON 12 67N050W - 67N040W - 67N030W - 66N020W - 64N010W - IPTON

Eastbound FL 320

EPMAN

Westbound FL 320

KU - 6000N05630W - PRAWN

Southbound FL 310

DISGU - UP - TOMAS - SAVIS - THT

Northbound FL 310

Radar service will be provided by ATC Söndreström.

Thule - inbound and outbound

INBOUND

DISGU - UP - TOMAS - SAVIS - THT

Northbound FL 310

JULET - LIPSI - THT

ALL LEVELS to LIPSI, after LIPSI FL290 at or below

OUTBOUND

SAVIS - TOMAS - DISGU - SF

Southbound FL 320, not ABV FL 310 until after SAVIS

LIPSI - JULET

ALL LEVELS, not ABV FL 310 until after LIPSI

Radar service will be provided by Thule RAPCON.

Traffic via ABERI

CANEL 73N060W 79N055W 84N040W ABERI

Eastbound FL300

Westbound NIL

Contingency tracks available 24 hours.  Refer to the text above for flight level allocation on the tracks.

LONG TERM CONTINGENCY ARRANGEMENTS

In development.

PROCEDURES BY ADJACENT AREAS IN EVENT OF REYKJAVIK EVACUATION

NONE

CONTACT DETAILS - REYKJAVIK OACC

Reykjavik OACC

Reykjavik Shift Manager (07:00-23:00)

+354 424 4343

atc@isavia.is

Reykjavik Shift Manager INMARSAT Satellite Phone (07:00-23:00)

+874 763 585 245

Reykjavik Shift Manager Iridium Satellite Phone (07:00-23:00)

+881 631 450 347

Supervisor (23:00-07:00)

+354 424 4141

Reykjavik OACC Telefax

+354 424 4200

North Sector primary commercial/1st backup

+354 424 4264

West Sector primary commercial/1st backup

+354 424 4263

South Sector primary commercial/1st backup

+354 424 4262

South Sector domestic operations commercial/1st backup

+354 424 4261

All Sectors 2nd backup

+354 568 3033

All Sectors 3rd backup

+354 568 3035

Reykjavik ARCC

+354 424 4323

System Operators and Flight Data Specialists

+354 424 4265

System Operators and Flight Data Specialists, Iridium Satellite Phone

+881 621 434 042

Systems Department

+354 424 4328

Systems Department, Mobile

+354 897 8483

Manager Reykjavik OACC

Ms. Þórdís Sigurðardóttir

+354 424 4140

+354 699 8504 (mobile)

thordis@isavia.is

Deputy Manager Reykjavik OACC

Mr. Árni Baldursson

+354 424 4183

+354 615 2565 (mobile)

arni@isavia.is

Manager Airspace and Procedures

Mr. Sigurleifur Kristjánsson

+354 424 4123

+354 897 0336 (mobile)

silli@isavia.is

Gannet – Iceland Radio

+354 563 6502

supervisor@gannet.is

EVACUATION MESSAGES - REYKJAVIK OACC

AFTN

DD BGGLZQZX BGSFYFYX CYQXYFYX CZEGZQZI CZQXZQZX

EGGXZQZX EGPXZQZX EIAAYFYX ENOBZQZX

XXXXXX BICCYFYX

SVC

Reykjavík Centre has been evacuated, personnel is on its way to BICC.

Telephone numbers: +354 568 4600, +354 568 4601.

ICELAND RADIO ON VOICE

Emergency evacuation of Reykjavik Centre is in progress. No air traffic control service will be provided by Reykjavik. Use extreme caution and monitor this frequency, emergency frequencies and air to air frequencies.

CHAPTER 7:  DETAILED CONTINGENCY PROCEDURES

SONDRESTROM FIR

PART III -   CONTINGENCY SITUATIONS AFFECTING MULTIPLE FIRs

SCOPE OF THE PLAN

This part of the Contingency Plan considers:

Action taken by ATSUs in the event of Arctic airspace being contaminated by volcanic ash.

- END -