ANCHORAGE CENTER TRACK ADVISORY PROCEDURES

This section provides specific procedures for westbound flights planning to enter the Russian Cross Polar and Trans East (CP/RTE) route system.

Dispatchers must transmit daily TKF options to Anchorage ARTCC (ZAN) for all westbound flights planning to enter the CP/RTE tracks according to the following table:

| Trackload Program | File TKF | Compile and Send GRL | Negotiation Period | Archive Time |
| --- | --- | --- | --- | --- |
| 1700Z-0700Z | 0900Z-1430Z | 1430Z | 1430Z-0000Z | 0000Z |
| 0700Z-1700Z | 0015Z-0430Z | 0430Z | 0430Z-0900Z | 0900Z |

1700 UTC – 0700 UTC: GRL is compiled and sent to users at 1430 UTC.

1430 UTC – 0000 UTC: The negotiation period.

0000 UTC – 0700 UTC: All TKF requests must be called in to ZAN - 907-269-1104.

0700 UTC – 1700 UTC: GRL is compiled and sent to users at 0430 UTC.

0430 UTC – 0900 UTC: The negotiation period.

0900 UTC – 1700 UTC:  All TKF requests must be called in to ZAN - 907-269-1104.

TKFs for CP/RTE routes requiring a metric altitude use the following designator:

| FL = Meters | Designator |
| --- | --- |
| FL280 = 8600 Meters | 86 |
| FL310 = 9600 Meters | 96 |
| FL350 = 10600 Meters | 106 |
| FL380/390 = 11600 Meters | 116 |
| FL430 = 13100 Meters | 131 |

Required trackload minima and useable altitudes at the gateway fixes:

| Gateway Fix | In-trail Spacing | Useable Altitudes |
| --- | --- | --- |
| DEVID | 10 minutes | 310, 350, 390 |
| RAMEL | 10 minutes | 320, 340, 360 |
| NIKIN | 10 minutes | 320, 340, 360 |
| ORVIT | 10 minutes | 320, 340, 380 |
| PILUN | 10 minutes | 320, 340, 380 |
| LISKI | 10 minutes | 320, 340, 380 |
| FRENK | 10 minutes | 86, 96, 106, 116, 131 |
| KUTAL | 10 minutes | 86, 96, 106, 116, 131 |
| VALDA | 15 minutes | 86, 96, 106, 116, 131 |
| ERNIK | 10 minutes | 86, 96, 106, 116, 131 |
| RUSOR | 10 minutes | 86, 96, 106, 131 |
| MARCC | 15 minutes | 86, 96, 106, 131 |
| BAMOK | 10 minutes | 86, 96, 106, 131 |

NOTE: For flights transiting the RTE/CP routes, dispatchers/flight crews are reminded that the GRL is a tool used to assist dispatchers in their flight planning and to provide air traffic facilities early intent information.  Tactical sequencing may be utilized to ensure required in-trail spacing for aircraft at the same altitude.  This may include speed control and/or vectors to accomplish the required spacing.

NOTE:  The TA program software does not consider aircraft performance characteristics when assigning slots on the GRL. Therefore, even though a flight may arrive at the GRL fix “on-time”, the flight may not be assigned the GRL altitude.

NOTE:  Until time of departure, dispatchers should update the information provided on the GRL on all planned flights by submitting a new TKF request as outlined in this guide.  To provide for more dynamic system awareness, operators may revise their GRL information including any changes to the wheels-up time, requested altitude, or estimate at the gateway fix, at any time after departure.  If further assistance is required, the customer may contact the ATCSCC or ZAN TMU as outlined below for assistance.

- Call the ATCSCC International Operations Position at +1 703-925-5312 with any issues pertaining to the CP routes DEVID, RAMEL, NIKIN, and ORVIT.

- Call ZAN TMU at +1 907-269-1840 with any issues pertaining to the RTE routes PILUN and south.

Example of ZAN TKF message format:

TKF

Callsign.2000.KSFO.RJAA.106.VALDA.0130.15.96.VALDA.0130.20.REM. DISPATCHER REMARKS GO HERE FOR THE FLIGHT

After the GRL is published, TKF messages will be processed automatically without TMU intervention during the negotiation period.

Transmit messages to the ZAN TA program via ARINC or NADIN/AFTN using the following addresses:

ARINC:	ANCDPXA

# NADIN/AFTN:	PANCZOZD

## 1430 - 0000 UTC & 0430 - 0900 UTC: Slot availability message function is active.

REVISED RESERVATION FOR UAL1 DEPARTING KSFO HAS BEEN APPROVED

TRACK=VALDA  ALTITUDE=106  DEPARTURE TIME=2040  VALDA TIME=0211

## PROCEDURES FOR COMPUTER OR COMMUNICATION

## LINK FAILURES AT ANCHORAGE CENTER

In the event that the TA system fails, the following procedure will be used until the service is restored:

Users will be notified to forward flight request via alternate AFTN address as listed on the Quick Reference sheet, via FAX, or via phone. Please include your name and phone number with your flight request.

After 1430 UTC ZAN will compile a GRL.  Dispatchers should not expect to receive the GRL until 1530 UTC.

Any changes or additions after 1430 UTC must be made by phone (907-269-1840).

In this manual mode of TA, the TKL function to request additional or updated GRLs is not available.

NOTE: There will not be a GRL compiled for flights between 0700 and 1700 UTC.

QUICK REFERENCE SHEET FOR TRACK ADVISORY

## FOR ANCHORAGE CENTER

Normal ARINC address:        						ANCDPXA

Normal NADIN/AFTN address:              				PANCZOZD

Alternate NADIN/AFTN address if line is down: 			PAZAZQZX

Anchorage Center TRACK ADVISORY PHONE NUMBER:  	907-269-1840

Anchorage Center TMU FAX PHONE NUMBER:          		907-269-1343

DISPATCHERS SHOULD USE THE RELEASE TIMES, ALTITUDES AND ROUTINGS FROM THE GRL FOR FLIGHT PLANNING PURPOSES.

DISPATCHER MESSAGES:

TKF    - Two or more line flight request.

TKC    - Cancel flight request.

TKL    - Request a GRL.

TRACK ADVISORY DAILY PROCEDURES CHART

FOR ANCHORAGE CENTER

ANCHORAGE ARTCC TMU

0015-0430Z

TA Processor Accepts

Gateway Requests

0900-1430Z

TA Processor Accepts

Gateway Requests

0430Z TA Processor Builds Gateway Reservation List

1430Z TA Processor Builds Gateway Reservation List

0430-0900Z and

1430-0000Z

TA Processor Updates List Automatically

Automated requests are coordinated before approval.  Automated messages returned to users.

0000-0700Z and

0900-1700Z

Call Anchorage TMU at (907) 269-1104 for Gateway Reservation.

PLANNING PHASE

Gateway Requests

Two Separate Request Periods

##### AIRLINE

DISPATCH

OFFICES

Gateway Reservation List

NEGOTIATION PHASE

### Change Request

New Reservation

### Change Request

New Reservation