Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France 2-5 November 2010)

Agenda Item 4: Provide Status on CPWG/9 Actions

- TRACK LOADING OF FIXES ENTERING THE MURMANSK, MAGADAN, AND PETROPAVLOVSK-KAMCHASTKY FLIGHT INFORMATION REGIONS FROM THE ANCHORAGE FIR

- (Action Item #CP01-18)

(Presented by Federal Aviation Administration)

SUMMARY This paper presents information regarding track loading of fixes entering the Murmansk, Magadan, and Petropavlovsk-Kamchatsky Flight Information Regions (FIRs) from the Anchorage FIR. It also provides information on how traffic flows are managed to these fixes.

- Introduction

1.1	Westbound traffic from North America to Russia must transition between two very different environments.  Close cooperation between users and air navigation service providers (ANSPs) along with advances in technology have reduced limitations on aircraft transitioning to 169W.  After passing the fixes on the Russian Flight Information Region (FIR) boundaries, aircraft are limited to fixed routes and, until November 2011, will operate under conventional vertical separation minima using meters.  It has been an ongoing process to determine the best way to get aircraft from a relatively unrestricted environment to specific points in space with an expectation a preferred altitude at those points.

1.2	A copy of the current Track Advisory Users Guide (TAUG) has been attached to this paper for easy reference and for planning for airlines and service providers. (See Appendix A)

- Discussion

2.1	The northernmost polar fixes (ORVIT, NIKIN, RAMEL, and DEVID) are track loaded and managed by the Federal Aviation Administration (FAA) Air Traffic Control System Command Center (ATCSCC).  The ATCSCC coordinates with Canadian traffic managers to create spacing to these fixes.  The polar fixes are track loaded 10 minutes in trail at each altitude.  DEVID is loaded at flight level (FL) 310, FL350, and FL390 for transfer from Anchorage Air Route Traffic Control Center (ZAN) to Murmansk Area Control Centre (ACC).  Magadan ACC receives aircraft loaded for three metric altitudes (9600m, 10,600m, and 11,600m) at NIKIN, RAMEL and ORVIT at 10 minute intervals for each altitude.

2.2	ZAN track loads and manages the fixes into the Russian Trans East (RTE).  The two northernmost of these fixes are PILUN and LISKI.  At CPWG/9 Magadan ACC agreed to take and give transfers in feet at these fixes.  This has allowed for the easing of NOTAM restrictions in this region and has made more altitudes available westbound.  These tracks are loaded at 10 minute intervals for each altitude available.

2.3	ZAN also track loads and manages the remaining seven fixes (FRENK, KUTAL, VALDA, ERNIK, RUSOR, BESAT and BAMOK).  Aircraft at these fixes are transferred at metric levels.  Aircraft are track loaded in 15 minute intervals for each altitude at VALDA and BESAT, and 10 minute intervals at each of the other fixes (See CPWG/10 WP/09 on reduced restrictions).  Radar and VHF coverage to the FIR boundary has provided greater flexibility in creating spacing and accommodating altitude requests at most of these fixes, with the exception of BAMOK and BESAT.

- Recommendation

- The meeting is invited to:

- note the information presented in this paper; and

- use this information, and that included in the TAUG, to better plan and coordinate flights between ZAN and Russia.