Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France, 2-5 November 2010)

Agenda Item 4: Provide Status on CPWG/9 Actions

TACTICAL RE-ROUTES PRIOR TO ENTERING RUSSIAN AIRSPACE

(Action Item #06-12)

(Presented by International Air Transport Association)

SUMMARY This working paper presents information for the Group’s consideration in reference to Action Item CP06-12 “Tactical Re-routes prior to entering Russian airspace.” A paper trial was conducted on April 7, 2010 between United Airlines, the Anchorage Center Traffic Management Unit, and the Russian Main Air Traffic Management Center (MATMC) in Moscow. A live test was attempted on October 8, 2010 between United Airlines, Anchorage Center, and the MATMC in Moscow. The results of this test are provided in this paper.

1.	Introduction

# A paper test of a tactical re-route from Anchorage Flight Information Region (FIR) into Russia was conducted on April 7, 2010 and reported at CPWG/9. As a follow up to the paper test in April, a live test was attempted on October 8, 2010 between the same participants, (United Airlines (UAL), the Main Air Traffic Management Center (MATMC), and Anchorage Air Route Traffic Control Center (ZAN)), with UAL881, a Boeing 747-400 operating from Chicago to Tokyo.

# Utilizing lessons learned from the April “paper” test, a live tactical re-route was planned, rather than a second paper test.

# Unfortunately the communications between UAL Dispatch in Chicago and the MATMC did not function properly and the tactical re-route was not executed. This paper updates the meeting on the issues concerning the live test.

- Discussion

# 2.1	WP/17 was presented at CPWG/9 as a follow up to the paper test of a tactical re-route from the Anchorage FIR into Russia on April 7, 2010. As stated in WP/17, tactical reroutes are conducted in many regions of the world with minimal complexity between the operator and the various air traffic service providers. Generally a re-route is either requested by the pilot when the filed routing and/or filed flight level are not available due to traffic. The Air Navigation Service Provider (ANSP) may also initiate a request to the operator, through the Airline Operations Center (AOC) or direct to the pilot, when a conflict of traffic will occur and a tactical re-route could eliminate that conflict.

2.2	A “simplistic” example would be a flight operating from Washington to Tokyo, within the

Anchorage FIR, with a flight plan (FPL) to operate on North Pacific (NOPAC) Air Traffic Services (ATS) route R220. Due to traffic the flight may have to stay at an undesirable flight level, and therefore the pilot may request a tactical re-route to NOPAC ATS route R580. Generally there is coordination between the AOC and the flight to evaluate options and to amend the flight release. The actual re-route is then provided by the Anchorage Center controller and the FPL information is forwarded by ATS Interfacility Data Communication (AIDC) to Japan (in this example).

2.3	There are several factors that make the same process more complex when a flight is approaching Russian airspace and the need for a tactical re-route exists:

# The transfer of control must be completed within 30 minutes of the Russian entry fix;

# Russia requires an initial route change message, (new FPL), to be sent to MATFMC for their evaluation prior to a reroute. This is not consistent with global practices for operators; and

# Evaluation of the re-route is important for the operator, due to onward routings within and or out of Russia to the west, which must be considered.

# 2.4	The paper test of a tactical re-route was accomplished on April 7. 2010, using UAL803 from Washington Dulles (KIAD) to Tokyo Narita (RJAA), as the test flight.

# 2.5	The process used in the April paper test would have worked for an actual tactical re-route, although it was somewhat labor intensive and time consuming. It is believed that AIDC may in fact alleviate most of this process, and reduce the transfer of control time.

2.6	An actual live test was planned for October 8, 2010 using the lessons learned from the April paper test, with the following scenario:

- UAL881, a Boeing 747-400 from Chicago to Tokyo would be the candidate for the tactical reroute.

- The flight was filed (FPL) via OME R338 ROCET B241 RUSOR B241 PAKLI

- The intended tactical reroute would be to ERNIK B240 or BESAT G583

2.7	UAL Dispatch initiated discussions with Anchorage Traffic Management Unit (TMU) shortly after the UAL881 crossed the 141W FIR “CHAPO” at 2216UTC.

2.8	The estimated time over RUSOR was 0025UTC with a goal of making the tactical reroute no less than 30 minutes prior to entry.

2.9	The UAL dispatcher requested a reroute to B240 ERNIK as the optimum routing. This would require the transfer of control to be accomplished prior to crossing IDROD at 2350UTC and crossing ERNIK at 0009UTC. ZAN TMU advised that UAL889 would be crossing ERNIK at 0006UTC at 10600Meters. With the time and flight level conflict between UAL881 and UAL889, the dispatcher and ZAN TMU agreed that the reroute should be planned via G583 with entry at BESAT at 0040UTC.

2.10	A SITA message, (complete FPL from Chicago to Tokyo), was sent by the UAL dispatcher to MOWYWYA at 2259UTC requesting approval for the tactical reroute via BESAT G583.

2.11	The UAL dispatcher attempted to contact the MATMC via telephone 7 495 601 0776 within ten minutes of sending the message. The telephone connection failed several times, either with a fast busy signal, or a drop off.

2.12	After several conversations back and forth between the UAL dispatcher and Gene Cameron, UAL enlisted the help of Edgar Vaynshteyn.

2.13	Edgar was able to make initial contact with the MATMC, however they had not received the SITA FPL reroute message.

2.14	The UAL dispatcher resent the message and a follow up call from Edgar to ATMC advised the FPL still was not received. Gene then used the Aeronautical Fixed Telecommunications Network (AFTN) address of UUUWZDZX, which ATFMC confirmed was received.

2.15	Verbal approval for the reroute was given to the UAL dispatcher via the Edgar/ATMC telephone conversation at approximately 2345UTC.

2.16	The UAL dispatcher called the ZAN TMU with the approval from MATMC and initiated a refiling of a complete FPL (Chicago to Tokyo) via OME R338 MARCC G583 BESAT. The FPL was sent to PAZAZQZX and UUUWZDZX and RJJJZQZX.

2.17	UAL had thought that the reroute was a success, even though the process was very labor intensive and time consuming between UAL and the MATMC.

2.18	At approximately 0005UTC the ZAN TMU called the UAL dispatcher to advise that the aircraft was on the original flight planned route via B241 RUSOR because the transfer of control was not received in time and the aircraft was in non-radar airspace.

2.19	The failure of the exercise was due to the following reasons:

- SITA FPL message with reroute was not received at the MATMC;

- The telephone contact between UAL dispatch and the MATMC would not function properly;

- The AFTN FPL reroute message was received, but after considerable time loss;

- The approval from MATMC to UAL dispatch required further coordination, and time, between UAL dispatch and ZAN TMU;

- ZAN stated that UAL881 had not requested the reroute. (This was a breakdown in process since the UAL dispatcher had advised the flight that the reroute approval was received from Russia and was being coordinated with the ZAN TMU). UAL881 did not request the reroute because they thought it was initiated between UAL dispatch and ZAN TMU; and

- By the time the FPL was received from ZAN and delivered to Ocean21, the aircraft was out of radar coverage and less than 30 minutes to RUSOR, and therefore the flight did not receive a reroute.

2.20	The following suggestions should be considered prior to another live test of a tactical re-route:

- Review and test telephone numbers, and both SITA and AFTN addresses at MATMC;

- An email exchange with UAL dispatch and the MATMC was successful on the day after the test. Email may not be monitored as closely as AFTN or SITA, and would require more time than a telephone call, but may be an alternative means of communications in this process;

- Consider having the Anchorage TMU initiate contact with MATMC for approval “after” a tactical reroute has been discussed between ZAN TMU and the AOC involved. (For non AOC operators, this would be after ZAN controller and pilot-in-command discussions);

- Evaluate the requirement for AOC to send an FPL to either PAZA or MATMC, with a reroute, if the aircraft is in the Anchorage FIR;

- Consider reducing the time required for transfer of control for flights into Russia from the Anchorage FIR, currently 30 minutes; and

- Determine the affect of AIDC capability and establish a timeline for its application between ZAN and MATMC.

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this Working Paper;

  - consider the suggestions outlined in 2.20; and

  - continue to work on the process and conduct another live test or tests prior to CPWG/11.

.