Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France 2-5 November 2010)

Agenda Item 4:	Provide Status on CPWG/9 Actions

- STATUS OF THE IMPLEMENTATION OF

- 50 NM LATERAL SEPARATION IN THE ARCTIC

- (Action Item CP03-30)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents a status update on the implementation of 50 nautical miles (NM) lateral separation between operators and aircraft authorized for Required Navigation Performance (RNP) 10 or RNP 4 in the Arctic Flight Information Regions (FIRs).

- Introduction

- Following the discussions of the Second Meeting of the Trans-Regional Airspace and Supporting ATM Systems Steering Group (TRASAS/2), CPWG/5 agreed to include the TRASAS 2/2 conclusion as part of its new CPWG Work Program. The CPWG identified a need to harmonize separation standards and improve efficiencies across the polar region.

- The Federal Aviation Administration (FAA), NAV CANADA and ISAVIA reported to the CPWG/7 that they had formed a team to explore implementing 50 nautical miles (NM) lateral separation between operators and aircraft authorized for Required Navigation Performance (RNP) 10 or RNP 4 in Anchorage Arctic Flight Information Region (FIR) and Canadian Minimum Navigation Performance Specifications (CMNPS) airspace.

- This information paper presents an update on the work conducted by the team.

- Discussion

  - The United States and Canada are planning to implement a reduction of the lateral separation standard within the Control Areas (CTAs) of Anchorage Arctic FIR and Edmonton FIR/CTA.  The lateral separation applied in the Anchorage Arctic FIR will be reduced from the existing 90 NM standard to 50 NM between aircraft authorized for RNP 10 or RNP 4 operations.  Once fully implemented, the change will harmonize the application of 50 NM lateral separation across the Pacific CTA/FIRs, the Anchorage Arctic FIR and Edmonton FIR/CTA.

  - A Safety Risk Management (SRM) Panel composed of representatives from FAA and NAV CANADA, and facilitated by CSSI Inc., was convened to ensure that hazards are identified and unacceptable risk is mitigated and accepted prior to any changes to the United States (US) National Airspace System (NAS).  Applying the principles of the SRM process, the panel identified no high or medium risk hazards and only two low risk hazards.  It was determined by the SRM Panel that the reduction of lateral separation in the Anchorage Arctic FIR could be safely implemented.

  - A Safety Assessment conducted by the FAA William J Hughes, Air Traffic Control Technical Center was completed in November 2009.  This safety assessment provided information that showed that RNP 10 would be a sufficient requirement for safe operations under the initial introduction of 50 NM lateral separation in the Arctic RNP 10 area.  The 95 percent upper confidence estimate of lateral occupancy results were used to prepare the collision risk estimate, which should account for demand well into the future for this airspace.  Under the assumptions outlined in the safety assessment and using the collision risk model parameter estimates, the estimate of lateral collision risk was determined to be 0.429 x 10-9 fatal accidents per flight hour due to the loss of planned lateral separation for the Arctic RNP 10 airspace when a lateral navigational performance of 7 NM/95% containment was used.

  - When it was assumed that aircraft population performance just met RNP 10 performance (10 NM/95% containment), the resulting estimate of lateral collision risk was 0.528 x 10-9 fatal accidents per flight hour due to the loss of planned lateral separation.

  - Both of these values satisfied the ICAO-endorsed target level of safety (TLS) value applicable to the lateral separation standard, 5 x 10-9 fatal accidents per flight hour due to the loss of planned lateral separation.    This assessment showed that given prevailing conditions and expected performance, the Arctic RNP 10 separation reduction would meet international guidelines for implementation.

  - The Safety Risk Management Document (SRMD) has been submitted to FAA approval authorities, and is awaiting approval.  The North American Regional Supplementary Procedures (ICAO Doc 7030) amendment awaiting approval of the ICAO North American, Central American and Caribbean (NAAC) Regional Office.  The Arctic RNP 10 Separation Reduction Project website can be found at: http://www.faa.gov/about/office_org/headquarters_offices/ato/service_units/enroute/oceanic.  The Operational Policy and Procedures have been added to the Domestic and International NOTAM booklet, Part 3, Section 2 (International Oceanic Airspace Notices).

  - The planned implementation date is 18 November 2010 for the Anchorage Arctic FIR and in 2011 for Edmonton FIR/CTA CMNPS airspace.  NAV CANADA will not be implementing simultaneously with the FAA due to installation of Canadian Automated Air Traffic System (CAATS) at Edmonton Area Control Centre.

- Recommendation

- The Meeting is invited to note the information provided in this paper.