Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France 2-5 November 2010)

Agenda Item 5:	2011-2012 Cross Polar Work Program

- IMPROVING EFFICIENCY WITH THE IMPLEMENTATION

- OF REDUCED LONGITUDINAL SEPARATION IN ARCTIC AIRSPACE

(Presented by the Federal Aviation Administration)

SUMMARY This paper requests that the air navigation service providers responsible for the Polar airspace begin to reduce longitudinal separation. Application of 50 NM longitudinal distance based separation (D50) and 30 NM longitudinal separation are approved in ICAO DOC 4444.

- Introduction

1.1	The implementation of reduced horizontal separation based on Performance Based Navigation (PBN) specifications brings about an improvement in safety, airspace capacity, flight efficiency and environmental benefits

- Discussion

- Over the Pacific, 50 NM lateral separation standard based on aircraft required navigation performance (RNP) 10 approval was implemented in 1998 and the 50 NM longitudinal separation standard in 2005.  For aircraft/operators to be eligible for the application of 50 NM longitudinal distance based separation (D50), the following CNS requirements must be met:

        - Communications:  Direct controller-pilot communication (DCPC) or controller pilot data link communication (CPDLC);

        - Navigation:  Approval for RNP 10 or better (e.g., RNP 4);

        - Surveillance:  Procedural position reports;

        - Distance verification requirement at least every 24 minutes; and

        - Maximum automatic dependent surveillance – contract (ADS-C) periodic reporting interval: 27 minutes

  - On 20 June 2004, the ICAO President on behalf of the Council authorized the use of the 30NM lateral/30NM longitudinal separation standard (30/30).  The 30/30 separation standard enables suitably equipped and approved aircraft to operate in closer proximity to each other to effectively utilise the airspace in a more efficient manner.  Aircraft are thus able to obtain more optimal flight paths and cruising levels leading to operational, economic and environmental benefits.  For aircraft operators to be eligible for the application of 30/30 separation, the following CNS requirements must be met:

        - Communications:  DCPC or CPDLC;

        - Navigation:  Approval for RNP 4 or better; and

        - Surveillance:  Satellite ADS-C reporting at least every 14 minutes.

  - Aircraft equipped with enhanced CNS capabilities will generally meet RNP 4 or better.

- Recommendation

- The meeting is invited to:

  - review the information contained in this Working Paper;

b.   add the application of reduced longitudinal separation to the CPWG Work Program