Tenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/10)

(Paris, France 2-5 November 2010)

Agenda Item 4:  Provide Status on CPWG/9 Actions

- STATUS OF HF AIR-GROUND DATA LINK TRIAL

- (Action Item # CP03-29)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information about the current status of the High Frequency Data Link (HFDL) trial.

- Introduction

1.1	The High Frequency Data Link (HFDL) service was launched in 1998 with only a few stations, an adventurous customer, and a great deal of promise.  It has evolved into a robust service using 15 ground stations strategically placed across the globe covering the earth, especially the Polar Regions, with 31 independent, interlocking frequencies, actively managed such that coverage is redundant and in effect continuous anywhere on the planet. In the last twelve years, the service has enjoyed double-digit message growth year after year and boasts more than 72 customers, more than 1400 equipped aircraft and worldwide coverage. Primarily, the HFDL service provides Aircraft Communications Addressing and Reporting System (ACARS) Airline Operations Control (AOC) messaging at a highly reliable success rate. In the past few years, more and more Air Traffic Service Providers (ATSPs) have been receiving critical position reports via HFDL as a “back up media” mostly during anomalies with satellite communications and the increasing demands for real-time aircraft data. In most cases, the delivery times were well within the acceptable performance criteria. Commercial airlines and cargo carriers, equipped with Very High Frequency (VHF) and HFDL, have been interested in accessing the highly desirable FANS routes for speed, efficiency and fuel savings. Coupling the carrier’s needs with the ATSP desire to minimize voice and optimize airspace usage, a trial to test the operational performance of the HFDL system was needed.

1.2	The Trial was executed in different phases.

Phase 1 was an operational evaluation of the ARINC 635-4 software. This phase was strictly between ARINC and Hawaiian Airlines and included an assessment of performance using AOC messages and applications. The test compared the upgraded aircraft HFDL performance to existing ARINC 635-4 radios. The results posted a higher efficiency for uplink success rates. Transit delay performance was equal to the existing 635-4 platforms.

Phase 2 was an operational test with the FAA/ARINC HFDL network using controller pilot data link communications (CPDLC) only. Captains logged onto to the Anchorage Center (KZAK) system and after confirmation, select the automatic dependent surveillance (ADS) to “off”.

Phase 3 was a full FANS test to include both ADS-C and CPDLC.

Periodic data were reported by Oakland FAA for evaluation by the stakeholders. Results are available to stakeholders in the FANS over HFDL (Data) report.

- Discussion

2.1	 The following is from a draft document discussed October 13-14, 2010 at the Performance Based Operations Aviation Rulemaking Committee Communications Working Group (PARC CWG) meeting held in Toronto, Canada.  This report is intended to be forwarded to the Associate Administrator for Aviation Safety for consideration.

“The PARC would like to highlight the benefits of using HFDL in FANS 1/A operations. Some operators are equipped only with HFDL. Others are equipped with both SATCOM and HFDL. When an operator is equipped only with HFDL, leveraging this capability in FANS 1/A operations can reduce the effects of operational errors and pilot deviations when appropriate separation standards are applied, and reduce congestion on the voice channel providing improved access to operators that are not FANS 1/A equipped. Additionally, FOH provides ATC communication coverage in the Polar region, where coverage is not provided with commonly used communication services that use high-earth orbit geosynchronous satellites. FOH can also provide greater reliability of the total system and facilitate safer transitions to alternative separations when the SATCOM service does fail.

The PARC recognizes that while FOH offers significant benefits, it may not be suitable for some reduced separations, such as 30 NM lateral and 30 NM longitudinal separation. As a result, certain provisions may be necessary to recognize aircraft with lesser capability yet still leverage it to gain operational and safety benefits. The recommendation in the FOI Recommendations report to implement a performance-based framework includes the provisions to address near to mid term and long term initiatives supporting the next generation air transportation system (NextGen). The FOH Recommendations report substantiates the need to phase in the performance-based framework and expedite those portions needed to realize immediate gains in operational and safety benefits from FOH operations.”

2.2	The report concludes with:

“The trial has been successfully on-going since November of 2008.

Measurements of the CPDLC and ADS-C data show that the HFDL system is fully capable of meeting the requirements for RCP400 as set forth in GOLD and DO-306. This conclusion is also supported by data collected out side of the HAL trial by the New Zealand Oceanic FIR. Results are available to stakeholders in FANS over HF Data Link (Data) report.”

- Recommendation

- The Meeting is invited to note the information provided in this paper.