Eleventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/11)

(St. Petersburg, Russia, 7-10 June 2011)

Agenda Item 4:  Provide Status on CPWG/10 Actions

- MANAGING GATE RESERVATION LIST COMPLIANCE

- (Action Item #CP01-18)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information regarding track loading of fixes entering the Murmansk, Magadan, and Petropavlovsk-Kamchatsky Flight Information Regions (FIRs) from the Anchorage FIR. It also provides information on how traffic flows are managed to these fixes.

- Introduction

1.1	Westbound traffic from North America to Russia must transition between two very different environments.  Close cooperation between users and air navigation service providers (ANSPs) along with advances in technology have reduced limitations on aircraft transitioning to 169W.  After passing the fixes on the Russian Flight Information Region (FIR) boundaries, aircraft are limited to fixed routes and, until November 2011, will operate under conventional vertical separation minima using meters.  It has been an ongoing process to determine the best way to get aircraft from a relatively unrestricted environment to specific points in space with an expectation of a preferred altitude at those points.

1.2	A copy of the current (February 10, 2011) Track Advisory Users Guide for Dispatchers (TAUG), containing the Anchorage Air Route Traffic Control Center (ZAN) portion only, has been attached to this paper for easy reference and planning for airlines and service providers. (See Attachment A)

- Discussion

2.1	The northernmost polar fixes (DEVID, NALIM, RAMEL, NIKIN, and ORVIT) are track loaded and managed by the Federal Aviation Administration (FAA) Air Traffic Control System Command Center (ATCSCC).  The ATCSCC coordinates with Canadian traffic flow managers to create spacing to these fixes.  While the longitudinal separation standard is 10 minutes, the polar fixes are temporarily being track loaded at 20 minutes in trail at each altitude during the implementation of the Canadian Automated Air Traffic System (CAATS).  DEVID is loaded at flight level (FL) 310, FL350, and FL390; NALIM/ORVIT are loaded at FL320, FL340, and FL380; and RAMEL/NIKIN are loaded at FL320, FL340, and FL360 for transfer from ZAN to Murmansk Area Control Centre (ACC).  Magadan ACC receives aircraft planned for three metric altitudes (9600m, 10,600m, and 11,600m) at RAMEL, NIKIN, and ORVIT at 10 minute longitudinal separation intervals for each altitude.

2.2	ZAN Traffic Management Unit (TMU) track loads and manages the fixes into the Russian Trans East (RTE).  The two northernmost of these fixes are PILUN and LISKI.  At CPWG/9 Magadan ACC agreed to take and give transfers in feet at these fixes.  This has allowed for the reduction of NOTAM restrictions in this region and has made more altitudes available westbound.  These tracks are loaded at 10 minute intervals for each altitude available.

2.3	ZAN TMU also track loads and manages the remaining seven fixes (FRENK, KUTAL, VALDA, ERNIK, RUSOR, BESAT (track loaded via MARCC), and BAMOK).  Aircraft at these fixes are transferred at metric levels.  Aircraft are track loaded in 15 minute intervals for each altitude at VALDA and BESAT, and 10 minute intervals at each of the other fixes (See CPWG/10 WP/09 on reduced restrictions).  Radar and VHF coverage to the FIR boundary has provided greater flexibility in creating spacing and accommodating altitude requests at most of these fixes, with the exception of BAMOK and BESAT.

2.4	For flights transiting the Russian Trans East (RTE) and Cross Polar routes, dispatchers and flight crews are reminded that the Gateway Reservation List (GRL) is a tool used to assist dispatchers in their flight planning and to provide air traffic facilities early intent information. Tactical sequencing may be utilized to ensure required in-trail spacing for aircraft at the same altitude. This may include speed control and/or vectors to accomplish the required spacing.

2.5	The Track Advisory (TA) program software does not consider aircraft performance characteristics when assigning slots on the GRL. Therefore, even though a flight may arrive at the GRL fix “on-time”, the flight may not be assigned the GRL altitude.  Information on the track loading results for December 2010 is provided in Attachment B.

2.6	Dispatchers are reminded, until time of departure; they should update the information provided on the GRL for all planned flights by submitting a new TKF request as outlined in the TAUG. To provide for more dynamic system awareness, operators may revise their GRL information including any changes to the wheels-up time, requested altitude, or estimate at the gateway fix, at any time after departure. If further assistance is required, the customer may contact the ATCSCC or ZAN TMU as outlined in the TAUG.

- Recommendation

- The Meeting is invited to note the information presented in this paper; and

- use this information, and that included in the TAUG, to better plan and coordinate flights between ZAN and Russia.

Track Load Results (Examples from December 2010)

Day

% of Flights at Requested Alt

% of Flights Higher

% of Flights Lower

% Greater Than 10’ Off

1

88

10

2

40

2

89

9

3

50

3

55

31

14

54

4

75

13

12

46

5

80

16

4

35

6

77

20

3

40