Twelfth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/12)

(Beijing, China 6-9 December 2011)

Agenda Item 4: Provide Status on CPWG/11 Actions

- MANAGING GATEWAY RESERVATION LIST

- (Action Item #CP01-18)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information regarding the air traffic flow management procedures for managing the Gateway Reservation List (GRL) for flights track loaded over Cross Polar (CP) fixes ORVIT and north; and Russian Trans East (RTE) fixes PILUN and south.

- Introduction

1.1	Previously, westbound traffic from North America transitioning across the Russian flight information regions (FIRs) were required to change from conventional vertical separation minima to metric altitudes. This limited the available altitudes over the Russian entry fixes. Effective November 17, 2011, State ATM implemented Reduced Vertical Separation Minimum (RVSM). Due to this implementation, operators are now able to transition from North America into Russian airspace seamlessly with more available altitudes.

  - A copy of the current (November 17, 2011) Track Advisory Users Guide for Dispatchers

(TAUG), containing the Anchorage Air Route Traffic Control Center (ZAN) portion only, is attached to this paper for easy reference for planning by airlines and air navigation service providers. (See Attachment A)

# At CPWG/9, defined responsibilities on managing the Gateway Reservation List (GRL) were discussed. (See highlighted portions in Attachment B) To date, no Letter of Agreement or Memorandum of Understanding has been formalized regarding the management of the GRL.

- Discussion

2.1	The cross Polar (CP) fixes (DEVID, NALIM, RAMEL, NIKIN, and ORVIT) are managed by the Federal Aviation Administration (FAA) Air Traffic Control System Command Center (ATCSCC).  The ATCSCC coordinates with Canadian air traffic flow managers to create spacing to these fixes.

2.2	The Russian Trans East (RTE) fixes (PILUN, LISKI, FRENK, KUTAL, VALDA, ERNIK, RUSOR, BESAT (track loaded via MARCC), and BAMOK are managed by the ZAN Traffic Management Unit (TMU).  Currently, aircraft are track loaded in 15 minute intervals for each altitude at VALDA and BESAT, and 10 minute intervals at all other Russian entry fixes.  CPWG/10 WP/09 detailed reduced restrictions.  With the exception of BAMOK and BESAT, radar and VHF coverage to the RTE boundary fixes has provided greater flexibility in creating spacing and accommodating altitude requests.

2.4	For flights transiting RTE and CP routes, dispatchers and flight crews are reminded that the GRL is a tool used to assist dispatchers in their flight planning and to provide air traffic facilities early intent information. Tactical sequencing may be utilized to ensure required in-trail spacing for aircraft at the same altitude. This may include speed control and/or vectors to accomplish the required spacing.

3          Recommendation

3.1	The meeting is invited to:

  - review the information contained in this working paper; and

  - endorse the information provided to formalize an agreement on managing the GRL.

Ninth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/9)

(Montreal, Canada, 28-30 April 2010)

Agenda Item 4:   Provide Status on CPWG/8 Actions

MANAGING GATEWAY RESERVATION LIST COMPLIANCE

(Action Item #CP01-18)

(Presented by the Federal Aviation Administration)

SUMMARY This working paper presents information regarding the Air Traffic Flow Management procedures for managing Gateway Reservation List (GRL) compliance for flights track loaded over Cross Polar (CP) fixes ORVIT and north; and Russian Trans East (RTE) fixes PILUN and south.

1.	Introduction

# Operators utilizing the Cross Polar (CP) or Russian Trans-East (RTE) routes will comply with the requirements outlined in the “Track Advisory User’s Guide for Dispatchers” (TAUG) for use of the Dynamic Ocean Tracking System (DOTS) Plus Track Advisory (TA) program.  Flight dispatchers are required to update the information on the Gateway Reservation List (GRL) on all planned flights.  The Anchorage Air Route Traffic Control Center (ZAN) portion of the TAUG is at Appendix A.

# The Air Traffic Control System Command Center (ATCSCC) notifies NAV CANADA Traffic Management Unit (TMU) West located at Vancouver Area Control Centre (ACC) and appropriate airline dispatchers of any identified conflicts on the CP routes.

Note:  TMU West’s area of responsibility encompasses the Vancouver, Edmonton, and Winnipeg Flight Information Regions (FIRs).

# TMU West monitors identified flights filed on the CP/RTE routes and works with the ATCSCC, the affected Canadian ACC and appropriate airline dispatchers to achieve early conflict resolution within Canadian airspace.  Note: Edmonton ACC utilizes the GRL for “tie breaking” which means that flights that are on time will normally be given priority for their planned altitude.

# ZAN is responsible for conflict resolution on the RTE routes for boundary fixes PILUN and south.  ZAN works with the ATCSCC, TMU West, and appropriate airline dispatchers to achieve early conflict resolution within Canadian or United States airspace as required.

- Discussion

- Responsibilities are defined as follows:

  - Operators will:

    - Brief dispatchers/flight crews on the procedures, emphasizing that the GRL is a tool used to assist dispatchers in their flight planning and to provide air traffic facilities early intent information.

    - Brief dispatchers/flight crews that the GRL does not offer or establish priority for any flight operating on the CP routes or via RTE fixes PILUN and LISKI.

    - Brief dispatchers/flight crews that ZAN is currently conducting a 60 day trial to reduce the 20 minute track load requirement for RTE fixes FRENK, KUTAL, VALDA, and ERNIK to 10 minutes, which institutes the same procedures currently used on all CP fixes and RTE fixes PILUN and LISKI.

    - Brief dispatchers/flight crews that flights operating on the RTE routes via RUSOR and south, aircraft must arrive at the gateway fix within the scheduled window (GRL time, plus 10 minutes) to increase the probability of GRL altitude assignment.  It should be noted, however, that the TA program software does not consider aircraft performance characteristics when assigning slots on the GRL. Therefore, even though a flight may arrive at the GRL fix “on-time”, the flight may not be assigned the GRL altitude.

    - Until time of departure, update the information provided on the GRL on all planned flights by submitting a new TKF request as outlined in the TAUG.  To provide for more dynamic system awareness, operators may revise their GRL information including any changes to the wheels-up time, requested altitude, or estimate at the gateway fix, at any time after departure.  If further assistance is required, the customer may contact the ATCSCC or ZAN TMU as outlined below for assistance.

    - Call the ATCSCC International Operations Position at +1 540-359-3158 with any issues pertaining to the CP routes.

    - Call ZAN TMU at +1 907-269-1840 with any issues pertaining to the RTE routes PILUN and south.

  - ATCSCC will:

    - Create and monitor Flow Evaluation Areas (FEAs) for all CP fixes to identify potential conflicts.

    - Utilize the most current version of the GRL to monitor customer CP route requests.

    - Implement early conflict resolution on CP route traffic and coordinate all actions with TMU West as appropriate.

    - Assist TMU West on implementing any TMU West/Canadian ACC initiated conflict resolution plan.

    - Work with airline dispatchers when appropriate.

    - Assist ZAN TMU on implementing any ZAN TMU initiated conflict resolution plan, including the issuance of controlled departure times via the National Traffic Management Log (NTML).

  - TMU West will:

    - Coordinate with the ATCSCC and other ATC facilities to determine an appropriate conflict resolution decision(s).

    - Coordinate with airline dispatchers when appropriate.

  - ZAN TMU will:

    - Operate and maintain the DOTS Plus TA Program.

    - Actively monitor the GRL and RTE route traffic PILUN and south for potential conflicts.  Alert the ATCSCC and TMU West when requiring assistance with conflict resolution on the RTE routes.  When required, issue controlled departure times via NTML through the ATCSCC.

- Recommendation

3.1	The meeting is invited to:

  - note the information provided in this working paper; and

  - discuss and support implementation of procedures and processes needed to mitigate compacted demand over Cross Polar and Russian Trans East routes.