Thirteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/13)

(Reykjavik, Iceland - 19-22 June 2012)

Agenda Item 6: Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) Issues

- CPDLC Route Re-clearances

(Presented by Iceland)

SUMMARY This paper presents information on work being done in the NAT to establish appropriate procedures in the GOLD for controller initiated CPDLC route re-clearances.

- Introduction

  - In the NAT region currently only New York and Santa Maria are providing CPDLC route re-clearances to aircraft. Reykjavik has not initiated that service since there have been uncertainties about how the applicable CPDLC messages are processed in various aircraft types. This especially applies to the processing of CPDLC route clearance messages in the case of route discontinuities that occur when aircraft are re-routed without connecting the oceanic cleared route to the FPL route after the oceanic landfall.

  - Information has been received from Airbus and Boeing on how the CPDLC route clearance messages are processed. This enables the NAT to make informed decisions about the issue of CPDLC route re-clearances.

  - This now also provides the opportunity to formalize unified NAT rules concerning the handling of aircraft routes under various conditions (such as route discontinuities) in terms of:

- Handling of routes in co-ordination messages (defined in NAT Doc 002);

- Handling of routes in oceanic clearances, both voice and data link; and

- Handling of routes in route re-clearances, both voice and CPDLC.

  - This information paper addresses the work that has recently been done in the NAT on the issue of controller initiated CPDLC route re-clearances. The objective of the NAT work was to identify the functional common denominator between Airbus and Boeing with regard to the processing of uplink route re-clearances.

- Discussion

  - Airbus have provided the NAT service providers with the document: “FANSA/A+ Function Integration with FMS Technical Report”. This document describes in section 4.1.2 the functionality of CPDLC route loading on Airbus aircraft.

  - Boeing have provided the NAT service providers with the document: “Loading of ATC Clearances into the Flight Management System (FMS)”. This document describes in section 4.3 the functionality of CPDLC route loading on Boeing aircraft.

  - When it comes to issuing route re-clearances on CPDLC there are three available uplink message options in the FANS 1/A message set:

| UM# | Message element | Message Intent/Use (GOLD) |
| --- | --- | --- |
| UM79 | CLEARED TO [position] VIA [routeclearance] | Instruction to proceed to the specified position via the specified route. |
| UM80 | CLEARED [routeclearance] | Instruction to proceed via the specified route. |
| UM83 | AT [position] CLEARED [routeclearance] | Instruction to proceed from the specified position via the specified route. |

  - It should be noted that any waypoint that is sent in an uplink message and loaded as part of a new route in the FMS will not contain weather data. It does not make any difference whether the waypoint was previously in the route or not. As a consequence, the crew will lose from the FMS all weather data for waypoints that were previously in the route but are uploaded again as part of the new route. Some flight crews may be able to request the missing weather data from the operator.

  - The processing of UM79, UM80 and UM83 described in the Airbus and Boeing documents is summarized in the table below. The following convention should be noted:

- AAAA = Destination airport.

- PPOS = Present Position.

- The convention [A, B, C, D, E, F, …., AAAA] indicates whole route to destination.

- Option a) for UM79 and UM83 always just specifies the route portion while option b) always attempts to specify both the route portion and the airport as the clearance limit. The route in UM80 replaces the entire route in all cases in both Airbus and Boeing aircraft.

- The following color coding is used in the tables below:

Uplink message works.

Uplink message does not work because route details are lost from the FMS.

Does placing the new route at the end of the current route work from a pilot perspective?

| Case #1 |  |  |
| --- | --- | --- |
| First fix in the new route is on the currently cleared route. The new route merges back onto the currently cleared route. In this case the FDPS and FMS can carry an unbroken route to destination. |  |  |
| Current route: | [A, B, C, D, E, F, …., AAAA] |  |
| New route: | [A, B, C, X, Y, E, F, …., AAAA] |  |
|  | Airbus – new route | Boeing – new route |
| UM79 Option a) | CLEARED TO [E] VIA [A, B, C, X, Y] |  |
|  | [A, B, C, X, Y, E, F, …., AAAA] Weather data in the FMS is lost for waypoints A, B, C, D, waypoints X, Y do not contain any weather data, weather data for waypoints E, F …. remains intact. | [A, B, C, X, Y, E, F, …., AAAA] Weather data in the FMS is lost for waypoints A, B, C, D, waypoints X, Y do not contain any weather data, weather data for waypoints E, F … remains intact. |
| UM79 Option b) | CLEARED TO [AAAA] VIA [A, B, C, X, Y, E, F, …….] |  |
|  | [A, B, C, X, Y, E, F, …., AAAA] Using this message with the whole route to destination is considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. Putting an airport as the “to” point in um79 does NOT change the destination. To accomplish that AAAA must be included as the destination airport inside the [routeclearance]. | [A, B, C, X, Y, E, F, …., AAAA] Using this message with the whole route to destination is considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. Putting an airport as the “to” point in um79 does NOT change the destination. To accomplish that AAAA must be included as the destination airport inside the [routeclearance]. |
| UM 83 Option a) | AT [C] CLEARED [X, Y, E] |  |
|  | [A, B, C, X, Y, E, F, …., AAAA] Weather data in the FMS is lost for waypoints D, E, waypoints X, Y do not contain any weather data, weather data for waypoionts A, B, C, F …. remains intact. | [A, B, C, X, Y, E, AAAA] The rest of the route after E to destination is lost in the FMS. |
| UM 83 Option b) | AT [C] CLEARED [X, Y, E, F, ……] |  |
|  | [A, B, C, X, Y, E, F, …., AAAA] Weather data in the FMS is lost for all waypoints after C, weather data for waypoints A, B, C remains intact. | [A, B, C, X, Y, E, F, …., AAAA] Weather data in the FMS is lost for all waypoints after C, weather data for waypoints A, B, C remains intact. |
| UM80 | CLEARED [A, B, C, X, Y, E, F, ……] |  |
|  | [A, B, C, X, Y, E, F, …., AAAA] This is to be considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. | [A, B, C, X, Y, E, F, …., AAAA] This is to be considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. |
| Conclusion | If the ATSP wants to apply a uniform CPDLC re-route procedure, irrespective of aircraft type, then UM80 and UM83 can only be used if the route is always specified all the way to destination. Whether it is better to use UM79 or UM83 from the “loss of weather data” point of view depends on whether there are more waypoints before C or after E. |  |

| Case #2 |  |  |
| --- | --- | --- |
| First fix in the new route is on the currently cleared route. The new route does not merge back onto the currently cleared route. In this case a route discontinuity is created. This happens for example in the cases where the aircraft is rerouted and the cleared route is not connected to the FPL route after the landfall point. Unless appropriate procedures are established, the crew will not know how to cope with the route discontinuity after Z. They will wonder whether they are cleared direct from Z to D or not. Prior to reaching Z the aircraft must receive further route clearance. |  |  |
| Current route: | [A, B, C, D, E, F, …., AAAA] |  |
| New route: | [A, B, C, X, Y, Z, …., AAAA] Route discontinuity. |  |
|  | Airbus – new route | Boeing – new route |
| UM79 Option a) | CLEARED TO [Z] VIA [A, B, C, X, Y] |  |
|  | [A, B, C, X, Y, Z, discontinuity D, E, F, …., AAAA] Weather data in the FMS is lost for waypoints A, B, C, waypoints X, Y, Z do not contain any weather data, weather data for waypoints D, E, F …. remains intact. | [A, B, C, X, Y, Z, discontinuity A, B, C, D, E, F, …., AAAA] Weather data in the FMS is lost for waypoints A, B, C, waypoints X, Y, Z do not contain any weather data, weather data for waypoints D, E, F …. remains intact. |
| UM79 Option b) | CLEARED TO [AAAA] VIA [A, B, C, X, Y, Z] |  |
|  | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. |
| UM 83 Option a) | AT [C] CLEARED [X, Y, Z] |  |
|  | [A, B, C, X, Y, Z, discontinuity D, E, F, …., AAAA] No weather data is lost and waypoints X, Y, Z do not contain any weather data. | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost. |
| UM 83 Option b) | AT [C] CLEARED [X, Y, Z, AAAA] |  |
|  | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. |
| UM 80 | CLEARED [A, B, C, X, Y, Z, AAAA] |  |
|  | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. | [A, B, C, X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. |
| Conclusion | If the ATSP wants to apply a uniform CPDLC re-route procedure, irrespective of aircraft type, then UM79 is the only option in this scenario. |  |

| Case #3 |  |  |
| --- | --- | --- |
| First fix in the new route is not on the currently cleared route. The new route merges back onto the currently cleared route. In this case the FDPS and FMS can carry an unbroken route to destination. |  |  |
| Current route: | [A, B, C, D, E, F, …., AAAA] |  |
| New route: | [X, Y, E, F, …., AAAA] |  |
|  | Airbus – new route | Boeing – new route |
| UM79 Option a) | CLEARED TO [E] VIA [ X, Y] |  |
|  | [PPOS, discontinuity, X, Y, E, F, …., AAAA] Weather data for waypoints A, B, C, D is lost, waypoints X, Y do not contain any weather data, weather data for waypoints E, F, … remains intact. | [PPOS, X, Y, E, F, …., AAAA] Weather data for waypoints A, B, C, D is lost, waypoints X, Y do not contain any weather data, weather data for waypoints E, F, … remains intact. |
| UM79 Option b) | CLEARED TO [AAAA] VIA [X, Y, E, F, …….] |  |
|  | [PPOS, discontinuity ,X, Y, E, F, …., AAAA] Using this message with the whole route to destination is considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance will be activated. | [PPOS, X, Y, E, F, …., AAAA] Using this message with the whole route to destination is considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. |
| UM83 Option a) | AT [X] CLEARED [Y, E] |  |
|  | [PPOS, discontinuity, X, Y, E, F, …., AAAA] Weather data for waypoints A, B, C, D is lost, waypoints X, Y do not contain any weather data, weather data for waypoints E, F, … remains intact. Airbus comment: From a pilot perspective, this clearance is not intuitive as X does not belong to the original F-PLN. | [A, B, C, D, E, F, …., AAAA discontinuity, X, Y, E] The new route X, Y, E is entered after the current route to destination and would have to be re-entered. |
| UM83 Option b) | AT [X] CLEARED [Y, E, F, ……] |  |
|  | [PPOS, discontinuity, X, Y, E, F, …., AAAA] All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. Airbus comment: From a pilot perspective, this clearance is not intuitive as X does not belong to the original F-PLN. | [A, B, C, D, E, F, …., AAAA discontinuity, X, Y, E, F, …., AAAA] The new route is entered at the end of the route after a discontinuity. |
| UM80 | CLEARED [X, Y, E, F, ……] |  |
|  | [PPOS, discontinuity, X, Y, E, F, …., AAAA] This is to be considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. | [PPOS, X, Y, E, F, …., AAAA] This is to be considered as a F-PLN initialization. All weather data already inserted in the current active F-PLN will be lost once this clearance is activated. |
| Conclusion | Placing the new route at the very end, after the current route in the FMS seems to make UM83 operationally unacceptable for the Boeing pilot in this case. UM79 and UM80 seem to work both on Airbus and Boeing aircraft for this case however using UM80 causes the crew to lose more weather data then using UM79. |  |

| Case #4 |  |  |
| --- | --- | --- |
| First fix in new route is not on the currently cleared route. The new route does not merge back onto the currently cleared route. In this case a route discontinuity is created. This happens for example in the cases where the aircraft is rerouted and the cleared route is not connected to the FPL route after the landfall point. Unless appropriate procedures are established, the crew will not know how to cope with the route discontinuity after Z. They will wonder whether they are cleared direct from Z to A or not. Prior to reaching Z the aircraft must receive further route clearance. |  |  |
| Current route: | [A, B, C, D, E, F, …., AAAA] |  |
| New route: | [X, Y, Z, …., AAAA] Route discontinuity. |  |
|  | Airbus – new route | Boeing – new route |
| UM79 Option a) | CLEARED TO [Z] VIA [X, Y] |  |
|  | [PPOS, discontinuity, X, Y, Z, discontinuity A, B, C, D, E, F, …., AAAA] No weather data is lost and waypoints X, Y, Z do not contain any weather data. | [PPOS, X, Y, Z, discontinuity A, B, C, D, E, F, …., AAAA] No weather data is lost and waypoints X, Y, Z do not contain any weather data. |
| UM79 Option b) | CLEARED TO [AAAA] VIA [X, Y, Z] |  |
|  | [X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. | [X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. |
| UM83 Option a) | AT [X] CLEARED [Y, Z] |  |
|  | [PPOS, discontinuity, X, Y, Z, discontinuity A, B, C, D, E, F, …., AAAA] No weather data is lost and waypoints X, Y, Z do not contain any weather data. Airbus comment: From a pilot perspective, this clearance is not intuitive as X does not belong to the original F-PLN. | [A, B, C, D, E, F, …., AAAA discontinuity, X, Y, Z] The new route X, Y, Z is entered after the current route to destination and would have to be re-entered. |
| UM83 Option b) | AT [X] CLEARED [Y, Z, AAAA] |  |
|  | [X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. | [A, B, C, D, E, F, …., AAAA discontinuity, X, Y, , AAAA] The new route X, Y, AAAA is entered after the current route to destination and would have to be re-entered. Boeing comment: After Y there’s nothing except AAAA. |
| UM80 | CLEARED [X, Y, Z, AAAA] |  |
|  | [X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. | [X, Y, Z, AAAA] The rest of the route after Z to destination is lost in the FMS. |
| Conclusion | If the ATSP wants to apply a uniform CPDLC re-route procedure, irrespective of aircraft type, then UM79 is the only option in this scenario. |  |

2.6	The following conclusions are drawn from this analyses:

      - ATC should be aware that any waypoint that is sent in an uplink message and loaded as part of a new route in the FMS will not contain weather data. It does not make any difference whether the waypoint was previously in the route or not. As a consequence, the crew will lose from the FMS all weather data for waypoints that were previously in the route but are uploaded again as part of the new route. ATC should therefore, as far as possible, restrict the uplinked waypoints to that part of the route that is being amended. Some flight crews may be able to request the missing weather data from the operator.

      - If the reroute clearance changes the NEXT or NEXT+1 waypoint, then an ADS-C report will be received based on zero wind (or the wind at the aircraft present position) at the next waypoint which may result in an inaccurate estimate for that waypoint.

      - If the ATSP wants to use UM83 to issue CPDLC re-route clearances, irrespective of aircraft type, then the following conditions must be satisfied:

        - The route must be specified all the way to destination.

        - The [position] in UM83 must be on the currently cleared route.

All weather data after [position] is lost from the FMS when the new route is activated.

      - If the ATSP wants to use UM80 to issue CPDLC re-route clearances, the route must be specified all the way to destination. All weather data is lost from the FMS when the new route is activated.

      - If the ATSP wants to apply a uniform CPDLC re-route procedure, irrespective of aircraft type and irrespective of the nature of the reroute, then UM79 is the only option. All weather data for the uplinked waypoints is lost from the FMS when the new route is activated.

      - When using UM79 the [position] field should not be populated with the destination airport unless the route is specified all the way to destination.

      - The [position] in UM79 should not be confused with the ATC concept of “clearance limit”. It must be made clear in relevant documentation that UM79: CLEARED TO [position] VIA [routeclearance] does NOT change the clearance limit for the flight. Due to the way that UM79 has been implemented by aircraft manufacturers it seems that the current FANS 1/A message set does not support the concept of specifying a new “clearance limit” in a re-route clearance. (It has been confirmed that FMSs do not cater for the concept of “clearance limit” at all which is something that may warrant further studies).

      - The procedures for using CPDLC for issuing re-route clearances to aircraft that are approaching the waypoint where the route change takes place need to be clarified. The proposed text for GOLD specifies that “ATC should uplink the reroute as soon as practicable to allow processing time by the flight crew prior to the divergence waypoint”. Nevertheless it is a fact that situations can arise where the aircraft must be re-routed on short notice when the aircraft is approaching the diversion point and it is also known that uplink messages can be delayed in the system. Consider the following example:

The aircraft is approaching waypoint C and ATSU1 wants to reroute the aircraft via C, X, Y, Z. ATSU1 coordinates the re-route with ATSU2 and by the time the aircraft receives the uplink clearance UM79: CLEARED TO [Z] VIA [C, X, Y] the aircraft has already passed waypoint C. In this case the ATMG agreed that the crew should select UNABLE and continue on the currently cleared route.

2.7	The subject of CPDLC re-route clearances was discussed at the NAT ATMG/39 meeting in March 2012. The following is from the summary of discussions from that meeting:

5.10 The NAT ATMG reviewed material proposed to incorporate procedures into the GOLD to support ATC-initiated route clearances delivered via CPDLC. Draft material, originally developed at NAT ATMG had been updated to account for information provided by Boeing and Airbus concerning system behaviour related to various possible uses of CPDLC UM79, UM80 and UM83 to provide a re-route clearance. The material also took account of the fact that, based on which waypoints would be uplinked, the associated wind and temperature data held in the FMS for those waypoints would be lost if the route provided in the CPDLC clearance were “auto loaded”. The results of the review by the NAT ATMG are provided in Appendix I and show the differences between what was agreed at NAT ATMG/39 and previously developed material. Iceland agreed to use the material as a basis for a working paper to be presented to NAT CNSG/6, seeking that group’s support for a proposal to amend the GOLD.

  - The subject of CPDLC re-route clearances was discussed at the NAT CNSG/6 meeting in March 2012. The following is from the summary of discussions from that meeting:

5.37 The draft proposal for amendment was reviewed and agreed to by the NAT ATMG. The Group provided some additional amendments that are reflected in Appendix F. It was noted that any waypoint that is sent in an uplink message and loaded as part of a new route in the FMS will not contain forecast weather data.

5.38 The Group agreed that this proposal for amendment could be submitted to the GOLD ad-hoc group for global coordination before eventual inclusion in the second edition and endorsement by the NAT SPG. The Group also agreed to recommend that the NAT service providers could start implementing route re-clearances based on the principles agreed in Appendix F.

  - The subject of CPDLC re-route clearances was discussed at the NAT IMG/40 meeting in May 2012. The following is from the draft summary of discussions from that meeting:

4.66 The NAT IMG agreed that this proposal for amendment (Appendix A_wp17 refers) could be submitted to the GOLD ad-hoc group for global coordination before eventual inclusion in Edition 2.0 of the GOLD. The ICAO Secretariat agreed to facilitate in preliminary coordination to enable the NAT service providers to start implementing the route re-clearances as soon as possible.

4.67 Therefore, the NAT IMG agreed to the following:

NAT IMG Decision 40/WP17-01 – Use of CPDLC for route re-clearances

That the ICAO Regional Director, Europe and North Atlantic;

a)	take appropriate measures to submit the proposal for amendment on the use of CPDLC for route re-clearances (Appendix A_wp17 refers) to the ICAO GOLD ad-hoc group for global coordination;

b)	once the preliminary coordination was completed, invite the NAT service providers to implement the use of CPDLC for route re-clearances using the principles provided in Appendix A_wp17.

  - The proposed GOLD amendment is provided in Attachment A to this information paper.

- Recommendation

- The Meeting is invited to note the information provided in this paper.

Proposal for the GOLD amendment on the CPDLC use for route re-clearances

6.1.3	Reroute procedures – ATC initiated

6.1.3.1	The purpose of the ATC initiated reroute procedure is to allow an ATSU to initiate the process to issue an amended route clearance to an airborne aircraft.

6.1.3.2	ATC should be aware that any waypoint that is sent in an uplink message and loaded as part of a new route in the FMS will not contain forecast weather data. It does not make any difference whether the waypoint was previously in the route or not. As a consequence, the crew will lose from the FMS all forecast weather data for waypoints that were previously in the route but are uploaded again as part of the new route. ATC should therefore, as far as possible, restrict the uplinked waypoints to that part of the route that is being amended. Some flight crews may be able to request the missing forecast weather data from the operator.

6.1.3.3 If the reroute clearance changes the NEXT or NEXT+1 waypoint, then an ADS-C report will be received based on zero wind at the next waypoint which may result in an inaccurate estimate for that waypoint.

6.1.3.4	ATC should uplink the reroute as soon as practicable to allow processing time by the flight crew prior to the divergence waypoint. For those cases where the aircraft is getting close to the divergence waypoint when the clearance is issued the controller should consider the option of clearing the aircraft direct to the next waypoint.

6.1.3.5	If the aircraft has passed the divergence waypoint when the CPDLC reroute message is received the crew should select UNABLE and continue on the currently cleared route.

6.1.3.6	Aircraft operators should establish procedures for crews to deal with clearances that create route discontinuities. Such procedures should include the crew taking an initiative to obtain further route clearance before reaching the waypoint where the route discontinuity occurs if such route clearance has not been received from ATC a reasonable time before reaching the discontinuity waypoint.

6.1.3.7	UM83 should only be used to issue CPDLC re-route clearances if the following conditions are satisfied:

      - The route is specified to destination; and

      - The [position] in UM83 is on the currently cleared route.

All forecast weather data after [position] is lost from the FMS when the new route is activated.

6.1.3.8	UM80 should only be used to issue CPDLC re-route clearances if the route is specified from the aircraft present position to destination. All forecast weather data is lost from the FMS when the new route is activated.

6.1.3.9	When using UM79 the [position] field should not be populated with the destination airport unless the route is specified to destination. All forecast weather data for the uplinked waypoints is lost from the FMS when the new route is activated.

6.1.3.10	The [position] in UM79: CLEARED TO [position] VIA [routeclearance] does NOT change the clearance limit for the flight. The clearance limit remains unchanged unless explicitly changed by ATC. Although the structure of UM79 contains an item which semantically resembles a clearance limit (“CLEARED TO [position] VIA [route]”) it is important to note that the FMS has no concept of a clearance limit, the word “TO” in UM79 merely signifies the far end of the route segment that is being changed. It may coincidentally be identical to the clearance limit previously specified by ATC but usually will not be.

6.1.3.11	Table 6-2 provides the procedures for an ATC initiated reroute, and Figures 6-2-(a-e) provide an overview of the process.

Table 6-2. ATC initiated reroute procedures

| Who | Procedures |
| --- | --- |
| ATSU (Step 1) | Uplink an amended route clearance to the aircraft and append the [reason] if possible. Example: UM 83 AT [position] CLEARED [route clearance] UM 166 DUE TO TRAFFIC |
| Flight crew (Step 2) | - On receipt of a CPDLC route clearance initiated by an ATSU, the flight crew should: - Load the uplink into the FMS and review the clearance. If the clearance is acceptable, respond with a DM 0 WILCO to confirm that the flight crew will comply with the clearance; or - Otherwise: - Respond with DM 1 UNABLE; and - Continue in accordance with the current ATC clearance. |
|  | - Where an uplinked clearance is acceptable to the flight crew but creates a route discontinuity, the flight crew should proceed to overcome the potential discontinuity by applying their existing company practices. |

| 1 | - ATC uplinks the clearance CLEARED TO [E] VIA [A B C X Y] to the aircraft using UM79. There is no discontinuity because the uplink fix (E) is in the existing cleared flight plan; or (Forecast weather data in the FMS is lost for waypoints A, B, C, D, waypoints X, Y do not contain any forecast weather data, forecast weather data for waypoints E, F, …. remains intact). - ATC uplinks the clearance CLEARED [A B C X Y E F …..] to the aircraft using UM80. There is no discontinuity because the entire route has been replaced. The route must be specified to destination; or (Forecast weather data in the FMS is lost for the whole route). - ATC uplinks the clearance AT [C] CLEARED [X Y E F …..] to the aircraft using UM83. There is no discontinuity because the entire route after C was specified. The route must be specified to destination. (Forecast weather data in the FMS is lost for all waypoints after C). |
| --- | --- |
| 2 | The flight crew responds to the clearance with a WILCO or UNABLE as appropriate. |

Figure 6-2-a.  The ATC initiated reroute process for the case where the first waypoint in the new route is on the current route and there is no route discontinuity

| 1 | ATC uplinks the clearance CLEARED TO [Z] VIA [A B C X Y] to the aircraft using UM79. (Forecast weather data in the FMS is lost for waypoints A, B, C waypoints, X, Y, Z do not contain any forecast weather data, forecast weather data for waypoints D, E F, G, H etc remains intact). Note . — CPDLC message elements UM80 and UM83 should not be used in this case. |
| --- | --- |
| 2 | The flight crew responds to the clearance with a WILCO or UNABLE as appropriate. This clearance creates a route discontinuity at Z. Further route clearance should be obtained from ATC before the aircraft reaches Z (that clearance could for example be from Z direct to G). In the meantime, the flight crew should overcome the discontinuity at Z by applying their existing company practices under the assumption that further route clearance will be received before reaching Z. |

Figure 6-2-b.  The ATC initiated reroute process for the case where the first waypoint in the new route is on the current route and there is route discontinuity.

| 1 | - ATC uplinks the clearance CLEARED TO [E] VIA [X Y] to the aircraft using UM79. There is no discontinuity at E because the uplink fix (E) is in the existing cleared flight plan; or (Forecast weather data in the FMS is lost for waypoints A, B, C, D, waypoints X, Y do not contain any forecast weather data, forecast weather data for waypoints E, F, etc, remains intact). - ATC uplinks the clearance: CLEARED [X, Y, E, F …..] to the aircraft using UM80. The entire route is replaced and the route must be specified to destination. (Forecast weather data in the FMS is lost for the whole route). The clearance in a) and b) above takes immediate effect and since the first fix specified (X) is not on the existing route of flight the new route effectively starts with "present position direct X” although this is not explicitly stated in the uplink message. Note . — CPDLC message element UM83 should not be used in this case. |
| --- | --- |
| 2 | The flight crew responds to the clearance with a WILCO or UNABLE as appropriate. |

- Figure 6-2-c.  The ATC initiated reroute process for the case where the first waypoint in the new route is not on the current route and there is no route discontinuity

| 1 | ATC uplinks the clearance CLEARED TO [Z] VIA [X Y] to the aircraft using UM79. (Waypoints X, Y, Z do not contain any forecast weather data, forecast weather data for waypoints A, B, C, D, E, F,G, H, I etc remains intact). The clearance above takes immediate effect and since the first fix specified (X) is not on the existing route of flight the new route effectively starts with "present position direct X” although this is not explicitly stated in the uplink message. Note . — CPDLC message elements UM80 and UM83 should not be used in this case. |
| --- | --- |
| 2 | The flight crew responds to the clearance with a WILCO or UNABLE as appropriate. This clearance creates a route discontinuity at Z. Further route clearance should be obtained from ATC before the aircraft reaches Z (that clearance could for example be from Z direct to I). In the meantime, the flight crew should overcome the discontinuity at Z by applying their existing company practices under the assumption that further route clearance will be received before reaching Z. |

Figure 6-2-d.  The ATC initiated reroute process for the case where the first waypoint in the new route is not on the current route and there is route discontinuity.

XXXX

I

H

G

F

E

D

C

B

A

1

| 1 | ATC uplinks the clearance UM 74 - PROCEED DIRECT TO [fix I] to the aircraft |
| --- | --- |
| 2 | The flight crew responds to the clearance with a WILCO, immediately loading the clearance into the FMC and proceeding direct to the cleared direct-to fix I. |

Figure 6 2 e. The ATC initiated reroute process for the case where the aircraft is cleared direct to a fix that is located downstream in the current route.

END