Thirteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/13)

(Reykjavik, Iceland, 19-22, 2012)

Agenda Item 4: Provide Status on CPWG/12 Actions

STATUS OF ADDITIONAL BOUNDARY FIXES IN THE NORTH PACIFIC (NOPAC)

(Action Item # CP12-13)

(Presented by the Federal Aviation Administration)

SUMMARY This Information Paper presents provides an update on additional boundary fixes in the NOPAC affecting Anchorage Air Route Traffic Control Center (ZAN), Petropavlovsk-Kamchatsky Area Control Center (ACC), and Fukuoka (Japan) Air Traffic Management Center (ATMC).

1.	Introduction

# CPWG/9 addressed the need for new fixes and routes in the North Pacific (NOPAC) airspace adjacent to the Anchorage (ZAN) Flight Information Region (FIR), Petropavlovsk-Kamchatsky (PK) FIR, Yuzhno-Sakhalinsk FIR, and Fukuoka FIR.

# Prior to CPWG/10, State ATM offered some possible fixes along the Anchorage FIR and Fukuoka FIR boundaries that could accommodate emergency reroutes and possibly offer alternative routings for efficiency.

# Several factors required consideration when opening new traffic patterns in this area.  These factors included automation, coordination capability, communication, and procedures.

# In 2011, State ATM established fixes LUMES (G73) and KUNAD (G801) on the common ZAN and PK FIR boundary.  State ATM also created BAGLI (G804) on the common FIR boundaries with ZAN, PK, and Fukuoka (Japan)  All three fixes were established with new westbound only airways that allowed aircraft transiting the NOPAC to join existing route structures into Russia.

- Discussion

2.1	On April 5, 2012 the FAA (See Attachment A):

- Established one new fix, NRKEY, on NOPAC Route R220

- Established a charted route segment from NRKEY on NOPAC R220, west to BAGLI extending airway G804

- Established a charted route segment from NUZAN on NOPAC R220, west to KUNAD extending airway G801

- Established a charted route segment from NYMPH on NOPAC R220, west to LUMES extending airway G73

2.2	ZAN formed a Safety Risk Management Panel (SRMP) on April 11, 2012 to consider potential hazards applicable to the proposed changes.  The SRMP identified two hazards associated with the establishment of the fix NRKEY, the extension of G804 between BAGLI and NRKEY, and the use of this airway and the fix BAGLI as a viable transition route into Russia.

2.2.1	The identified hazards were:

- Flight crew confusion over BAGLI due to inappropriate Controller Pilot Datalink Communications (CPDLC) next data authority message

  - CPDLC is one of the systems used for communications with aircraft operating in the oceanic areas.  As an aircraft leaves one facility's airspace, the facility sends an automated message known as a "next data authority" message to the aircraft identifying the next facility in which the aircraft will establish a CPDLC connection.  BAGLI is on the boundary of three FIRs: ZAN, PK and Fukuoka.  PK ACC does not currently use CPDLC, so the Ocean21 system sends the "next data authority" message to Japan Civil Aviation Bureau (JCAB) Air Traffic Management Center (ATMC).  This information could cause confusion with flight crews, as their communications transfer information received from ZAN controllers will be to contact PK ACC on VHF.

- Increased complexity for controllers during inter-facility coordination for aircraft over BAGLI

  - Due to the location of BAGLI on the FIR boundaries of ZAN, PK and Fukuoka, ZAN controllers will need to ensure that JCAB ATMC receives the appropriate coordination message pointing out that the protected airspace of the aircraft will penetrate the Fukuoka FIR as it transitions into PK FIR.

2.3 	Treatment or Risk/Mitigation of Hazards:

- Mitigations to Treat Hazards – The Initial Risk Assessment for all hazards was “Low”.  The SRMP felt the “Severity/Rationale” of the hazards would be reduced by simply moving the fix 51NM east along the ZAN/PK common FIR boundary.  This would eliminate flight crew confusion and the additional controller coordination during transfer.

- Hazard Analysis – As the proposed changes are currently authorized and applied within the United States (US) National Airspace System (NAS), the SRMP evaluated changes that could be made to reduce the potential or frequency of occurrence of the identified hazards.

- Risk Determination – Risk is the composite of predicted severity and likelihood of the potential effect of a hazard in the worst credible system state.  Risk is determined by two factors: severity of consequence and likelihood of occurrence.  Current system state controls and requirements effectively address risk mitigation controls.

- Rationale – The Ocean21 automation platform provides a level of safety by sending Air Traffic Services Interfacility Data Communications (AIDC) transfer messages to JCAB ATMC 45 flying minutes in advance of the aircraft penetrating their FIR.  Additionally, Ocean21 utilizes conflict probe 90 flying minutes into PK airspace to assist in detecting any potential traffic conflicts prior to transfer.

2.4	Until the SRMP risks are mitigated, ZAN will not use the route via NRKEY G804 BAGLI to transition into the PK FIR.  In addition to the option of moving BAGLI 51NM east of its current location, two other options should be discussed.  One solution is to remove both fixes NRKEY and BAGLI, and the associated airway G804.  Another solution would be to move the boundary fix BAGLI, 30 minutes flying time into JCAB airspace to allow proper CPDLC logon/logoff capabilities.

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this Information Paper, and

  - provide feedback on the information provided.