Third Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group

Pacific Project Team Meeting (CPWG/PPT/3)

(Reykjavik, Iceland, 19 June 2012)

Agenda Item 3:  Review progress on CPWG/PPT/3 Action Items

Flexible Tracks within the Russian Trans East Region

(Action Item #PP02-04)

(Presented by IATA (United Airlines))

SUMMARY Pacific Project Action Item 02-04 was established to consider implementation of flexible tracks between approved entry and exit points within the RTE region on a daily basis. This paper provides a report of two separate flex track scenarios by United Airlines in March and May 2012.

1.	Introduction

# The Second Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group Pacific Project Team (CPWG/PPT/2) identified new action item PP02-04 for the consideration of flexible tracks between approved entry and exit points within the RTE region on a daily basis.

# United Airlines produced three “paper” scenarios for consideration. A March 30, 2012 paper trial using Chicago to Shanghai was produced, and a second date of May 6, 2012 produced two  separate scenarios for a Chicago  to Shanghai operation.

# The results of these flexible routing paper trials are outlined below.

- Discussion

- Flexible routing within Russian Airspace was discussed at the Pacific Project Meeting in Beijing in December 2011 as a potential enhancement to the fixed route system. Currently flexible routing is not permitted within Russia.

- The candidate flight by United Airlines was UAL835 Chicago to Shanghai which routinely operates via the Russian Trans East region and also via Polar route from ORVIT.

- The dates selected for the paper trial were March 30 and May 6, 2012.

2.4	The March 30 flexible routing used PILUN (72 00.0N 168 58.4) as the entry, and MAGIT (47 41.5N  131 09.0E) as the exit of Russian airspace. The following routings were computed by the United Airlines Sabre flight planning system:

- Standard Routing: PILUN B970 OKEPA A803 ASKIB B722 GIRUD R213 MAGIT

Flight time for Chicago to Shanghai 14:43	Fuel Burn 237398 pounds

- Flexible Routing: PILUN DCT 72N170W DCT 72N175W DCT 72M180E DCT 72N175E DCT 71N170E DCT 70N165E DCT 69N160E DCT 68N155E DCT 66N150E DCT 65N147E DCT 64N145E DCT 60N140E DCT 55N135E DCT 50N132E DCT MAGIT

Flight time for Chicago to Shanghai 14:41	Fuel Burn 237231 pounds

Time saved:	2 minutes			Fueled saved:	 167 pounds

2.5	The May 6, 2012 paper trial included a similar scenario with UAL835 from Chicago to Shanghai, using the PILUN entry and MAGIT exit.

- Standard Routing: PILUN B970 OKEPA A803 OSKON A218 BANOT R213 MAGIT

Flight time for Chicago to Shanghai 13:51	Fuel Burn 207764 pounds

- Flexible routing:  PILUN DCT 72N170W DCT 72N175W DCT 71N180E DCT 70N175 DCT 69N170E DCT 67N165E DCT 65N160E DCT 63N155E DCT 60N150E DCT 58N145E DCT 55N140E DCT 52N135E DCT 50N133E DCT MAGIT

Flight time for Chicago to Shanghai 13:52	Fuel Burn 208022 pounds

Time saved:	-1 minute			Fuel saved:	 -258 pounds

2.6	A second scenario was conducted on May 6 using ORVIT as the entry and SIMLI as the exit fixes.

- Standard routing: ORVIT G493 SIMLI

Flight time for Chicago to Shanghai 13:57	Fuel Burn: 210694 pounds

- Flexible routing: ORVIT DCT 79N170W DCT 78N175W DCT 77N180E DCT 76N175E DCT 75N170E DCT 74N160E DCT 70N155E DCT 68N150E DCT 65N145E DCT 62N150E DCT 60N137E DCT 59N135E DCT 55N131E DCT 54N130E DECT SIMLI

Flight time for Chicago to Shanghai 13:55	Fuel Burn 209794 pounds

2.7	The paper trial results indicate the potential for some time and fuel savings, however they are inconclusive based are a variety of reasons. The ATS route network within the Russian Trans East region has been significantly expanded for international operations over the past several years. This has greatly improved the flexibility and efficiency of most operations.

2.8	It is likely that further paper trials run on a more frequent basis may indicate greater savings, however the current prohibition of actually flying flexible routes within Russia make it difficult to fully realize those savings.

2.9	It should also be noted that the results of these paper trials did not take into consideration any specific airspace constraints that may have an adverse effect on the results.

- Recommendation

3.1	The meeting is invited to:

  - review the information contained in this Working Paper; and

  - discuss whether further trials are desired, and

  - to update the action item list.