Thirteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/13)

(Reykjavik, Iceland - 19-22 June 2012)

Agenda Item 6:  Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) Issues

- STANDBY MESSAGES TO AIRBUS AIRCRAFT

(Presented by Isavia)

SUMMARY This paper advises about an issue that has recently been discovered concerning the processing of CPDLC messages in Airbus aircraft.

- Introduction

  - It has recently come to the attention of Isavia that Airbus has implemented a 5 minute timer that is set when the aircraft transmits a downlink CPDLC message. The processing by the aircraft of uplink messages that are received after the 5 minute timer has expired has implications on whether it is advisable to uplink STANDBY messages to aircraft that have such functionality.

- Discussion

2.1	It has recently come to the attention of Isavia that Airbus have implemented a 5 minute timer that is set when the aircraft transmits a downlink message. After the 5 minute timer has expired, the aircraft will only accept one message in the same dialogue. This functionality has the following effects:

      - If an uplink STANDBY message is received by the aircraft before the 5 minute timer expired then the timer is re-initiated for another 5 minutes, enabling the flight crew to receive the actual uplink response message.

      - If an uplink STANDBY message is received by the aircraft after the 5 minute timer has expired then the actual uplink response message (which could be a clearance) will be rejected by the aircraft and not displayed to the flight crew.

2.2	The following information was received by Isavia from Airbus concerning this functionality:

(Note:	The NAT is only concerned with the functionality of FANS A and FANS A+ Airbus aircraft. Information concerning FANS B+ is therefore indicated with light grey text below since the NAT is not concerned with that system.)

When a request is sent by the flight crew to the ATC center, a response is expected. As soon as the request is sent a timer is launched waiting down for an answer. If no message is received from the ground after a given time (see table below), the crew is reminded of this through an amber “NO ATC REPLY” message displayed on the DCDU (A330/340 see figure to the right) (or dedicated mailbox (A380).

The following table gives the differences between the Airbus a/c:

| Aircraft | Timer value | NO ATC REPLY msg displayed on DCDU/Mailbox | Uplinks received after timer expiration |
| --- | --- | --- | --- |
| FANS A and A+ (A320/330/340) | 5 min (systems certified before 2011) 7 min 30 (from 2011) | Yes | The first uplink, whatever it is (STBY, DEFERRED or answer), is accepted (1) then any subsequent message will be rejected and an error message “Unrecognized MRN” will be down-linked. |
| FANS B+ (A320) | 270 seconds(2) | Yes | Rejected (and an error message is automatically sent to the ground) (2) |
| FANS A+ A380 | 5 minutes | Yes | Accepted (1) |

The European standards ED-100A and ED-110B are respectively applicable to FANS A+ and FANS B+ (ATN continental environment).

(1): According to ED-100A, in FANS A+, no timer is mentioned, which means that the on-board system has to accept all messages sent by the ATC center.

(2): According to ED-110B, in FANS B+, 250 seconds after the ATC center has received a request, ground and on-board systems close the dialogue phase, which means that both the controller cannot reply anymore to the request, and the on-board system don’t expect anymore a response. Then, in normal situation, the NO ATC REPLY indication should never appear. It would be a means to ensure robustness against ground system defect.

In the current case of the A330/340, once the 5 min timer has been triggered, (i.e. as soon as the NO ATC REPLY is displayed), only one message is accepted as an answer to close the MIN/MRN loop. This means that if a STDBY or DEFERRED msg is received after the 5 min, it closes the loop. The ensuing “true” response to the request that would be sent after will be rejected and will not be displayed. It will trigger a downlink error message: “Unrecognized MRN”

To cope with this situation, we would recommend the following:

- To send a STDBY message before the 5 min timer has expired (but as the timer has been started at the time the message was being sent, by the time the controller receives the request through Satcom, a good amount of time has gone). Once the STDBY message is received, the timer is reinitiated for another 5 minutes.

- Then, only one answer will close the loop, whatever the time it is being sent.

A subsequent STDBY could be re-sent within the 5 min of the first one, and it would re-arm the timer. But we do not recommend this as the crews might be lost receiving several STDBY messages.

2.3	Needless to say, an alternative to Airbus's suggestion that STANDBY messages be sent early enough to ensure that the five minutes have not elapsed is to never send STANDBY messages to their aircraft (except the A380).

2.4	The effect of the functionality described by Airbus can be seen in the following actual CPDLC dialogue:

| DN | 04:35:27 | [DM6] REQUEST F380 |
| --- | --- | --- |
| UP | 04:42:49 | [UM1] STANDBY |
| UP | 04:43:17 | [UM20] CLIMB TO AND MAINTAIN F380 [UM46] CROSS 63N030W AT F380 [UM129] REPORT LEVEL F380 |
| DN | 04:43:39 | [DM62] ERROR UNRECOGNIZED MRN |

In this case the STANDBY message was received by the aircraft more than 5 minutes after it had sent the downlink message and as a result the clearance was automatically rejected by the aircraft and not displayed to the flight crew.

2.5	As a result of this functionality an advisory has been issued to Reykjavik controllers recommending that STANDBY messages not be sent to Airbus aircraft other than the A380.

2.6	The following GOLD provisions concerning the uplink STANDBY messages are pertinent to this discussion:

4.3.3.4 The controller should use the uplink UM 1 STANDBY message element to provide advice to the flight crew that their requested clearance is being assessed, but is not readily available, for example, due to traffic or delays in coordination with the next sector or ATSU.

4.3.3.5 The ATSU should not automatically or manually send a UM 1 STANDBY to unconditionally acknowledge that it received a downlink request.

4.3.3.6 If the controller sends a UM 1 STANDBY response, the controller should subsequently respond again within a reasonable period of time, e.g. 10 minutes, or as required.

2.7	This issue was discussed at the recent at the NAT ATMG/39 meeting in March 2012 and the following is from the summary of discussions from that meeting:

4.26 The NAT ATMG agreed that further examination of this subject was necessary but it appeared this system behaviour was inconsistent with the associated procedures documented in the GOLD.  Canada, Portugal and the United Kingdom confirmed that automatically sending STANDBY uplink messages was part of the standard operating practices instituted to avoid duplicate downlink messages or unnecessary voice contacts to confirm that CPDLC requests had been received by ATC.  Portugal advised that only some downlink messages would automatically elicit a STANDBY response and Iceland advised that this subject would be addressed at the upcoming NAT CNSG/6 meeting, planned to take place from 26 to 30 March 2012.  Portugal and Iceland would provide updates to NAT ATMG/40.

2.8	This issue was discussed at the NAT CNSG/6 meeting in March 2012 and the following is from the summary of discussions from that meeting:

3.24 Airbus recognized that this was not an intended behavior and the fix is not expected until after 2018.

3.25 The Group felt that the anomalous behavior of these specific aircraft types should be documented in the GOLD. ATC procedures and/or systems should be designed so as to minimize the impact. One option might be to not issue STANDBY messages to these aircraft types when it is considered that there will be a delay in the processing of the request.

3.26 The Group noted that Canada, Portugal and the United Kingdom were automatically sending STANDBY uplink messages as a form of acknowledgement of receipt of a downlink. Portugal advised that only some downlink messages would automatically elicit a STANDBY response. The Group noted that the GOLD did not support the automatic STANDBY. If these would continue to be used in the NAT, then Appendix E of the GOLD would need to be amended. IFALPA noted that the use of STANDBYs should be limited only to the cases where requests required longer time to respond. The NAT service providers agreed to study whether the removal of automatic STANDBY responses would be beneficial and report to the next meeting.

- Recommendation

- The Meeting is invited to note the information provided in this paper.

END