Thirteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/13)

(Reykjavik, Iceland - 19-22 June 2012)

Agenda Item 4:  Provide Status on CPWG/12 Actions

LOST FUEL BURN SAVINGS DUE TO LACK OF FANS AND RNP4

(Action Item #CP12-01)

(Presented by the Federal Aviation Administration)

SUMMARY

This paper identifies denied aircraft requests for climb to optimum altitudes and places a value on the increased fuel burn due to lack of Future Air Navigation System (FANS) equipment and RNP certification

- Introduction

  - Currently in the Oakland Oceanic Flight Information Region (FIR), approximately 50 percent of the flights are Future Air Navigation System (FANS) equipped.  However, only around 25 percent of flights flight plan Required Navigation Performance 4 (RNP 4) equipage.  When aircraft are FANS equipped and RNP 4 certified, Oakland Oceanic controllers can apply Automatic Dependent Surveillance – Contract (ADS-C) 30 nautical mile (NM) lateral/30 NM longitudinal separation between pairs of like equipped aircraft.  Smaller separation standards allow aircraft to operate at more fuel efficient routes and altitudes.  This paper focuses on extra fuel burn due to denied requests because of lack of aircraft FANS and RNP 4 equipage.

- Discussion

  - Any FANS equipped aircraft should be able to qualify for RNP 4 certification. Even though 50 percent of flights in the Oakland Oceanic FIR are FANS equipped, there is still a gap of 25 percent of flights that are capable of RNP 4 but that do not flight plan indicating RNP 4 equipage.  This gap produces a cost to operators in terms of extra fuel burn.

  - Some operators do not flight plan RNP 4 because of the extra cost associated with more frequent ADS-C reports.  A FANS RNP 4 flight planned aircraft in the Oakland Oceanic FIR receives an ADS-C reporting rate  of 832 seconds (13 minutes 52 seconds).  A FANS  RNP 10 aircraft receives an ADS-C reporting rate of 1600 seconds (26 minutes 40 seconds).  So it is true that a FANS RNP 4 aircraft will have more ADS-C reports operating on the same routes in the Oakland FIR.

  - However, when you examine the overall costs, it is more efficient to flight plan with RNP 4 equipage. Over an 8 hour flight, an RNP 4 aircraft will send 35 ADS-C periodic reports.  Over the same 8 hour flight, an RNP 10 aircraft will send 18 ADS-C periodic reports.  The difference is 17 extra ADS-C reports for an RNP 4 aircraft.  Assuming an average cost for an ADS-C periodic position report of 0.25 US dollars ($0.25), the extra cost in ADS-C reports adds up to $4.25.  Consider that a gallon of fuel weighs 6.65 pounds (lbs) and costs a conservative $3.25 a gallon.  A B744 held 1000 feet below its optimum altitude burns approximately 288 pounds per hour of fuel more than at their optimum altitude.  That means that the B744 will burn up that $4.25 in fuel in only 1.81 minutes by operating 1000 feet below its optimum altitude.  RNP 4 and FANS will greatly increase the likelihood that the aircraft will be able to operate at its optimum altitude.

  - States need to work with their operators to help them certify their aircraft as RNP 4 capable.  RNP 4 FANS equipped aircraft operate at more fuel efficient altitudes and reduce carbon dioxide (CO2) emissions.  Reductions in CO2 emissions reduce the impact of global aviation on the environment.

  - Oakland Air Route Traffic Control Center (ARTCC) conducted a study to place a value on the extra fuel burn that is caused by aircraft operating at altitudes below their optimum altitude due to lack of RNP 4 and FANS equipment.  The Federal Aviation Administration (FAA) felt this analysis would help operators recognize the potential savings with RNP 4 and FANS equipage.  The following are the details on how the extra fuel burn is calculated:

- To calculate the extra fuel burn, the FAA worked with the operators and International Air Transport Association (IATA) to develop a table of how much extra fuel each aircraft type burns when it is in thousand feet increments below the aircraft’s optimum altitude.  This table is listed in Attachment 1.

- To determine when an aircraft is below its optimum altitude, the program tracks when an aircraft makes a request for a climb clearance and the climb is denied by air traffic control (ATC).  The requested altitude is tracked as the aircraft’s optimum altitude.  The program examines the blocking traffic and looks to see if the conflict is same direction traffic and the distance to the traffic is 16 nautical miles (NM) or more (ADS-C Climb Descent Procedure[CDP]).  If these conditions are met, the program will track the time the aircraft is below their optimum altitude.

- The time the aircraft is below their optimum altitude is multiplied by the data in the extra fuel burn table.  This allows us to calculate the extra fuel burned because an aircraft is operating below optimum altitude.  The program also tracks interim step climbs and updates in requested altitude and figures this data in the calculation.

- For this initial calculation, 15 days of data (1 April– 16 April. 2012) were examined in the Oakland Oceanic FIR.  The results show that an extra fuel burn of 27,331kilograms (kg) (60,128 lbs) was experienced due to lack of RNP 4 and FANS equipment.  If the data are extrapolated over a 1 year time period, an annual extra burn of 665,045kg (1,463,114lbs) of fuel and an extra 2.1 million kg of CO2 emissions would be realized.

  - While this data is based on every aircraft being RNP 4 and FANS equipped, it does not capture all of the benefits that can be realized by this equipage:

- This paper does not capture the benefits related to the application of 30 NM lateral separation for pairs of RNP 4 aircraft.  It would be much more difficult to make this calculation.

- This paper does not capture the benefits associated with the application of 30 NM longitudinal separation for opposite direction pairs of RNP 4 aircraft after the aircraft have passed.  It would be much more difficult to make this calculation.

- This paper does not capture the benefits that are lost when an aircraft is denied a request for climb due to traffic, and the aircraft does not make subsequent requests for higher optimum altitudes because of the traffic.

- ATS route structures and Pacific Organized Track System (PACOTS) are developed based on a 50 NM lateral separation standard.  Extra savings could be realized if route structures could be revised based on a 30 NM lateral separation standard.

- This paper only captures the lost savings in the Oakland FIR.  It does not capture the lost savings in other FIRs.

- In the study, there were an additional 100 RNP 4 aircraft that were denied altitude changes with a spacing of 16 NM or more.  These aircraft were impacted by other non RNP 4 aircraft.  A savings loss was not calculated for these 100 aircraft, but the RNP 4 savings would be significantly higher.

- Recommendation

  - The meeting is requested to:

- Recognize the benefits of RNP 4 and FANS equipage;

- Consider certifying FANS equipped aircraft as RNP 4; and

- Consider equipping aircraft with satellite FANS and RNP 4 certification.

| Aircraft Type A320, Flight length 2500NM, Average weight |  |
| --- | --- |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 36 |
| 2000 ft below optimum altitude | 72 |
| 3000 ft below optimum altitude | 118 |
| 4000 ft below optimum altitude | 172 |
| 5000 ft below optimum altitude | 254 |
| 6000 ft below optimum altitude | 336 |
|  | No data used B757 data |
| Aircraft Type A332, Flight length 4454NM, Average weight |  |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 35 |
| 2000 ft below optimum altitude | 71 |
| 3000 ft below optimum altitude | 136 |
| 4000 ft below optimum altitude | 182 |
| 5000 ft below optimum altitude | 251 |
| 6000 ft below optimum altitude | 321 |
|  | Extrapolated Data |
| Aircraft Type B737, Flight length 2100NM, Average weight |  |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 13 |
| 2000 ft below optimum altitude | 24 |
| 3000 ft below optimum altitude | 53 |
| 4000 ft below optimum altitude | 89 |
| 5000 ft below optimum altitude | 142 |
| 6000 ft below optimum altitude | 272 |
| Aircraft Type B738, Flight length 2100NM, Average weight |  |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 13 |
| 2000 ft below optimum altitude | 24 |
| 3000 ft below optimum altitude | 53 |
| 4000 ft below optimum altitude | 89 |
| 5000 ft below optimum altitude | 142 |
| 6000 ft below optimum altitude | 272 |
| Aircraft Type B744, Flight length 5500NM, Average weight |  |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 131 |
| 2000 ft below optimum altitude | 133 |
| 3000 ft below optimum altitude | 348 |
| 4000 ft below optimum altitude | 397 |
| 5000 ft below optimum altitude | 761 |
| 6000 ft below optimum altitude | 800 |

| Aircraft Type B752, Flight length 2100NM, Average weight |  |
| --- | --- |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 48 |
| 2000 ft below optimum altitude | 81 |
| 3000 ft below optimum altitude | 119 |
| 4000 ft below optimum altitude | 150 |
| 5000 ft below optimum altitude | 214 |
| 6000 ft below optimum altitude | 254 |
| Aircraft Type B763/B764, Flight length 2100NM, Average weight |  |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 52 |
| 2000 ft below optimum altitude | 84 |
| 3000 ft below optimum altitude | 117 |
| 4000 ft below optimum altitude | 164 |
| 5000 ft below optimum altitude | 238 |
| 6000 ft below optimum altitude | 327 |
| Aircraft Type B772, Flight length 5500NM, Average weight |  |
| Altitude | Ave Additional Fuel burn per hour kg |
| 1000 ft below optimum altitude | 20 |
| 2000 ft below optimum altitude | 139 |
| 3000 ft below optimum altitude | 292 |
| 4000 ft below optimum altitude | 312 |
| 5000 ft below optimum altitude | 595 |
| 6000 ft below optimum altitude | 640 |