Thirteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/13)

(Reykjavik, Iceland - 19-22 June 2012)

Agenda Item 6 Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) Issues:

- Risks Associated with Bearing/Distance Waypoints

(Presented by Isavia)

SUMMARY This information paper explores the risks associated with bearing/distance waypoints and asks the question if the use of such waypoints is safe within the NAT operating environment.

- Introduction

  - Discussion has recently started at Isavia concerning the processing of waypoints that are defined as bearing/distance from navaids or significant points. This discussion has led Isavia to question whether the use of such waypoints is safe within the NAT operating environment.

- Discussion

2.1	After the November 2012 change the PANS-ATM specifies the following for waypoints specified as bearing and distance from a significant point:

— Bearing and distance from a significant point:

The identification of the significant point, followed by the bearing from the point in the form of 3 figures giving degrees magnetic, followed by the distance from the point in the form of 3 figures expressing nautical miles. In areas of high latitude where it is determined by the appropriate authority that reference to degrees magnetic is impractical, degrees true may be used. Make up the correct number of figures, where necessary, by insertion of zeros — e.g. a point 180° magnetic at a distance of 40 nautical miles from VOR “DUB” should be expressed as DUB180040.

2.2	If waypoints defined as bearing and distance from a significant point are used in the NAT environment then the Flight Data Processing Systems (FDPSs) need to automatically convert the “bearing/distance from significant point” information into a latitude/longitude position. The derived latitude/longitude position then becomes the basis for all route based computations such as the conflict probe.

2.3	The Flight Management Computer (FMC) in the aircraft will as well derive a certain location from the “bearing/distance from significant point” information. It is vital that the FDPS derived position and the aircraft derived position is identical.

2.4	Isavia have identified the following issues with regard to the processing of “bearing/distance from significant point” information in the context of the Reykjavik CTA environment:

      - According to the PANS-ATM the bearing from the significant point shall normally be magnetic. The variation at most significant points is not published and must therefore be derived from a data base of isogonic lines in the FDPS and aircraft FMS and those must be identical in order to derive the same position. In the Reykjavik CTA the variation is up to values that are over 60° with changes up to more than 1° a year.

      - The variation information that is stored in FMCs and FDPSs must be obtained from official documentation such as AIPs. Some AIPs only publish variation for navaids and not any other waypoints. The rapid change of variation mentioned in a) above requires a frequent update of variation information in different source documents, but those updates may not happen at the same time. If the source information is different between FMCs and FDPSs then the derived position from the bearing/distance waypoint will different. At least some FMCs seem to obtain the variation from the system magnetic variation model regardless of the variation defined for the waypoint in the data base (see Attachment B).

      - Isavia have received information that the data base of isogonic lines in several aircraft of at least one airline had an error of several degrees. This could result in a significant difference in the position derived by the FDPS and the FMC. See Attachment A for a Boeing Aeromagazine article about the update of Magnetic Variation Tables in aircraft.

      - The PANS-ATM notation for a “bearing/distance from significant point” (DUB180040) does not specifically indicate whether the bearing is magnetic or true, the default is magnetic except at "high latitudes" where true may be used. No definition of high latitudes or suggested method of indicating true is given. In the absence of knowing whether the position is based on a magnetic or true bearing ATC cannot positively derive the position from the FPL.

      - Isavia consider that using true bearing is the only safe way of using bearing/distance from non-navaids waypoints. Unfortunately ICAO have not stipulated that requirement in the PANS-ATM.

      - At least some FMSs automatically switch to True navigation from Magnetic navigation when passing a specific latitude, for example 73°N. It is not clear to Isavia how such FMCs treat VORs that are aligned magnetic north or NDBs that have all directions defined in magnetic. It is not at all clear to controllers whether the aircraft is navigating true or magnetic in the northern latitudes.

2.5	As a result of those concerns Isavia controllers are not allowed to clear aircraft via waypoints defined as bearing and distance from a significant point and the Clearance window in the FDPS has been amended to preclude entering waypoints defined as bearing and distance from a significant point.

2.6	The following attachments are provided as background reading material:

a)	Boeing Aeromagazine article “Correcting the Effects of Magnetic Variation”.

b)	Universal Avionics Operator´s Training Manual 1000/1100, chapters concerning FMS vs. VOR Navigation, Magnetic Variation Anomalies, FMS Variation & VOR Declination and Application of Magnetic Variation of FMS Navigation.

2.7	This issue was discussed at the NAT SARSIG/15 meeting in March 2012. The following is from the summary of discussions from that meeting:

7.2 The NAT SARSIG was advised that a safety issue had been identified with the use of waypoints defined as a range and bearing from a significant point or a navigation aid. It had been discovered that different navigational data bases might not define the same magnetic variation for the same locations; in many cases, this would differ also from the values used in Flight Data Processing Systems (FDPS). As a result, the actual position of an aircraft could be considerably different from what would be calculated by the FDPS and so could the actual spacing between two aircraft. Accordingly, Iceland had implemented a policy whereby flight planned waypoints defined as range and bearing from a navigation aid or a significant point would not be used in ATC route clearances. Rather, the flight would be cleared using waypoints defined by latitude and longitude or via defined significant points or via navigation aids. The NAT SARSIG was advised this issue would also be raised at the upcoming meeting of the Cross Polar Working Group, as the effects would be most severe at higher latitudes. The NAT SARSIG agreed this matter should be examined by the NAT OPS/AIR sub-group and also agreed to bring it to the attention of the NAT IMG. Canada advised that a similar issue had been studied as part of the work done to support the introduction of ATS surveillance services based on Automatic Dependent Surveillance – Broadcast (ADS-B) in the Hudson Bay area and that relevant information from this study would be forwarded to Iceland.

2.8	This issue was discussed at the NAT IMG/40 meeting in May 2012. The following is from the draft summary of discussions from that meeting:

7.26 The NAT IMG was advised that a safety issue had been identified with the use of waypoints defined as a range and bearing from a significant point or a navigation aid. It had been discovered that different navigational data bases might not define the same magnetic variation for the same locations; in many cases, this would differ also from the values used in Flight Data Processing Systems (FDPS). As a result, the actual position of an aircraft could be considerably different from what would be calculated by the FDPS and so could the actual spacing between two aircraft. Accordingly, Iceland had implemented a policy whereby flight planned waypoints defined as range and bearing from a navigation aid or a significant point would not be used in ATC route clearances. Rather, the flight would be cleared using waypoints defined by latitude and longitude or via defined significant points or via navigation aids. The NAT IMG noted that this issue would also be raised at the upcoming meeting of the Cross Polar Working Group, as the effects would be most severe at higher latitudes. It was noted that the NAT SARSIG agreed this matter should be examined by the NAT OPS/AIR sub-group and also brought it to the attention of the NAT IMG. Canada advised that a similar issue had been studied as part of the work done to support the introduction of ATS surveillance services based on Automatic Dependent Surveillance - Broadcast (ADS-B) in the Hudson Bay area and that relevant information from this study would be forwarded to Iceland.

7.27 Therefore the NAT IMG agreed to the following:

NAT IMG Decision 40/WP07 – Waypoints defined by range and bearing

That:

a)	the provision of route clearances which include waypoints defined as a range and bearing from a navigation aid or significant point be forbiden in the airspace where ATS surveillance services are not provided; and

b)	determine what steps should be taken to address this issue on a more global basis.

- Recommendation

- The Meeting is invited to note the information in this paper.

- ATTACHMENT A

Boeing Aeromagazine article on “Correcting the Effects of Magnetic Variation”.

is provided in a separate document.

ATTACHMENT B

Universal Avionics Operator´s Training Manual 1000/1100, chapters concerning FMS vs. VOR Navigation, Magnetic Variation Anomalies, FMS Variation & VOR Declination and Application of Magnetic Variation of FMS Navigation

is provided in a separate document.

END