Thirteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/13)

(Reykjavik, Iceland 19-22, June 2012)

Agenda Item 4:   Provide Status on CPWG/12 Actions

Status of Six-Month Trial to Track Load Russian Trans East Entry Points

FRENK, KUTAL, and ERNIK at Zero Minutes

(CP12-05)

(Presented by Federal Aviation Administration)

SUMMARY This working paper presents an update on the status of the six-month trial to track load Russian Trans East entry points FRENK, KUTAL, and ERNIK at zero minutes.

1.	Introduction

# Prior to November 2011, westbound traffic flying from North America through Russian airspace had to transition between two very different environments.  Close cooperation between operators and Air Navigation Service Providers (ANSPs) along with advances in technology have reduced limitations on aircraft transiting 169W.  West of the Russian Flight Information Region (FIR) boundary, aircraft are limited to fixed routes and, until November 2011, were required to operate under conventional vertical separation minima using meters.  Anchorage Air Route Traffic Control Center (ZAN) continues efforts to move aircraft from a relatively unrestricted environment to specific fixes with an expectation of a preferred altitude at those points.

# 1.1.1	Aircraft at the same altitude transiting the Cross Polar (CP) and the Russian Trans East (RTE) routes are routinely track loaded at 10 minutes-in-trail.

# On November 17, 2011, State ATM implemented Reduced Vertical Separation Minimum (RVSM). Operators are now able to transition from North America into Russian airspace seamlessly with more available altitudes.

1.2.1	The implementation of RVSM in Russia created additional usable altitudes, which allows a greater number of aircraft to transit between the Anchorage FIR and the Russian FIRs in a given hour.  Realistically, RVSM allowed for three additional altitudes over the majority of fixes, with four new altitudes becoming available over fixes DEVID, NALIM, RAMEL, NIKIN, ORVIT, PILUN, and LISKI.

- Discussion

2.1	ZAN recognized the increased efficiency created by the additional RVSM altitudes and proposed a six-month trial to track load three RTE fixes at zero minutes versus the normal 10 minutes, for aircraft operating at the same altitude and on the same track.  The trial commenced on January 25, 2012, and aircraft crossing ZAN/Russian FIR boundary fixes FRENK, KUTAL, and ERNIK have been track loaded at zero minutes. (See Attachment A, RTE Fixes & PAZA NOTAM)

  - RTE Zero Minute Track Load Numbers from February thru April, 2012:

  - Based on positive customer feedback and comments, and observations by ZAN controllers and the Traffic Management Unit, the trial appears to be meeting everyone’s objectives.

- “From Air Canada’s perspective, no news is good news! I had one report shortly after the trial began of a conflict with one of our flights and 2 others – ours was a B767 so much slower; however that’s all I have heard, everything seems to be going very well from our side.”

- “Therefore, I would say from Delta’s perspective all is well and we thank you for you efforts in proposing and initiating this trial and we look forward to this being SOP for these entry points and expanding on a trial basis to other RTE entry points.”

- “CPA has no comment to offer, as there has been no noticeable impact or change on our operation.”

- American Airlines:  “I have attached a spreadsheet of the AAL flights from Jan. 25 to April 5 that was planned over the 3 waypoints in question. Of the 30 flights that flew over those fixes, six did not report to us their crossing altitude. Of the remaining 24, only four received a crossing altitude lower than planned and only one of those four was what we would consider significant (FL340 vs. FL380). From our point of view, the zero minute track load program appears to be working well and not causing significant issues for our operations.”

- TMU Response to AAL:  “I was able to fill most of the gaps in the spreadsheet that you attached. In the cases where there was missing data, the planned altitude and actual altitude received were the same. In addition, in every case where the actual and planned altitudes differed, there was not a traffic situation and the altitude issued was a pilot request. I am attaching the revised version that includes an additional column that describes the reason for the altitude change. Thanks for providing the feedback and I hope that what I provided to you helps as well.”

2.4	With the apparent success of this trial, ZAN is prepared to extend the length of the trial and possibly include additional boundary fixes.

- Recommendation

  - The meeting is invited to:

  - review the information contained in this paper; and

  - endorse the extension of the trial and the inclusion of additional boundary fixes.

A0077/12 - QPFCH EFFECTIVE JANUARY 25, 2012 AT 0700 UTC, ANCHORAGE ARTCC WILL BEGIN A 6 MONTH TRIAL OF TRACK LOADING RUSSIAN TRANS EAST (RTE) PAZA ANCHORAGE (ARTCC), AK.. ENTRY FIXES FRENK, KUTAL, AND ERNIK UTILIZING ZERO MINUTES SAME ALTITUDE, SAME TRACK. TACTICAL SEQUENCING VIA SPEED CONTROL AND/OR VECTORS MAY BE UTILIZED TO ENSURE REQUIRED IN TRAIL SPACING FOR AIRCRAFT AT THE SAME ALTITUDE. IT IS THE RESPONSIBILITY OF EACH OPERATOR TO SUBMIT ACCURATE DEPARTURE DATA TO ASSIST OTHER FLIGHT DISPATCHERS WITH THEIR FLIGHT PLANNING AND TO PROVIDE AIR TRAFFIC FACILITIES WITH EARLY INTENT INFORMATION. CUSTOMERS MUST FILE A TRACK REQUEST MESSAGE (TKF) UTILIZING THE EXISTING RTE TRACK ADVISORY PROGRAM PROCEDURES AS DESCRIBED IN PAZA INTERNATIONAL NOTAM A0071/12. CONTACT ANCHORAGE ARTCC TRAFFIC MANAGEMENT AT (907) 269-1840 FOR ANY QUESTIONS. RTE TRACK USERS ARE REQUESTED TO PROVIDE FEEDBACK TO DALE.ALEXANDER@FAA.GOV AND GREG.HOWARD@FAA.GOV. FL280 - FL600, WIE UNTIL UFN.