Fourteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/14)

(Chicago, USA – 10-14, December 2012)

Agenda Item 5:   Provide Status on CPWG/13 Actions

Status of Six-Month Trial to Track Load Russian Trans East Entry Points

PILUN, LISKI, and MARCC (BESAT) at Zero Minutes

(Action Items # CP12-05 and CP13-03)

(Presented by Federal Aviation Administration)

SUMMARY This working paper presents an update on the status of the six-month trial to track load Russian Trans East entry points PILUN, LISKI, and MARCC (BESAT) at zero minutes.

1.	Introduction

# Prior to November 2011, westbound traffic flying from North America through Russian airspace had to transition between two very different environments.  Close cooperation between operators and Air Navigation Service Providers (ANSPs) along with advances in technology have reduced limitations on aircraft transiting 169W.  West of the Russian Flight Information Region (FIR) boundary, aircraft are limited to fixed routes and, until November 2011, were required to operate under conventional vertical separation minima using meters.  Anchorage Air Route Traffic Control Center (ZAN) continues efforts to move aircraft from a relatively unrestricted environment to specific fixes with an expectation of a preferred altitude at those points.

# The procedure established for traffic transiting the Cross Polar (CP) and the Russian Trans East (RTE) routes is to track load aircraft at the same altitude at 10 minutes-in-trail.

# On 17 November 2011, State ATM implemented Reduced Vertical Separation Minimum (RVSM). Operators are now able to transition from North America into Russian airspace seamlessly with more available altitudes.  The implementation of RVSM in Russia created additional usable altitudes, which allows a greater number of aircraft to transit between the Anchorage FIR and the Russian FIRs each hour.  Realistically, RVSM allowed for three additional altitudes over the majority of fixes, with four new altitudes becoming available over fixes DEVID, NALIM, RAMEL, NIKIN, ORVIT, PILUN, and LISKI.

# State ATM also established three new fixes in the Arctic, which created additional routing and altitude alternatives.

- Discussion

- ZAN recognized the increased efficiency created by the additional RVSM altitudes and proposed a six-month trial to track load three RTE fixes at zero minutes versus the normal 10 minutes, for aircraft operating at the same altitude and on the same track.  The original trial commenced in January 2012, at which time aircraft crossing the ZAN/Russian FIR boundary at fixes FRENK, KUTAL, and ERNIK were track loaded at zero minutes.  On 28 August 2012, with agreement from the adjacent FIRs, ZAN began permanently track-loading fixes FRENK, KUTAL, and ERNIK at zero minutes.

- In August 2012, ZAN began a new six-month trial of track loading the ZAN/Russian FIR boundary fixes PILUN, LISKI, and MARCC (BESAT) at zero minutes.  (See Attachment A, RTE Fixes & PAZA NOTAM)

- RTE Zero Minute Track Load Numbers from August 28 thru October 2012:

| RTE - ZERO MINUTE TRACKLOAD NUMBERS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Aug 28-31 | Sept | Oct | Totals | Daily Avg. |
| PILUN | 5 | 147 | 112 | 264 | 4 |
| LISKI | 8 | 153 | 148 | 309 | 5 |
| MARCC | 5 | 94 | 185 | 284 | 5 |
| Totals | 18 | 394 | 445 | 857 |  |

- Based on positive customer feedback, and observations by ZAN controllers and Traffic Management Unit, the trial appears to be meeting everyone’s objectives.

- Recommendation

  - The meeting is invited to:

  - review the information contained in this paper; and

  - endorse the extension of the trial and the inclusion of additional boundary fixes.

A0158/12 – QPFCH EFFECTIVE AUGUST 28, 2012 AT 0700 UTC, ANCHORAGE ARTCC WILL PAZA ANCHORAGE (ARTCC), AK.. BEGIN A 6 MONTH TRIAL OF TRACK LOADING RUSSIAN TRANS EAST (RTE) ENTRY FIXES PILUN, LISKI AND MARCC UTILIZINING ZERO MINUTES SAME ALTITUDE, SAME TRACK. TACTICAL SEQUENCING VIA SPEED CONTROL AND/OR VECTORS MAY BE UTILIZED TO ENSURE REQUIRED IN TRAIL SPACING FOR AIRCRAFT AT THE SAME ALTITUDE. IT IS THE RESPONSIBILITY OF EACH OPERATOR TO SUBMIT ACCURATE DEPARTURE DATA TO ASSIST OTHER FLIGHT DISPATCHERS WITH THEIR FLIGHT PLANNING AND TO PROVIDE AIR TRAFFIC FACILITIES WITH EARLY INTENT INFORMATION. CUSTOMERS MUST FILE A TRACK REQUEST MESSAGE (TKF) UTILIZING THE EXISTING RTE TRACK ADVISORY PROGRAM PROCEDURES AS DESCRIBED IN PAZA INTERNATIONAL NOTAM A0071/12. CONTACT ANCHORAGE ARTCC TRAFFIC MANAGEMENT AT (907) 269-1840 FOR ANY QUESTIONS. RTE TRACK USERS ARE REQUESTED TO PROVIDE FEEDBACK TO GREG.HOWARD@FAA.GOV OR BILL.COMER@FAA.GOV. FL280 - FL510, 28 AUG 07:00 2012 UNTIL 26 FEB 06:59 2013.