Fourteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/14)

(Chicago, USA – 10-14 December 2012)

Agenda Item 8: Communication, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) Issues

- Survey – Level requests within Reykjavik CTA

(Presented by Isavia)

SUMMARY This paper presents information on a survey made by Isavia to find out the number of pilot change requests and the percentage of which they were approved.

- Introduction

- At a NAMEUR meeting in May 2012, International Air Transport Association (IATA) requested information from Air Navigation Service Providers (ANSPs) in the North Atlantic (NAT) region about the number of pilot change requests made and the percentage which were approved.

- Discussion

- Survey method

- Isavia does not have an automatic process in the Flight Data Processing System for exporting statistics about level requests. Therefore a certain period had to be selected for closer observation.

- The period chosen was 15-07-2012 to 31-07-2012. All westbound aircraft transiting Reykjavik CTA crossing 60W at or north of 66N060W (EPMAN) were counted. Note, that traffic departing or arriving in Greenland, Iceland or Faroe Islands was excluded.

- Aircraft entry and exit flight level were noted as well as any flight level requests made via datalink (CPDLC or ARINC 623) or Iceland radio. It is important to note that this excludes any requests made by aircraft to Reykjavik Control on VHF as those requests are not available in the AFTN or datalink log. The number of requests is therefore likely to be higher than indicated in this report.

- This survey only focuses on level changes as those are the most frequent type of requests made by aircraft in Reykjavik CTA. It is the opinion of the surveyor that most of speed change requests are approved. Route change requests are not common in the area.

## Results from the survey

- Requests

A total of 864 aircraft match the criteria used in this survey.  406 aircraft (47%) made a level request and 458 (53%) did not make any level request.

| Level change request in Reykjavik CTA |  |  |
| --- | --- | --- |
| No request | 458 | 53% |
| 1 request | 366 | 42% |
| 2 requests | 34 | 4% |
| 3 requests | 6 | 1% |
| TOTAL | 864 |  |

- Approved requests

A total of 452 requests were made. 357 requests (79%) were approved and 95 requests (21%) rejected. It should be noted that of those 95 rejected requests many were partially approved.

Example: 	Aircraft is at F340 and requests F360. F360 is not available but the aircraft is cleared to F350.

| Change requests in Reykjavik CTA |  |  |
| --- | --- | --- |
| Request approved | 357 | 79% |
| Request rejected or partially approved | 95 | 21% |
| TOTAL | 452 |  |

As shown above a total of 95 requests were rejected. 44 aircraft (46%) were cleared at a flight level 1000 feet below the requested flight level and 27 aircraft (29%) were cleared at a flight level 2000 feet below the requested flight level. 4 aircraft (4%) were cleared 3000 feet or more below the requested flight level.

| Difference between requested flight level and cleared flight level |  |  |  |
| --- | --- | --- | --- |
| Aircraft cleared above requested flight level | 2000‘ | 1 | 1% |
|  | 1000‘ | 19 | 20% |
| Aircraft cleared below requested flight level | -1000‘ | 44 | 46% |
|  | -2000‘ | 27 | 29% |
|  | -3000‘ | 2 | 2% |
|  | -4000‘ | 2 | 2% |
| TOTAL |  | 95 |  |

- Level changes

Of the 864 aircraft in the survey 548 aircraft (63%) changed level within Reykjavik CTA and 316 aircraft (37%) maintained the same flight level through the area.

| Level changes in Reykjavik CTA |  |  |
| --- | --- | --- |
| Changed flight level | 548 | 63% |
| Maintained flight level | 316 | 37% |
| TOTAL | 864 |  |

- Aircraft not requesting a level change

As indicated earlier this survey only takes into account level change requests made via datalink or Iceland Radio. That leaves out all change requests made directly to Reykjavik Control via VHF.

Of those 458 aircraft not requesting a level change, 168 aircraft (37%) exited Reykjavik CTA at a higher level than the entry level. It may be concluded that some of the aircraft climbed as a result of a level request made directly to Reykjavik Control on VHF. Other will have climbed on ATC instruction due to conflicting traffic.

| Level changes on aircraft not requesting a level change through CPDLC or Iceland Radio |  |  |  |
| --- | --- | --- | --- |
| Exit level > Entry level | 4000‘ | 3 | 0.7% |
|  | 3000‘ | 12 | 2.6% |
|  | 2000‘ | 92 | 20.1% |
|  | 1000‘ | 61 | 13.3% |
| Exit level = Entry level | 0‘ | 288 | 62.9% |
| Exit level < Entry level | -1000‘ | 0 | 0.0% |
|  | -2000‘ | 2 | 0.4% |
| TOTAL |  | 458 |  |

- Recommendation

- The Meeting is invited to note the information provided in this paper.