Fifth Meeting of the Pacific Project Team (PPT/5)

(Bodo, Norway – 13-17 May 2013)

Simulation of TRACK2 User Preferred Route (UPR)

(Divergence from TRACK2)

(Presented by Civil Aviation Bureau of Japan)

SUMMARY This paper provides information on the outcome obtained from the computer simulation under relaxation of restrictions to setup for User Preferred Route (UPR) of Pacific Organized Track System (PACOTS) from TRACK 2

- Introduction

1.1	This paper provides information that was discussed during the 38th Meeting of the Informal Pacific ATC Co-ordinating Group (IPACG/38) on the outcome obtained from the computer simulation under relaxation of restrictions to setup for User Preferred Route (UPR) of Pacific Organized Track System (PACOTS) from TRACK 2.

- Action by the Meeting

2.1	The meeting is invited to:

  - review the information contained in this Paper for discussion.

THE THIRTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/38)

(Mountain View, California, USA, 22-26 April 2013)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Simulation of TRACK2 UPR (Divergence from TRACK2)

(Presented by Civil Aviation Bureau, Japan and Electronic Navigation Research Institute)

SUMMARY

This information paper provides the outcome obtained from the computer simulation under relaxation of restrictions to setup for User Preferred Route (UPR) of Pacific Organized Track System (PACOTS) from TRACK 2.

- Introduction

  - The User Preferred Route (UPR) of Pacific Organized Track System (PACOTS) TRACK 2 has  not been applied  to avoid problems with extreme complex route constructions caused by crossing/converging with TRACK 1 or TRACK 3. For the same reason, the UPRs of both TRACK 1 and TRACK 3 must be planned so as to remain 50NM north or south of TRACK 2.

  - Currently, a certain benefit was estimated when the gate of TRACK1 was set above KALNA which is the waypoint on G344, or the point of 160 degrees east of TRACK2 was set above 43 degrees north by the result of the computer simulation executed by Electronic Navigation Research Institute (ENRI). On this basis, Japan Civil Aviation Bureau (JCAB) has a plan to introduce the divergent route from North Pacific route for the east bound PACOTS.

  - Divergence from NOPAC is enabled, and opportunities to make the effective course to the North America area increase. That’s why operators will have more opportunities to make beneficial routes than at present.

  - On the other hand, it is up to operators which TRACKs the operators will take into their flight plans. So the usage rate of TRACKs differs day by day, but the utilization level of TRACK 3 is relatively low compared with TRACK 1 or TRACK 2 in daily operations within the Fukuoka Flight Information Region (FIR).

  - For the purpose of beneficial use of the airspace, the computer simulation under conditions that UPR is allowed to diverge from TRACK 2, supposed that there is no TRACK3, within Fukuoka FIR was executed to make southward TRACK2 UPR for operators.

- Discussion

  - ENRI executed the simulation under two different conditions relevant to traffics for KSFO and KLAX as follows.

- ・Model 1 is based on a current rule for setting UPR of TRACK3 that must be planned so as to remain 50NM south of TRACK 2.

- ・Model 2 is based on an assumption that TRACK2 UPR is allowed only to branch southward without re-converging on TRACK2.

  - Specific days (= Target days) were chosen that the start gate of TRACK2 shows a different waypoint, SEALS, EMRON and KALNA in order from south. We picked up two days from each of the start gate, so target days are six days in total. The target days are defined by taking the initial letter of each start gate and add the number like DAY_S1, S2, E1, E2, K1 and K2.

  - Fifty-five flights from Asian airport including domestic airport to KSFO/KLAX were provided in flight control simulation. The flight schedule scenario was generated from the actual flight plan of the day of heavy traffic volumes.

  - The result shows fuel consumption benefit of the Model 2 in all target days by comparison with Model 1 (Fig. 1). The value varied from day to day. The DAY_E1 showed the biggest benefit of approximately 230 lbs per flight, and the DAY_E2 showed the smallest benefit of approximately 30lbs per flight, based on B747 fuel consumption rate.

Figure1.     The advantage of Model 2

  - The number of flights at the start gate of TRACK 2 in Model 2 increased by 39 percent from Model 1 at total count of the target days. Some traffic using a southern gate than TRACK 2 in the Model 1 move to a TRACK 2 start gate in Model 2.

  - Figure 2 shows the fuel consumption benefit per one flight for each destination. There was the difference in fuel consumption benefit between KLAX arrival flight and KSFO arrival flight. The KLAX flight shows more advantages than that for KSFO. The tendency is strong so that the start gate is the southern latitude.

Figure2.    The fuel consumption benefit per one flight for KLAX and KSFO

  - The result is that the Model 2 was more beneficial in total than Model 1 under the computer simulation environment. However, the traffic might concentrate on one gate and the route constructions tend to be complex with plural converging points. Upon thinking of possibilities to set TRACK2 UPR, careful consideration of complexity of route construction is required.

  - ATMC cooperates with ENRI from now on and performs inspection about the route constructions in order to provide operators with benefits as much as possible.

- Action by the meeting

  - The meeting is invited to note the information provided.