Fifth Meeting of the Pacific Project Team (PPT/5)

(Bodo, Norway – 13-17 May 2013)

USER PREFERRED ROUTE (UPR) AND PACOTS UPDATE

(Presented by Federal Aviation Administration and Civil Aviation Bureau, Japan)

SUMMARY

This information paper provides an update on the status of User Preferred UPRs and Pacific Organized Track System (PACOTS) within the Pacific Region.

- Introduction

1.1	This paper provides information that was discussed during the 38th Meeting of the Informal Pacific ATC Co-ordinating Group (IPACG/38) and provides an update on the status of User Preferred UPRs and Pacific Organized Track System (PACOTS) within the Pacific Region.

- Action by the Meeting

2.1	The meeting is invited to:

`a. review the information contained in this Paper for discussion

THE THIRTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/38)

(Mountain View, California, USA, 22-26 April 2013)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

USER PREFERRED ROUTE (UPR) AND PACOTS UPDATE

(Presented by Federal Aviation Administration and Civil Aviation Bureau, Japan)

SUMMARY

This information paper provides an update on the status of User Preferred UPRs and Pacific Organized Track System (PACOTS) within the Pacific Region.

- Introduction

  - UPRs have been utilized in the Pacific Region for some time.  The usage of UPRs has grown significantly, and trials of UPRs in conjunction with various organized tracks are still being conducted.

- Discussion

  - To ensure efficient and safe management of air traffic, operational requirements from the various air navigation service providers (ANSPs) are necessary.  These requirements are developed and published in Aeronautical Information Publications (AIPs), International Notices to Airmen (NOTAMs), and/or the Pacific Chart Supplement.

  - It is recognized that these operational requirements often create negative impacts on the efficiency of various UPRs.  ANSPs must constantly review the restrictions imposed in order to ensure that potentially negative impacts on the efficiency of a particular UPR are eliminated wherever possible.

  - Also, it is important to realize that UPRs are not always an efficient means of moving aircraft through oceanic airspace.  For example, within Oakland OCA, the one area where unrestricted use of UPRs is not feasible is the Central East Pacific.  Due to the volume of traffic transiting the airspace between North America and Hawaii, adherence to a structured Air Traffic Service (ATS) Route system provides the most overall efficient operations.

  - The PACOTS Track 3 UPR trial has been ongoing since February 2012, without any operational issues.  Analysis of its use has revealed the following information:

    - The number of aircraft to San Francisco International Airport (KSFO) and Los Angeles International Airport (KLAX) from 1 March 2012 to 28 Feb 2013 (365 days) are approximately as follow:

To KSFO	To KLAX

7100 (19 /day)	12300 (34/day) - These figures included, “departed from Asia”

2000 (6/day)	4100 (11/day) - These figures included, “departed from Japan” only

Out of the aircraft departed from Japan not planned on PACOTS Track -

To KSFO	To KLAX

256	593

They are classified as follows:

On North Pacific (NOPAC)

147	153

Random route (cross multiple PACOTS Tracks)

30	195

Planned on PACOTS Track 3 UPR properly

79	245

  - During this period, 324 aircraft filed PACOTS Track 3.  A PACOTS Track 3 UPR bound for KSFO or KLAX has been operated approximately every day (1 aircraft/day), an increase from IPACG/37.

  - Based on usage and the fact that no operational problems result from the use of UPRs associated with Track 3, FAA and JCAB have mutually agreed that their use is no longer in a trial stage, but rather they are considered as a standard operational procedure.  The main operational requirement for PACOTS Track 1 and Track 3 UPRs is to remain at least 50 nautical miles (NM) laterally separated from PACOTS Track 2, which remains as the only eastbound PACOTS track that does not have an alternative UPR.

  - Two seasonal paper trials of UPRs associated with PACOTS Tracks E and F have been completed. The data shows a projected summer savings of 880 kilograms (kg) and winter savings of 299 kg of fuel burn per flight.  While these are significant savings, the operators noted that the paper trial restriction to remain 50 NM south of PACOTS Tracks C/D impaired the savings. If that restriction was removed, there was a potential average fuel burn savings of 7715 kg in the November paper trial. Recognizing this potential, the IPACG meeting looked for a way to realize these significant savings

  - The development of advanced oceanic air traffic control systems has allowed for efficient control of an increased volume of traffic.  As a result, an operational trial began on 15March 2012 to merge traffic from PACOTS Track C and E which is resulting in development of overall more efficient westbound PACOTS tracks between North America and Japan.

  - The following procedures are used when merging PACOTS Tracks C and E:

    - When Oakland Air Route Traffic Control Center (ARTCC) generates the westbound PACOTS Tracks, they will evaluate the tracks to see if there is at least a 200 pounds (lbs) of fuel burn savings on either Track C or Track E by merging the two tracks.  If the minimum savings exists, then Track C and E will be published with a merge.

    - Oakland ARTCC Traffic Management uses the Track Advisory (TA) system to manage traffic loading onto the merged tracks.  TA is a gateway reservation program that is used to plan the loading of aircraft onto PACOTS and the Russian Routes.  Operators submit a request for their choices with a time window and altitude at a gateway point for a flight.  TA compiles the operator choices at a 1650 UTC and then publishes a Gateway Reservation List for the flights. The point where Tracks C and E merge would be used as the “gateway” fix for a merged scenario. TA will provide for a managed traffic flow at the merge point for the two tracks.

    - The proposed operational trial will run for one year, in order to evaluate the seasonal differences in the merged tracks.  During the trial, collaborative methods will be utilized to enhance the process, and if safety hazards are encountered which cannot be mitigated, the trial will be suspended or terminated.  Periodic teleconferences will be scheduled to discuss the trial, its effectiveness, and provide a forum for sharing any user concerns.

  - As an adjunct to this trial, an operational trial of UPRs associated with PACOTS Track F UPRs south of PACOTS Track E will begin this summer.  The details of the Track F UPRs are being finalized at this time and will be completed by the IPACG 38 meeting.  The basic guidelines for flight planning Track F UPRs are as follows:

    - Normal UPR guidelines such as avoid active military airspace, flight plan an appropriate departure and arrival routing.

    - Track F UPRs must remain at least 50 NM south of PACOTS Track E.

    - Pacots Track F upr aircraft do not have priority for altitude assignment over aircraft on an existing pacots, NOPAC or Central East Pacific (cep) traffic.

  - The paper trial to merge PACOTS Tracks C and E was very beneficial.  In the trial development several ways were discovered to improve westbound PACOTS track generation.  These improvements will yield more efficient PACOTS Tracks even when the PACOTS routes are not merged.  JCAB and the FAA will be further discussing ways to improve PACOTS track generation at the IPACG 38 meeting.

- Conclusion

  - The meeting is invited to:

    - Note the information provided;

    - Discuss ways to improve the efficiency of PACOTS and UPRs.