Fifteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/15)

(Bodo, Norway – 13-17 May 2013)

Agenda Item 5: Status on CPWG/14 Actions

- Progress to Improving Flight Planning Efficiency Through the Anchorage Flight Information Regions (FIRs)

- (Action Items CP08-12, 13-03, 14-05 and 14-11)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information describing improvements in flight planning efficiency through the Anchorage Flight Information Region (FIRs).

- Introduction

- By definition, the Cross Polar Work Group’s (CPWG’s) efforts, collectively and individually, are designed to, “ identify and resolve issues that impact the efficiency of ATS…”  Any steps taken in this regard, large or small, are worth noting.  This paper reports on three individual, but similar, efficiency improvements which have been completed for the Anchorage Flight Information Regions, (FIRs.)  These items correspond to CPWG Action Items CP13-03, 14-05 and 14-11.  These three in combination correspond to item CP08-12.

- Discussion

- The FAA’s Dynamic Ocean Track System-Plus (DOTS+) system is designed to organize air traffic so as to provide for a safe, orderly and efficient flow of traffic over the Anchorage / Magadan, Murmansk and Petropavlovsk-Kamchatsky FIR boundaries.  DOTS+ accomplishes this by accepting Operators’ Track Slot Requests, analyzing for conflicts, and issuing a Gateway Reservation List (GRL).  The system permits variability in the track load time parameter used to create the GRL.  Historically this time parameter, for most gateways, was set between 15 and 20 minutes.  The selection of the parameter is affected by considerations such as the cross border longitudinal separation minima required, current air/ground communications effectiveness, available altitudes for deconfliction, etc.   Action item CP13-03 reported on a trial undertaken to reduce the time parameter for gateway Fixes LISKI, PILUN and MARCC (to zero minutes) and CP14-05 contained a suggestion to reduce the parameter for fixes KUNAD, LUMES, KOKES, RUSOR and BAMOK.

- The trial over LISKI, PILUN and MARCC has concluded.  Satisfactory results were observed and the DOTS+  track load time parameter has been operationally set at zero minutes.  After analysis (and in response to Action Item CPWG 14-05), the decision was taken to also set the operational track load parameter at zero for KUNAD, LUMES, KOKES, BAMOK, and RUSOR.  This information is reflected in the most recent addition of the DOTS+ Track Advisory User’s Guide (TAUG).

- Effective January 31, 2013, Anchorage has begun a zero minute track load trial for boundary point VALDA.  The outcome of this 6-month trial will be reported at CPWG16.

- Action Item CP14-11 concerns a request, made by the International Air Transport Association (IATA), that FAA examine the feasibility of removing Operators’ requirement to file and report a 141 West Longitude fix for westbound flights crossing the Anchorage Arctic FIR.  This procedural requirement serves multiple purposes; the most important of which is to assist Anchorage Air Traffic Controllers to ensure separation between parallel flights.

- Anchorage Air Route Traffic Control Center (ARTCC) has reviewed the ramifications of the suggested change.  Until such time as the FAA is able to bring the Anchorage Arctic FIR airspace into the Advanced Technologies and Oceanic Procedures (ATOP) Ocean21 system, controllers need a method, such as aircraft position reports, to achieve procedural separation.  An alternative method, beyond the 141 West position reports, has been suggested.  This alternate method entails changes in the current Anchorage automation system and relies on leveraging Nav Canada’s Canadian Automated Air Traffic System (CAATS).

- The modification of the Anchorage automation system is under review.  Negotiations to create the framework for leveraging the CAATS system are underway.  Successful outcomes in these efforts will permit Anchorage to lift the requirement for westbound aircraft to file a 141 West crossing point.  Eastbound flights will still be required to file a 141 West point and, due to the requirements associated with ICAO Alerting Service, westbound flights will still be required to report entering the Anchorage FIR.  Implementation of this change will be announced via Notices to Airmen (NOTAM).  Until then, operators are expected to comply with existing requirements.

- Recommendation

- The Meeting is invited to note the information provided in this paper.