Fifteenth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/15)

(Bodo, Norway13-17 2013)

Agenda Item 5: Provide Status on CPWG/14 Actions

- Status of Oakland Flight Information Region (FIR) Trial to Merge PACOTS Tracks C/E

- (Action Item CP 14-10)

(Presented by the Federal Aviation Administration)

SUMMARY This information paper provides an update on the current trial to merge Pacific Organized Track System (PACOTS) Tracks C and E to improve efficiency for westbound aircraft.

- Introduction

- Westbound PACOTS tracks are generated by the Oakland Traffic Management Unit (TMU), with Track E being the first track generated.  As a result, subsequent tracks may not be able to follow the most efficient route since they must be separated from Track E.  Analysis was conducted and a trial proposed to examine improved efficiencies by merging Tracks C and E.

- Discussion

- The development of advanced oceanic air traffic control systems has allowed for efficient control of an increased volume of traffic.  As a result, an operational trial began on 15March 2012 to merge traffic from PACOTS Track C and E which is resulting in development of overall more efficient westbound PACOTS tracks between North America and Japan.

- The following procedures are used when merging PACOTS Tracks C and E:

  - When Oakland Air route Traffic Control Center (ARTCC) generates the westbound PACOTS Tracks, they will evaluate the tracks to see if there is at least 200 pounds (lbs.) of fuel burn savings on either Track C or Track E by merging the two tracks.  If the minimum savings exists, then Track C and E will be published with a merge.

  - Oakland ARTCC Traffic Management uses the Track Advisory (TA) system to manage traffic loading onto the merged tracks.  TA is a gateway reservation program that is used to plan the loading of aircraft onto PACOTS and the Russian Routes.  Operators submit a request for their choices with a time window and altitude at a gateway point for a flight.  TA compiles the operator choices at a 1650 UTC and then publishes a Gateway Reservation List for the flights. The point where Tracks C and E merge would be used as the “gateway” fix for a merged scenario. TA will provide for a managed traffic flow at the merge point for the two tracks.

  - The proposed operational trial will run for one year, in order to evaluate the seasonal differences in the merged tracks.  During the trial, collaborative methods will be utilized to enhance the process, and if safety hazards are encountered which cannot be mitigated, the trial will be suspended or terminated.  Periodic teleconferences will be scheduled to discuss the trial, its effectiveness, and provide a forum for sharing any user concerns.

- As an adjunct to this trial, an operational trial of UPRs associated with PACOTS Track F UPRs south of PACOTS Track E will begin this summer.  The details of the Track F UPRs are being finalized at this time and will be completed by the IPACG 38 meeting.  The basic guidelines for flight planning Track F UPRs are as follows:

  - Normal UPR guidelines such as avoid active military airspace, flight plan an appropriate departure and arrival routing.

  - Track F UPRs must remain at least 50 NM south of PACOTS Track E.

  - Pacots Track F upr aircraft do not have priority for altitude assignment over aircraft on an existing pacots, NOPAC or Central East Pacific (cep) traffic.

- The trial to merge PACOTS Tracks C and E was very beneficial.  In the trial development several ways were discovered to improve westbound PACOTS track generation.  These improvements will yield more efficient PACOTS Tracks even when the PACOTS routes are not merged.

- Recommendation

- The Meeting is invited to note the information provided in this paper.