Fourth Meeting of the

Cross Polar Trans East Air Traffic Management Providers’ Working Group

(Edmonton, Alberta, 18-20 September 2007)

Agenda Item 4:  Review and update of Action Items

(Presented by the Federal Aviation Administration)

Update to Action Item 040730:  Standardize Polar Region Separation Standards

SUMMARY

This information paper presents an update on Action Item 040730.

1.	Introduction

# Action Item 040730 was opened during the third CPWG meeting as a result of a working paper presented by FAA.  That working paper suggested that the Cross Polar work group undertake the task of harmonizing lateral separation minimas used through out the Arctic FIRs.

2.	Discussion

# CPWG/3 acknowledged that harmonization of lateral separation standards would benefit users and ATS providers alike, however, due to various internal issues, i.e. regulatory, airspace management, etc., the group was not able to form a consensus on what the specific standard should be.

# As an interim step the group agreed to complete an ATS/CNS “adequacies” table so as to delineate and define what separations standards, as well as what air traffic management tools, are currently in use in the Arctic region.  The table is attached as appendix 1.

- Recommendation

  - The meeting is invited to review the attached “Adequacies Table” and to take such other action as may be possible to achieve the action item goal of a harmonized standard.

Arctic FIRs ATM/CNS Adequacies

| ADEQUACY | REYKJAVIK | EDMONTON | ANCHORAGE | MAGADAN | MURMANSK |
| --- | --- | --- | --- | --- | --- |
| COMMUNICATIONS |  |  |  |  |  |
| HF | Iceland Radio | Gander Radio | Gander Radio | Magadan Radio | Murmansk Control |
| CPDLC | Limited (2008 full implementation) | 2008? | Below 84° N | Below84°N | No |
| SAT VOICE | Yes | Below 84° N | Below 84° N | Below84°N | No |
| SURVEILLANCE |  |  |  |  |  |
| ADS | Yes | Below 84° N | 2008? | Below84°N | No |
| Coordination |  |  |  |  |  |
| Direct Voice | Yes | Yes | Yes | Yes | Yes |
| AFTN | Yes | Yes | Yes | Yes | Yes |
| AIDC | Yes | NAM 2008? | NAM & A/PAC ICD | No | No |
| SEPARATION STANDARDS |  |  |  |  |  |
| VERTICAL |  |  |  |  |  |
| Units | Feet | Feet | Feet | Meters | Meters |
| RVSM | Yes | Yes | Yes | No | No |
| LONGITUDINAL |  |  |  |  |  |
| 15 MINUTES Standard | No** | Yes | Yes | Yes | Yes |
| Use of MACH Number Technique? | Yes | Yes | Yes | Yes | Yes |
| LATERAL |  |  |  |  |  |
| RNP4 (30NM*) |  |  |  |  |  |
| RNP 10 (50NM) |  |  |  |  |  |
| CMNPS (60NM) |  | X |  |  |  |
| NAT MNPS (60NM) | X |  |  |  |  |
| Russian Domestic (60KM) |  |  |  | X | X |
| ARCTIC (90NM) |  |  | X |  |  |