Seventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/7)

(European and North Atlantic (EUR/NAT) Office of ICAO, Paris, France, 3-5 June 2009)

Agenda Item 4:	Provide Status on CPWG/6 Actions

- POLAR COMMUNICATIONS

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information on the performance of communications in the polar region. It also presents guidance on selection of high frequency (HF) communications for both air traffic control (ATC) and airline operations centers (AOC). During Sun Spot activity the polar region can be affected more than other regions and different forms of communication need to be attempted including different HF operators.

- Introduction

- Communication capability

- Operators must have effective communication capability for all portions of the flight route. Operators accomplish this by using a combination of very high frequency (VHF) voice, VHF data link, high-frequency high frequency (HF) voice, HF data link, satellite communication (SATCOM) voice, and SATCOM data link systems.  Communication in the polar region should be handled according to the applicable procedures described on en route charts. Both VHF and HF equipment are needed to communicate with air traffic control (ATC).  It is important to use only standard International Civil Aviation Organization (ICAO) terminology for effective communication with ATC; nonstandard terminology or jargon may create confusion.  SATCOM voice must be considered as a backup, although it is generally not available above 82 degrees north latitude with Inmarsat.  Iridium SATCOM voice, if equipped, will give coverage from pole to pole.

- Discussion

- History

2.1.1 	HF radio is perhaps the oldest form of airborne radio communications.  It provided the first primitive air-to-ground radio contacts as early as World War 1, as well as the radio links for Richard Byrd, Rene Fonk, Amelia Earhart and many other early pioneers of the 1920s and 1930s. It allowed World War II bombers and transports and piston-engine airliners to communicate over long distances.

2.1.2 	It was the standard means of en route communications in the continental United States until approximately 1960. Since that time, however, VHF has completely replaced HF for routine communications in most of countries.  Still, HF remains a viable technology and actually has grown in usefulness. In the past few decades, HF radio has benefited from digital tuning, single-sideband (SSB) efficiency and power, new antenna concepts, compact size, affordable price and installation simplicity.

- The Basics

2.2.1 	HF radio is not really very high on the frequency spectrum-at least not by modern standards.  HF is specifically defined as those radio frequencies from 2000 to 29999.9999 kHz, sometimes expressed as 2.0 to 29.9999 MHz.  This band starts just above the medium-frequency (MF) band, which includes the commercial AM radio spectrum.  It, in turn, is just above the low-frequency (LF) band used for "non-directional" navigational locators and beacons.  That terminology was frozen in the early decades of this century before the advent of very-high frequency, ultra-high frequency and other bands.

2.2.2 	HF's single greatest value is its ability to provide reliable long-range transmission and reception. HF routinely works over thousands of miles for two specific reasons: First, HF signals are reflected back to Earth by the ionosphere, an ionized layer 60 to 200 miles above the Earth; second, all modern HF transmitters use an SSB process that puts virtually all of the transmitter's power into the audio signal, providing about eight times the "talking" power of a basic AM transmitter.

2.2.3 	HF signals are propagated in two ways:

- The so-called "ground wave" travels downward from the antenna and is rapidly attenuated by the ground. Ground-wave signals from a typical HF transmitter may travel 100 miles or less.

- The "sky-wave" radiates at an upward angle from the antenna. If the frequency used is correct for the time of day, this sky wave will strike the ionosphere at an oblique angle that allows it to be reflected back to Earth. This reflection is what causes HF signals to travel thousands of miles.  Sky-wave propagation is, however, greatly dependent on the time of day and the frequency selected. In general, higher frequencies work better during daylight and lower frequencies better at night, but you will notice that HF radio operators use any of several different frequencies during a 24-hour period as they search for the best one under prevailing conditions.

- HF Limits

2.3.1 	HF theory and reality are often quite different, because this frequency band is particularly susceptible to disturbances in the ionosphere.  When those disturbances are serious, HF communications can be difficult at best.  Essentially, HF radio propagation is highly dependent on the multi-layered ionosphere for reflection.  And, since the ionosphere is formed primarily by the action of the sun's ultraviolet radiation, its effectiveness as a reflector changes in relation to the amount of sunlight passing though it.

2.3.2 	HF propagation is often called a black art even by the engineers who design the equipment, but the following general rules apply:

- Higher frequencies, those above 10.0 MHz, generally work better during the day.

- Lower frequencies, those below 10.0 MHz, generally work better when the sun is down.

- Any frequency may skip over the intended ground station when reflecting off the ionosphere. That so-called "quiet zone" under the skip contains no usable signal, so your options will be to try another station on the same frequency or to try the same station on another frequency.  Such "skips" are not uncommon.

- Solar flares and magnetic storms can severely impact all frequencies at all hours.  As they strike the ionosphere, the range of usable frequencies changes, and the quality of communications may be reduced, sometimes dramatically. At times, the entire band of frequencies may be marginally usable or even unusable, and especially so when you are flying in polar regions above 60 degrees north or below 60 degrees south.  Ironically, this very problem may scatter the radio energy in unpredictable ways that result in usable radio contacts at absurd distances and locations. When necessary, you can use those contacts to relay information.  HF operators are generally understanding and helpful in such conditions. In extreme conditions, they can call each other by landline.  You can use any available HF contact to relay important information, such as required position reports.

- HF Services

2.4.1 	HF communication is the standard for ATC en route services over approximately 75 percent of the Earth's surface.  HF can be used for personal phone calls over long distances, for flight following and for the receipt of pertinent weather forecasts and communications.

2.4.2 	HF Data Link is also available now through ARINC.  In the polar region it will operate primarily through the Barrow Station but it can also use other stations such as San Francisco.  It is equivalent to or better than voice and is a very good alternative for those aircraft that are equipped.

2.4.3 	ICAO, along with national aviation authorities, has assigned HF frequencies to various areas of the world for ATC purposes.  Across the North Atlantic, for instance, Gander Radio (Newfoundland), New York Radio, Iceland Radio, Shanwick Radio (Ireland) and Santa Maria Radio (Azores) provide HF ATC functions.  Communications with these and like facilities are limited to "flight safety" messages, those which involve ATC or weather communication.  When cleared through one of these areas, you will be assigned both a primary and a secondary frequency to assure radio contact even when one frequency fades or distorts.

2.4.4 	It is recommended to initiate a radio check on both frequencies and always insisting on having two usable frequencies at all times. If your airplane is equipped with selective calling (SELCAL), get a positive check on both frequencies whenever possible, but you must have a check of at least one unless you plan to monitor the frequency.

2.4.5 	In addition to its use for ATC, HF radio also offers several other services. In many areas of the world, you may have to communicate with the controlling ATC facility through a General Purpose (GP) radio facility.  GP stations are staffed by professional radio operators who can relay messages by voice from aircraft or telegraph to ATC, your company or both.  Conversely, GP stations can relay messages from ATC or the company to the pilot.

2.4.6 	Position reports given to GP radio facilities are automatically relayed to ATC.

2.4.7 	Long Distance Operational Control Facilities (LDOC) can provide "flight regularity" messages. Flight-regularity messages are defined as those that require flight-crews or aircraft operators to take immediate action in order to minimize travel interruptions and maximize aircraft utilization. You would use an LDOC station to relay pertinent flight information to and from designated company personnel. Flight-regularity messages must pertain to the operation of the aircraft itself, or to the aircraft load, requiring radio handling to ensure safety and efficiency.

2.4.8 	In the polar region Stockholm Radio is probably the best LDOC system to use when you have difficulty with other HF stations.  Stockholm has directional antennas that help when the Sun spot interference is a problem and at any other times.  When calling them you need to give your approximate latitude and longitude and then be patient.  They should call back in 3 to 5 minutes after adjusting their directional antennas.

2.4.9 	For aviation purposes, Coordinated Universal Time (UTC) is identical to Greenwich Mean Time (GMT), sometimes referred to as "Zulu" time.  At 18 minutes past each hour, both stations make geophysical alert announcements that forecast the quality of HF-radio propagation based on a so-called "K" index.  The K-index scale ranges from one to nine, signifying that the higher the K index, the poorer the HF propagation will be.  Another index reported on these hourly broadcasts is the "solar flux."  Solar flux values run from approximately 65 to 400, with higher numbers favoring HF propagation.  At 16 minutes past each hour, WWV and WWVH broadcast on 2.5, 5.0, 10.0 and 15.0 MHz.  At eight, nine and 10 minutes after each hour, weather information about major storms in the Atlantic and Pacific is broadcast.  The National Institute of Standards and Technology (NIST) publishes a guide to these broadcasts, and you should ask for NIST Special Publication 432, "Time and Frequency Services," from NIST Radio Station , 2000 E. County Rd. 58, Fort Collins, CO 80524 (USA). You may also find some interesting data in your Jeppesen books.

- HF Techniques

2.5.1	Modern HF airborne transceivers use digital tuning techniques so that assigned frequencies can be easily and quickly tuned.  Some HF radios have the capability to preset several frequencies that can be recalled with a two- or three-digit code, like autodialing a telephone.  It is the nature of HF radio to require different antenna characteristics for each frequency, so all HF transceivers include an "antenna coupler."  The coupler electronically and automatically optimizes the antenna for each tuned frequency and identifies its action with a tone in the sender's headset.  The tone is initiated when the transmit button is first pushed.

2.5.2 	Subsequent transmissions on the same frequency will not require additional tuning of the antenna. After tuning a new frequency on the HF, briefly click the transmit button and listen for the coupler tone. When the tone ceases in a few seconds, the antenna is tuned to that frequency, and the radio is ready for use.

2.5.3 	HF transmissions are normally more distorted than VHF or UHF.  There is often a warbling or static on the frequency (or both), and patience is a distinct virtue.  If the frequency is unusable, keep trying different frequencies until you find one that is usable.  You may have to try frequencies from different agencies or facilities, but persistence usually yields a usable station.  Any station can relay to others if you ask for that service.  In extreme cases, it may be possible to contact another airplane on VHF and ask them to relay for you.

2.5.4 	Aircraft operating on polar routes can contact any of the following facilities as well as San Francisco ARINC on their LDOC frequencies and through the Barrow, Alaska remote site:

- ARINC HF LDOC Facility

3013

3494

6637

6640

8921

8933

10033

10075

11342

13330

13333

13348

17940

17925

21964

- ARINC Pacific LDOC Remote Site Barrow, Alaska

3494 KHz

6637 KHz

6640 KHz

11342 KHz

13333 KHz

13348 KHz

17925 KHz

- General Purpose Radio

ARINC RADIO

SITA

ICELAND (ARCTIC) RADIO

EDMONTON

ANCHORAGE

  - GANDER RADIO

  - BODO RADIO

  - MAGADAN

  - MURMANSK

- Satellite Communications

INMARSAT SATCOM DATA & VOICE

IRIDIUM SATCOM VOICE

- LDOC

ARINC

STOCKHOLM RADIO

PORTISHEAD RADIO

CEDAR RAPIDS

- SATCOM Voice

3.1 	During the summer of 2009 a trial using SATCOM Voice for ATC will be conducted.  The participants will include both Inmarsat and Iridium as the SATCOM Voice Communication Providers.  Several operators with Satellite Voice capabilities will use these systems in the trial.  We know that Inmarsat will only work up to approximately 82 degrees north latitude.  Iridium on the other hand will be tested pole to pole as their system provides this coverage.

3.2 	SATCOM Voice will only be used with a third party radio operator the same as HF.  Procedures are now being developed for this operation, and if successful, SATCOM Voice may be authorized as a Long Range Communication System (LRCS), the same as HF.  In the meantime it should only be used as a backup to HF when communication is unsuccessful.

- Conclusion

4.1 	Modern HF radios are remarkably easy to operate, extremely powerful, reliable and provide a spectrum of services.  Good operator training and patience is required.

- Recommendation

The Meeting is invited to note the information provided in this paper.