Seventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/7)

(European and North Atlantic (EUR/NAT) Office of ICAO, Paris, France, 3-5 June 2009)

Agenda Item 4:  Provide Status on CPWG/6 Actions

- FLIGHT LEVEL 360 AT RAMEL

- (Action Item 030603, CP1-03)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information regarding the current status of the flight level test at fix RAMEL for aircraft crossing from Edmonton (CYEG) through Anchorage (ZAN) into Magadan (FANA) Flight Information Regions (FIRs).

- Introduction

1.1	Aircraft transiting the Anchorage (ZAN) Flight Information Region (FIR) are required to be at specific flight levels when transferred from Edmonton Area Control Centre (ACC) (ZEG) airspace to ZAN’s airspace and subsequently transferred to Magadan ACC’s airspace.  The letters of agreement require an altitude of either flight level (FL) 300 or 320 (treated as the same altitude for track loading), FL340, or FL380.  Due to the poor quality of communications in the polar area and the short duration of time the flight is in ZAN’s airspace, once an aircraft is transferred from ZEG to ZAN, it remains at that altitude until cleared by Russian controllers to meters.

1.2	Very few aircraft are able to attain an altitude of FL380 by the time ZEG is required to transfer the flight information to ZAN.  This results in the lower altitudes being routinely used while FL380 is generally unused.

1.3	During the Sixth Meeting of the Cross Polar Working Group held in Hong Kong, 3-5 November 2008, discussions began regarding the feasibility of conducting a trial to look at this situation.  It was agreed that ZAN would work with their colleagues in Russia and Canada to modify letters of agreement and on a trial basis, change the available altitude from FL380 to FL360 starting 30 March 2009 for a three month period.

- Discussion

2.1	The operational trial began on 30 March 2009.  During the first month of the trial, 16 percent of all flights over RAMEL were assigned FL360.  This was a much greater utilization of the third altitude than occurred at FL380.

2.2	Customers then requested that FL380 also be made available as an option for this transition. A second operational trial began 19 May 2009, which allows the use of FL360 or FL380 over RAMEL.  Once an aircraft transitions into Russian airspace, if assigned FL360 or FL380, it transitions to 11,600 meters.  This is similar to FL300 and FL320 which both transition to 9,600 meters.

2.3	The trackloaded altitudes for RAMEL remain at FL320, FL340, and FL360.  Aircraft can be assigned either FL360 or FL380 as long as they are longitudinally separated by 10 minutes continuous or increasing at RAMEL for assignment to the same altitude, 11,600 meters, in Russian airspace.  This same procedure already exists for FL300 or FL320 for assignment to 9,600 meters in Russian airspace.

- Conclusion

3.1  	The FAA wishes to acknowledge the cooperative attitude and the rapid response by FANA and NAV CANADA to the FAA’s request that this procedure be implemented and modifications be made to the respective letters of agreement.

- Recommendation

- The Meeting is invited to:

- note the information presented in this paper, and

- at the end of the trial, look at the viability of continuing utilization of FL360 or FL380 in the airspace.