Seventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/7)

(European and North Atlantic (EUR/NAT) Office of ICAO, Paris, France, 3-5 June 2009)

Agenda Item 4:  Provide Status on CPWG/6 Actions

CROSS POLAR WORKING GROUP (CPWG) PROPOSAL FOR AMENDMENT OF

- NAM REGIONAL SUPPLEMENTARY PROCEDURES (SUPPS) (Doc 7030)

(Action Item # CP6-13)

(Presented by the Federal Aviation Administration)

SUMMARY This working paper provides a proposed amendment to the North American (NAM) Regional Supplementary Procedures (Doc 7030). Appendix A is provided to support CPWG discussions and present a Proposal for Amendment of NAM Regional Supplementary Procedures (SUPPS) (Doc 7030).

- Introduction

1.1 	Due to the complexity of transfer of control and sometimes marginal reliability of High Frequency (HF) communications in the polar region, a standardized procedure is required for aircraft entry into Russian airspace.

1.2	Without a standardized loss of communications procedure in the polar region there is the potential of increased safety risk to flights based on individual expectations by flight crews and Air Navigation Service Providers (ANSP).

1.3	During the Third Meeting of the CPWG held in Washington on 24-26 April 2007, it was agreed that an action be taken to harmonize Polar Region standards.  The action was further discussed during the CPWG/6 meeting in Hong Kong, 3-5 November 2008, as part of the group’s Work Program.

- Discussion

2.1	The CPWG agreed to establish a standardized procedure for lost communications in the polar region for flights entering Russian airspace as well as an action to submit a proposal for the amendment to the NAM Regional Supplementary Procedures (Doc 7030).

2.2	 A proposed Amendment to the NAM Regional Supplementary Procedures (SUPPS) (Doc 7030) for aircraft communications failure is attached as Appendix A.

2.3	This paper supports the harmonization of procedures.

- Recommendation

3.1	The meeting is requested to:

- Note the information in this working paper;

- Support measures to reach the goal of establishing standardized loss of communication procedures; and,

- Review and come to agreement on the proposed language submitted in Appendix A to amend loss of communications procedures in Doc. 7030.

CROSS POLAR WORKING GROUP PROPOSAL FOR AMENDMENT OF

NAM REGIONAL SUPPLEMENTARY PROCEDURES (SUPPS) (Doc 7030)

(Serial no. NAT xxxx:)

a)	Proposed by:	The United States of America

b)	Proposed amendment:  Add Section 9.3 as shown below to the SUPPs in the NAM Region:

CHAPTER 9.  SPECIAL PROCEDURES

9.3  Air-Ground Communication Failure

(A2 – Chapter 3; P-ATM – Chapter 15; P-OPS, Vol. 1)

Note:  The following procedures are intended to provide general guidance for aircraft operating within, to or from the NAM region experiencing a communications failure. These procedures are intended to complement and not supersede Annex 2, the PANS-ATM and State procedures/regulations. It is not possible to provide guidance for all situations associated with a communications failure

9.3.1 	The following procedures apply to aircraft operating in the airspace of Anchorage Arctic FIR, Anchorage Continental FIR, and the Edmonton FIR CTA. These procedures are intended to complement and not supersede State procedures/regulations.

    - In the event of total loss of communication, an aircraft shall:

- try to re-establish communication by all other means;

- if all attempts to re-establish communication with ATC are unsuccessful:

    - squawk 7600;

    - if able, broadcast in the blind at suitable intervals: flight identification, flight level, aircraft position (including the ATS route designator or the track code) and intentions on the frequency in use, as well as on frequency 121.5 MHz (or as a back-up, the VHF inter-pilot air-to-air frequency 123.45 MHz);

    - attempt to contact either another aircraft or any ATC facility and inform it of the difficulty and request that information be relayed to the ATC facility with whom communications are intended.

    - watch for conflicting traffic both visually and by reference to airborne collision avoidance systems or traffic displays (if required);

    - turn on all aircraft exterior lights (commensurate with appropriate operating limitations);

    - maintain the last assigned speed and level, without regard for changes filed in the flight plan.

Note: In airspace where the strategic lateral offset procedures (SLOP) has been authorized, aircraft experiencing communication failure may also elect to initiate SLOP in accordance with Sate AIP, including an offset of 1.8 or 3.7 km (1 NM or 2 NM) right of track.

7) 	upon exiting oceanic airspace, conform to the relevant State procedures and

regulations.

9.3.3    In the event of lost communication, ATC shall maintain separation between the aircraft having the communication failure and other aircraft, based on the assumption that the aircraft having the communication failure will operate in accordance with procedures in 9.3.2.

c)	Intended date of implementation: 04 June 2010

d)	Proposal circulated to the following States and international organizations:

Algeria

Argentina

Angola

Australia

Austria

Bahamas

Bangladesh

Barbados

Belgium

Bolivia

Brazil

Brunei Darussalam

Bulgaria

Cambodia

Cameroon

Canada

Cape Verde

Central African Republic

Chile

China

Colombia

Congo

Cote d´Ivoire

Cuba

Czech Republic

Democratic People´s Republic of Korea

Democratic Republic of the

Congo

Denmark

Dominican Republic

Ecuador

Egypt

Ethiopia

Finland

France

Gabon

Gambia

Germany

Ghana

Greece

Guinea Bissau

Guyana

Haiti

Hungary

Iceland

India

Indonesia

Ireland

Israel

Italy

Jamaica

Japan

Jordan

Kazakhstan

Kenya

Lebanon

Luxembourg

Madagascar

Malaysia

Mexico

Monaco

Mongolia

Morocco

Mozambique

Namibia

Nepal

Netherlands

New Zealand

Niger

Nigeria

Norway

Pakistan

Panama

Paraguay

Peru

Philippines

Poland

Portugal

Republic of Korea

Romania

Russian Federation

Saudi Arabia

Senegal

Sierra Leone

Singapore

Somalia

South Africa

Spain

Suriname

Sweden

Switzerland

Tajikistan

Thailand

Trinidad and Tobago

Tunisia

Turkey

Uganda

Ukraine

United Arab Emirates

United Kingdom

United Republic of Tanzania

United States*

Uruguay

Uzbekistan

Venezuela

Viet Nam

ASECNA*

COCESNA*

EUROCONTROL*

IACA*

IAOPA*

IATA*

IBAC*

IFALPA*

IFATCA*