Seventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/7)

(European and North Atlantic (EUR/NAT) Office of ICAO, Paris, France, 3-5 June 2009)

Agenda Item 5:  2009-2010 Cross Polar Work Program

- OPTIONS FOR A FIX STRUCTURE IN THE ARCTIC REGION

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents a proposal to establish a fix structure to facilitate flex tracks in the Arctic Region.

- Introduction

- As reductions in separation standards progress in the Arctic Region, some structure will be required to gain the full benefits of the reduced separation.  This paper introduces some options for a fix structure to enable flex track generation.  Once the fix structure is established the States can determine how to flow traffic via the fixes.  The goal is for States to work concurrently to develop the project so that implementation can occur simultaneously.

- Discussion

- This paper is presented to generate discussion for a fix structure in the Arctic Region.  The first step is fix placement on the Russian/Anchorage Center common boundary.  Currently there are 6 fixes in the Arctic region (DEVID, RAMEL, NIKIN, ORVIT, PILUN and LISKI).  The seventh fix which lies just across Anchorage’s domestic Flight Information Region (FIR) is FRENK.

- There are some options for fix alignment in this airspace.  Since the Arctic RNP 10 project to provide 50 NM lateral separation to this airspace is in progress, the first option is to space fixes at least 50 NM apart.  The second option would be to place the fixes 60 NM apart in order that the airspace would be ready to reduce to 30 NM lateral by adding fixes between the fixes that were placed 60 NM apart.

- Option 1a:  Place fixes at least 50 NM apart between existing fixes.

- North Pole to DEVID is about 60 NM so 1 fix could be added.

- DEVID to RAMEL is about 270 NM so 4 fixes could be added.

- RAMEL to NIKIN is about 220 NM so 3 fixes could be added.

- NIKIN to ORVIT is about 110 NM so 1 fix could be added.

- ORVIT to PILUN is about 420 NM so 7 fixes could be added.

- PILUN to LISKI is about 96 NM so not enough room to add a fix unless the fixes are realigned.

- LISKI to FRENK is about 227 NM so 3 fixes could be added.

- There could be 19 additional fixes in addition to the 7 fixes all ready established for a total of 26 fixes.

- Option 1b:  Realign existing fixes to facilitate the most fixes 50 NM apart.  It is approximately 1400 NM between the North Pole and FRENK so 28 fixes could be established.

- Option 2a:  Place fixes at least 60 NM apart between existing fixes.

- North Pole to DEVID is about 60 NM so 1 fix could be added.

- DEVID to RAMEL is about 270 NM so 3 fixes could be added.

- RAMEL to NIKIN is about 220 NM so 2 fixes could be added.

- NIKIN to ORVIT is about 110 NM so 0 fixes could be added.

- ORVIT to PILUN is about 420 NM so 7 fixes could be added.

- PILUN to LISKI is about 96 NM so 0 fixes could be added.

- LISKI to FRENK is about 227 NM so 2 fixes could be added.

- There could be 15 additional fixes in addition to the 7 fixes all ready established for a total of 22 fixes.

- Option 2b:  Realign existing fixes to facilitate the most fixes 60 NM apart.  It is approximately 1400 NM between the North Pole and FRENK so 23 fixes could be established.

- The next step would be to determine how many additional fixes are needed on the Anchorage/Russian boundary and if additional fixes are needed on the Anchorage Center/Canadian common boundary, the Canadian/Iceland common boundary and the Iceland/Russian common boundary.

3.	Recommendation

- The meeting is invited to:

  - note the information of this paper and discuss/select an option to progress work; and

  - determine if this project should be added to the Arctic RNP 10 project.