Seventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/7)

(European and North Atlantic (EUR/NAT) Office of ICAO, Paris, France, 3-5 June 2009)

Agenda Item 4:  Provide Status on CPWG/6 Actions

- Crossing Restrictions for PILUN and LISKI

- (Action Item #CP6-15)

(Presented by the Federal Aviation Administration)

SUMMARY In response to CPWG/6 Action Item CP6-15, this paper presents information concerning flight restrictions for aircraft crossing the Canadian/Alaskan boundary on course for PILUN or LISKI.

- Introduction

  - Prior to the CPWG/6 meeting of 3-5 November 2008, aircraft overflying PILUN were required by Notice to Airmen (NOTAM) to (1) cross the Anchorage/Edmonton Flight Information Region (FIR) boundary over TAYTA (141W), and also flight plan over 72N157W, or to (2) cross the Anchorage/Edmonton FIR boundary over or north of point 7330N141W, and also flight plan over 7330N157W.  For LISKI, flights were required by NOTAM to cross the Anchorage/Edmonton FIR boundary over or south of position VOLOB (141W), and then flight plan over or south of 7030N157W.  This ensured procedural lateral separation outside of radar coverage and beyond very high frequency (VHF) pilot/controller frequency range.  During CPWG/6, the International Air Transport Association (IATA) requested Anchorage Air Route Traffic Control Center (ARTCC) consider deleting the 141W restrictions in favor of a 150W restriction in order to gain efficiencies.

- Discussion

  - On 18 November 2008, Anchorage ARTCC issued new NOTAMs which deleted the 141W restrictions.  Operators flight planning via PILUN were required to flight plan over or north of 72N157W.  Operators flight planning via LISKI were required to flight plan over or south of 7030N157W.  The 157W restriction was necessary to ensure aircraft have the necessary 90 nautical mile lateral separation required in the Anchorage Arctic FIR when they are beyond radar coverage west of Barrow.  Based on the operators’ description of how they conduct their flight planning, it was considered unlikely that PILUN and LISKI-bound traffic would cross prior to157W.

  - Aircraft transiting 141W at and north of VOLOB are subject to poor or no VHF communications.  Communication in this area is generally accomplished through Gander high frequency (HF) Radio.  In addition, there is a complete lack of radar surveillance at TAYTA and north.

  - Periods of heavy PILUN/LISKI traffic have created great difficulties in maintaining separation. Anchorage ARTCC air traffic controllers have seen user preferred route (UPR) flight paths crossing prior to 157W.  In addition, the ability to radar vector for spacing is limited due to the lack of radar and frequency coverage.  In order to meet the FAA’s primary function of “safe movement of traffic,” it was necessary to reestablish the restrictions at 141W and to maintain the restrictions at 157W.  On 02 May 2009, the following NOTAMs were issued:

A0291/09 - PILUN REFERENCE ATS ROUTE B970 AND RUSSIAN ENTRY/EXIT POINT PILUN:OPERATORS FLIGHT PLANNING VIA PILUN WESTBOUND MUST FLIGHT PLAN OVER OR NORTH OF TAYTA AND OVER OR NORTH OF 7200N15700W.  OPERATORS FLIGHT PLANNING VIA PILUN EASTBOUND MUST FLIGHT PLAN VIA PILUN THENCE OVER OR NORTH OF 7200N15700W THENCE VIA FLIGHT PLAN ROUTE.  ADDITIONALLY, OPERATORS MUST FILE A POINT ALONG 157W IN THE ROUTE OF FLIGHT.  REFERQUESTIONS TO ANCHORAGE INTERNATIONAL PROCEDURES AT 907-269-2730. FL260 - FL430,WIE UNTIL UFN.

A0292/09 - LISKI REFERENCE ATS ROUTE A218 AND RUSSIAN ENTRY/EXIT POINT LISKI:OPERATORS FLIGHT PLANNING VIA LISKI WESTBOUND MUST FLIGHT PLAN OVER OR SOUTH OF VOLOB AND OVER OR SOUTH OF 7030N15700W. OPERATORS FLIGHT PLANNING VIA LISKI EASTBOUND MUST FLIGHT PLAN VIA LISKI THENCE OVER OR SOUTH OF 7030N15700W THENCE VIA FLIGHT PLAN ROUTE. REFER QUESTIONS TO ANCHORAGE INTERNATIONAL PROCEDURES AT 907-269-2730. FL260 - FL430, WIE UNTIL UFN.

- Conclusion

- While the FAA realizes this is less than optimal for the customers, safety must be ensured.  We continue to explore other alternatives to increase efficiencies in the Arctic.

- Recommendation

- The Meeting is invited to note the information provided in this paper.