Seventh Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/7)

(European and North Atlantic (EUR/NAT) Office of ICAO, Paris, France, 3-5 June 2009)

Agenda Item 7: ANSP and Airline Updates

ATS Route Network Development in the Russian Airspace

(Presented by State ATM Corporation)

SUMMARY This information paper presents a progress report on the latest achievements in development of ATS route network and consolidation of the existing ATC system in the Russian Federation

1. Introduction

  - The Air Navigation System designed by the Russian Federation is aimed at ensuring the national interests of the State in the field of airspace utilization and air traffic services and based on the main provisions the ICAO Global Operational Concept and the decisions adopted by the ICAO Contracting States to support transition to the future CNS/ATM system.

2. General

  - The total area of the airspace where the State ATM Corporation of Russia provides air navigation services is 26 million square kilometers. It is serviced by 57 Area Control Centers (ACCs) and 18 auxiliary Area Control Centers (aux ACCs) supported by 7 Zonal Flow Management Centers and 1 Main ATFM Center. The system handles more than 1 million flights annually.

  - Effective from January 1, 2008 the Russian Air Navigation System became a “single fragment” by taking over the responsibility for the entire airspace from the military counterparts. From now on the civil Air Traffic Management Units are in charge of providing air traffic planning and air traffic services in both en-route and off-route airspace sectors.

  - The work is underway to establish 12 + 1 consolidated Area Control Centers by 2015. In 2008 – 2009 the State ATM Corporation phased out 17 smaller ACCs/ aux. ACCs and integrated their control areas as new sectors to larger ACCs as follows: Bodaybo ACC (UIKB) ->Irkutsk (UIII); Khatanga (UOHH) and Vanavara (UNIW) -> Krasnoyarsk (UNKL); Kolpashevo (UNLL) -> Novosibisk (UNNT); Sovetskaya Gavan (UHKM), Okhotsk (OHOO), Komsomolsk-on-Amur (UHKK), Yekimchan (UHBP) -> Khabarovsk (UHHH); Arkhara (UHBA) and Magdagachi (UHBI) -> Blagoveshchensk (UHBB); Severouralsk (USSE) ->Yekaterinburg (USSS), Beryozovo (USHB) -> Khanty-Mansiysk (USHH), Ukhta (UUYH), Pechora (UUYP) -> Syvkar (UUYY), Podkamennaya Tunguska (UNIP) - > Krasnoyarsk (UNKL), Nadym (USMM) -> Salekhard (USDD) and Tarko-Sale (USDS), Novokuznetsk (UNEE) -> Novosibirsk (UNNT). It’s expected to close down more ACCs in 2009.

Consolidation of ATC Facilities in 2000 – 2009

| ATC facility | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Main ATM Center | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Zonal ATM Center | 8 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 7 | 7 |
| Aux. ACC | 6 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | - | - |
| ACC (civil/military) | 38 | 42 | 41 | 40 | 40 | 32 | 29 | - | - | - |
| ACC (civil) | 31 | 26 | 25 | 25 | 24 | 30 | 33 | 60 | 40 | 39 |
| Aux. ACC | 45 | 44 | 44 | 44 | 40 | 36 | 38 | 32 | 22 | 18 |
| Aux ACC (without ATC functions) | 4 | 2 | 1 | 1 | 1 | 1 | 0 | 0 | - | - |
| ACC (with ATFM responsibilities) | - | - | - | - | - | - | - | - | 18 | 18 |
| Total: | 133 | 126 | 124 | 123 | 118 | 112 | 113 | 105 | 88 | 82 |

  - The existing route network covering 460 international airways has a total length of some 409,795 kilometers. In the first five months of 2009 the State ATM Corporation implemented 52 new international airways. All routes were implemented with the strict adherence to the ICAO requirements of two AIRAC cycles for any significant airspace changes.

Implemented ATS Routes

| Year | Number of Implemented routes |
| --- | --- |
| 2004. | 75 |
| 2005 | 19 |
| 2006 | 43 |
| 2007 | 57 |
| 2008 | 115 |
| 2009 (as of July 2, 2009) | 52 |

  - In 2008, State ATM Corporation handled 1 094 754 flights, an increase of 7,64 % over the last year. However the traffic analysis showed a decrease of traffic in late 2008. Thus in January 2008 the increase was + 16.67 %, while November and December traffic dropped to – 6.63 % and – 9.49 % respectively. This trend continued in 2009 (January – 11.95%, February – 15.71%, March – 13.36%, April – 13.64 %).   The decrease in traffic by the main ATS transit routes is as follows: Crosspolar + 1.5 %, Transpolar - 10.8 %, Transeast + 18.9 %, Transasian – 5.5 %, Asian – 5.4 %, Transsiberian - 3.5 %.

  - In the view of the global financial crisis, State ATM Corporation has intensified its efforts to establish new international ATS routes in order to provide more benefits such as shorter mileage, time and fuel savings for the international airlines. In 2009 – 2010 it’s planned to implement 159 new routes. Further details will be presented in a slide presentation.

3. Action by the Meeting

3.1 	The Meeting is invited to:

- note the above information and reflect it in the final report.