Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4:	Provide Status on CPWG/7 Actions

Reduced Vertical Separation Minimum (RVSM) Transitions

(Action Item #CP01-02)

(Presented by IATA)

SUMMARY This paper discusses the operational implications of Altitude transitions between Reduced Vertical Separation Minimum (RVSM) and Conventional Vertical Separation Minimum (CVSM) airspace and between differing Tables of Cruising.

1.	Introduction

# The issue of RVSM transitions must be carefully considered to ensure the transition arrangements do not negatively impact operational effectiveness in adjacent bodies of airspace. This is also necessary when different Tables of Cruising are in use across the Flight Information Regions (FIR) boundaries.

- Discussion

  - The introduction of RVSM has enabled a significant increase in efficiency simply by increasing the airspace capacity.

  - At CPWG/5, IATA introduced WP/15 (Appendix A) to discuss the applicability of International Law for RVSM transitions over the high seas. This principle is supported by the International Civil Aviation Organization (ICAO) and well established in legal practise although at this stage not always practised in reality.

  - The principles of altitude transitions however should not only be regulated by legal practise, they should also consider operational efficiencies and safety implications in adjacent airspace.

  - This is complicated further when differing Tables of Cruising are utilised. An aircraft that crosses an FIR boundary using a different Table of Cruising can effectively “block” two levels in the next body of airspace until the transition between tables is completed

  - The proximity of crossing tracks to an FIR boundary in particular needs to be considered when planning transition arrangements. When an aircraft crosses at a differing table of cruising, not only does it potentially block two levels on the crossing track but it also raises potential workload and flight safety issues for the adjacent sector.

  - If two aircraft were to cross the FIR boundary in relatively close proximity, while these aircraft would be separated, they could potentially be “blocking” 4 altitudes in the adjacent sector.

  - The FIR boundaries between the Anchorage and the Russian FIRs (Anadyr, Magadan and Petropavlovosk-Kamchatsky) represent situations of both “high seas and entry points with crossing tracks in relatively close proximity. Anchorage utilise ICAO Standard Imperial RVSM Table of Cruising whereas Russia utilises non-RVSM, non-ICAO metric.

  - The entry fixes of RAMEL, NIKIN, ORVIT and DEVID currently have transitions to metric within Russian airspace. The entry fixes of PILIN, LISKI, RUSOR, BESAT and BAMOK should also be considered high seas allowing transition to occur within Russian airspace.

  - The other entry fixes, ERNIK, VALDA, KUTAL and FRENK may not meet the high seas criteria. There is however a primary route (R338), relatively close to the FIR boundary that does not cross into Russian airspace.  This route also passes fixes RUSOR, BESAT and BAMOK.  Current procedures are such that transition must be completed within Anchorage FIR.

  - The net effect is that Anchorage must transition aircraft to the Russian Table of Cruising protecting against traffic on similar tracks as well as against aircraft on a crossing track that may not be entering Russian airspace.

  - On the Russian side of the boundary, the same conflicts do not occur with most tracks diverging or a convergence not occurring until at least 100nm past the boundary.

  - Effectively the efficiencies enabled by RVSM airspace are being directly compromised by the lack of agreement to transition between Tables in the less efficient airspace.

  - This in no way implies aircraft be accepted at less than the required separation standard. Anchorage will still have to transition aircraft to/ from non RVSM levels but they should not have to “block” extra levels simply because of a procedural requirement.

  - IATA contends that RVSM and altitude transition arrangements must:

- Ensure that the transitions are conducted in accordance with International Law and

- Where crossing tracks occur on one side of an FIR boundary only, transitions should be conducted on the other side of the FIR boundary

  - IATA proposes that a transition buffer zone be established on the Russian side of the FIR boundary where altitude transitions should normally be completed.

  - While communication reliability does represent a complication, it should not be used as an excuse to prevent the development of appropriate arrangements. If reliable communication is an issue, then consideration of delegation of airspace should be considered.

- Recommendation

  - The meeting is invited to:

  - Discuss the issues relevant to RVSM and Altitude transitions

  - Endorse the concept that RVSM/ altitude transitions should not (where possible) compromise the efficiencies in neighbouring airspace

  - Russia is invited to:

- Review RVSM/ Transition arrangements

- Establish procedures where possible to enable RVSM/ Altitude transitions to be completed within the Russian FIR (including necessary infrastructure support)