Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4:	Provide Status on CPWG/7 Actions

PROPOSED NEW ENTRY/EXIT FIXES ON THE RUSSIAN

FLIGHT INFORMATION REGION (FIR) BOUNDARY

(Action Item #07-02)

(Presented by the Federal Aviation Administration)

SUMMARY This working paper presents proposals for new Russian FIR boundary entry/exit fixes. The meeting is invited to consider and endorse these proposals.

1.	Introduction

# It was agreed at the Seventh Meeting of the CPWG (CPWG/7) that some structure will be required to gain the full benefits of reduced separation in the Arctic Region.  The FAA introduced options for a fix structure to enable flex track generation.  Once the fix structure is established, ANSPs can determine how to flow traffic using the fixes.  The goal is for States to work concurrently to develop the project so that implementation can occur simultaneously.

# One of the options considered was to place the fixes 60 NM apart in order that the airspace would be ready to reduce to 30 NM lateral by adding fixes between the fixes that were placed 60 NM apart.

# Based on the movement toward performance based navigation (PBN), it was agreed that this could be a later phase of the Arctic RNP 10 project.  A new action item, CP07-02, was developed to address this issue.

# This working paper presents proposals for discussion by the meeting.

- Discussion

- Four new fixes are proposed for the Russian/Anchorage FIR boundary.  Transfer of control from Anchorage Air Route Traffic Control Center (ARTCC) to the Russian Area Control Centers (ACC) would need to be conducted in feet due to limited communications.  The proposed fixes would be located at:

- 85 30 N which is north of current fix RAMEL;

- 83 30 N which is south of current fix RAMEL;

- 78 N which is south of current fix ORVIT; and

- 73 N which is north of current fix PILUN.

- New corresponding fixes are proposed on the Anchorage/Edmonton FIR boundary at 85 N, 84 N, 77 N and 74 N.  Additional studies will need to be done to verify that these fixes are adequately separated.

- Finally, the following fix realignments are proposed:

- NIKIN to 81N (80 49.6N);

- LISKI to 70N (70 24.3N);

- RESUM to 84N (84 14N); and

- OMEKA to 78N (78 10N).

- Recommendation

3.1	The meeting is invited to:

  - consider the information contained in this working paper,

  - endorse the proposed fixes and fix realignments, and

  - identify a small project team made up of the affected ANSPs to progress the work.