Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 6  Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) Issues

- REVIEW OF ERNIK WAYPOINT IN RUSSIAN TRANS EAST

(Presented by Continental Airlines for International Air Transport Association (IATA)

SUMMARY This working paper presents information for the Group’s consideration about Waypoint ERNIK in the Russian Trans East.

1.	Introduction

# Waypoint ERNIK has been opened for bi-directional use on the Russian/US Border in the Alaska area.

# PAZA has published NOTAM A0301/09:

A0301/09 NOTAMN

A) PAZA
B) 0905070901
C) UFN
E) QARCA EASTBOUND AND WESTBOUND VIA ERNIK B240.  OPERATORS FLIGHT 
PLANNING VIA ERNIK EASTBOUND MUST FILE VIA B240 TO ENM VOR THEN VIA 
USER PREFERRED ROUTE (UPR).  OPERATORS MUST COMPLY WITH EXISTING 
ANCHORAGE TRACK ADVISORY AND UPR REQUIREMENTS FOR WESTBOUND FLIGHTS. 
REFER QUESTIONS TO ANCHORAGE ARTCC INTERNATIONAL PROCEDURES OFFICE AT 907-269-1801.

- Discussion

- Thank you to the Russian Federation and to Anchorage for working together to open this highly beneficial route.  Unfortunately the trajectory per the published Notices to Airmen (NOTAM) removes much of the benefit for Aircraft Operators planning flights to the East Coast (New York area).

Preferred Trajectory

Actual trajectory per the NOTAM

Close up of the same area with airways removed, but radar superimposed.

Close up –as above but with the airways.

- The area consists of a complex network of airways and Radar/Non-Radar control as well as hand off from Russia to the US Controllers and Transitioning from Russian CVSM(metres) to US RVSM (feet).

- After analysis of average ACTUAL traffic flows at the times this fix is used; can we ask PAZA and the Russian Federation to work together to see if the NOTAM can be amended to ALLOW PLANNING from:-

|  | Distance saved | Fuel saved | Emissions Saved | Time Saved | Additional Payload ABLE to carry (pounds) |
| --- | --- | --- | --- | --- | --- |
| (NOTAM is ERNIK B240 ENM UPR to GOATS) | 0.00 | 0.00 | 00.00 | 0.00 | 0.00 |
| ERNIK DCT OME DCT GOATS | 58Nm | 1,600 | 4,960lbs | 7mins | 1,500 |
| ERNIK DCT IDROD DCT GOATS | 39Nm | 1,200 | 3,720lbs | 5mins | 900 |
| ERNIK DCT AVUBA DCT GOATS | 29Nm | 800 | 2,480lbs | 4mins | 600 |

- Recommendation

3.1	The meeting is invited to:

  - Review the information contained in this Working Paper;

  - Advise if a working solution is available from those provided or suggest another.