Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 6:	Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) issues

- PACIFIC ORGANIZED TRACK SYSTEM (PACOTS) STUDY ON

- FLEXIBLE TRACKS TO R220 AND R580

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information on an FAA study to allow PACOTS flexible tracks to merge onto R220 and R580 closer to or at the Fukuoka boundary.

- Introduction

1.1	When the northern westbound Pacific Organized Track System (PACOTS) tracks (i.e., Tracks C, D, E and F) merge onto R220 or R580, the merge is constrained to occur by NOLTI and OGGOE while the flight is in Anchorage radar airspace.

1.2	The FAA has already implemented some beneficial routings to allow the PACOTS track to join the North Pacific (NOPAC) southern tracks closer to the Fukuoka boundary.  Tracks can join R591 at the Fukuoka boundary and join G344 at CARTO

1.3	At the Twenty-Ninth Meeting of the Informal Pacific Air Traffic Control Coordination Group (IPACG/29), the FAA agreed to review the constraint that currently limits PACOTS tracks to merge onto NOPAC tracks R220 and R580 by NOLTI and OGGOE.  The FAA will study the possibility of allowing the PACOTS to merge onto R220 and R580 closer to or at the Fukuoka boundary.

- Discussion

2.1	Background

2.1.1	The study will generate study tracks to be evaluated for impact on airspace users and air traffic control (ATC).

2.1.2	The effort will be divided into two phases:  1) Generation of the PACOTS study tracks and estimation of potential reductions in fuel consumption, and 2) Assessment of the impact on ATC complexity, and estimation of ground delays and allowable step climbs.

2.1.3	Currently, PACOTS tracks either join the NOPAC tracks R220 and R580 under radar airspace or join R591 at AKISU (Fukuoka FIR boundary) or G344 at CARTO (east of the Fukuoka FIR boundary for automation reasons).  Past dates will be selected to show that the study tracks generated might be advantageous to merge the PACOTS with R220 and R580 closer to (or at) the Fukuoka FIR boundary.

2.1.4	The study tracks will be evaluated for any additional restrictions on the flights (e.g., altitude restrictions).

2.1.5	Fuel burn and time flown metrics will be calculated.

  - Phase I

2.2.1	Track Definition Messages for the PACOTS tracks C, D, E and F were reviewed for 369 days, from mid July 2008 until mid July 2009, to determine track placement.  Track D, from the North American west coast is optional. Track C, from the (northernmost) North American west coast, is always generated.  At their northern end, Tracks C and D merge onto R220 or  R580.  Tracks E and F are separated tracks that originate out KSFO and KLAX. If winds cause them to track into the NOPAC, they are merged with R591 or G344, never onto R220.

    - The jet stream is at its strongest in the winter, implying that westbound traffic is most likely to prefer northward routes in the winter.  Figure 1 shows the frequency with which traffic on the westbound PACOTS merged into the NOPAC.  (The traffic that did not merge into the NOPAC stayed south of the Anchorage-Oakland FIR boundary.)  It confirms the trend toward higher use in the winter, and lower use in the summer.

Figure 1.  Fraction of traffic merging from PACOTS to NOPAC

2.2.3	The Dynamic Ocean Tracking System Plus (DOTS+) network was modified to reduce the constraints on generating PACOTS Tracks C, D, E and F. Modifications to the DOTS+ network included developing nodes and creating new fixes as needed to allow for track generation.  This modification accommodated flexible (flex) tracks further west of the PACOTS into the NOPAC without restrictions, and allowed for smooth transition to R220 and R580.  Figure 2 shows the nodes from which DOTS+ chooses the time-and-fuel-optimum tracks.

Figure 2.  DOTS+ Nodes

Figure 3.  Sample PACOTS Track changes

2.2.4	One possibility for the PACOTS on a day of winds like those from January 13 is shown in Figure 2.  The original PACOTS routes are shown as thin, dashed lines.  The more fuel-efficient routes from DOTS Plus are shown in heavy, solid lines. The circles in Figure 1 describe areas of radar coverage, where merging onto the route is currently done.  Outside of radar coverage, where separation standards are much larger, changing routes is rare and highly constrained.  The west edge of Anchorage airspace is the boundary with Fukuoka Flight Information Region (FIR) in Japan.

2.2.5	For 25 days in January 2009, fuel efficient tracks and preferred altitudes were run on the original DOTS+.  Tracks were generated from CYVR, KSFO to RJAA.  For the baseline end-to-end track times and projected fuel burns at preferred altitudes were collected for each day, later to be compared to the new track times and altitudes for the same city pairs.

2.2.6	Two particular city pairs were chosen for detailed time and fuel calculations.  The first comparison was for track C from CYVR to RJAA, the second was Track E from KSFO to RJAA. Table 1 shows the total fuel consumed by a flight on this track over 25 days in January 2009.

| Route | Original Track | Modified Track | Reduction | Average per Day |
| --- | --- | --- | --- | --- |
| CYVR - RJAA | 6,043,763 | 6,026,985 | 0.57% | 671 |
| KSFO - RJAA | 6,522,328 | 6,478,309 | 0.68% | 1,760 |

Table 1.  Monthly Fuel Consumption and Savings (lbs.)

2.2.7	The day-to-day variation in benefits, due to winds that push more or less against the great-circle trajectory, is shown in Figure 3.  On days of light winds, the benefit is zero.  The average from Table 1 is shown by a horizontal mark. On the days when the winds blow strongest, the fuel benefits can be substantial:  almost a ton on the Vancouver route, and two and a half tons on the San Francisco route.

Figure 4.  Benefits Variation

  - Phase 1 Conclusions

2.3.1	Unrestricted PACOTS tracks that are permitted to flex further west would provide a substantial fuel savings for the aircraft using those tracks.  This calculation was done for January, the peak month.  Annual savings can be estimated from the average use of PACOTS/NOPAC transitions in Figure 2, above.  It follows that aircraft on Track C/D could project an annual fuel savings of 186,870 lbs, and those on Track E/F could project an annual fuel savings of 379,016 lbs.

2.3.2	These are substantial benefits if required separation can be maintained between flights executing such merges in a non-radar environment.  Establishing an estimate of the consequences of competing traffic is the objective of Phase 2.

  - Phase 2

2.4.1	The study is currently in Phase 2.

2.4.2	In Phase 2, the flights on the NOPAC and PACOTS tracks are simulated  using the Total Airport and Airspace Modeler (TAAM).

2.4.3	The objective of this simulation is to identify difficult air traffic situations that might result from allowing PACOTS tracks to flex north and merge further west into the NOPAC.  These could be merges, crosses, or requested altitudes or speeds.

2.4.4	The baseline for this phase will be filed trajectories from Ocean21.  For comparison, the alternative will be the DOTS+ tracks generated in Phase 1, with step climbs at the same distances from the departure airport as in the filed trajectories.

2.4.5	The simulation analysis will be done in an iterative fashion.  In both cases, with baseline and changed tracks, the flights will attempt to depart on schedule and fly their planned trajectories, including step climbs.  The TAAM simulation engine will enforce required separation as the flights pass through each simulated volume.

2.4.6	The output will be reviewed to find any maneuvers TAAM chose that are not supported by the current surveillance and communications infrastructure.  (Usually these will be vectors or holds.) If these are found, the step climb that caused the maneuver will be revised, in location or altitude, until a permissible trajectory is found.

2.4.7	The benefits and costs of each routing scheme will be assessed simultaneously.  Benefits will be computed in terms of flying time and fuel consumption for each flight.  Costs will be expressed in:

- conflicts, if each aircraft is permitted to fly according to its flight plan;

- denied altitudes, for those aircraft that request a step climb through other traffic in un-covered airspace; and

- delays, for flights that would be too close to other aircraft in the same flow.

2.4.8	These costs are not additive.  Conflicts will not exist between flights that are delayed or climb-restricted.

2.4.9	Results from Phase 2 are expected to be available in spring 2010.

- Recommendation

- The Meeting is invited to note the information.