Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, Georgia, 2-4 December 2009)

Agenda Item 4:  Provide Status on CPWG/7 Actions

PROPOSAL FOR AMENDMENT TO THE NORTH AMERICAN (NAM)

REGIONAL SUPPLEMENTARY PROCEDURES (DOC. 7030) -

REDUCED SEPARATION STANDARDS

(Action Item # CP03-30)

(Presented by Federal Aviation Administration)

SUMMARY This working paper provides a proposed amendment to the North American (NAM) Region Regional Supplementary Procedures (Doc 7030) to implement a reduction of the lateral separation standard within the Control Areas of Anchorage Arctic and Edmonton Flight Information Regions. The meeting is invited to endorse the proposed amendment.

- Introduction

  - In an effort to globally harmonize separation standards, the United States and Canada, in coordination with the Trans-Regional Airspace and Supporting ATM Systems Steering Group (TRASAS), are planning to implement a reduction of the lateral separation standard within the Control Areas (CTAs) of Anchorage Arctic Flight Information Region (FIR) and Edmonton FIR/CTA.  The lateral separation applied in the Anchorage Arctic FIR will be reduced from the existing 90 NM standard to 50 NM between aircraft authorized for required navigation performance (RNP) 10 or RNP 4 operations and will harmonize the application of 50 NM lateral separation across the Pacific CTA/FIRs, the Anchorage Arctic FIR and Edmonton FIR/CTA.

  - The planned implementation date is 18 November 2010.

- Discussion

  - This working paper provides a draft revision of the NAM Regional Supplementary Procedures (SUPPS) (Doc 7030) based on the update of the North Atlantic (NAT) and Caribbean (CAR) SUPPS for the application of 50 NM lateral separation in the West Atlantic Route System (WATRS) Plus airspace. (See Appendix A). The agreed upon language and proposed amendment will go through FAA coordination for submittal to ICAO Mexico.

- Recommendation

  - The meeting is invited to discuss and agree upon the language in the attached proposed amendment to the NAM Regional Supplementary Procedures (Doc 7030).

PROPOSAL FOR AMENDMENT TO THE

REGIONAL SUPPLEMENTARY PROCEDURES – DOC. 7030

NORTH AMERICAN (NAM) REGION

(Serial No. ______ – ______)

a)	Regional Supplementary Procedures:

NAM

b)	Proposed by:

United States in coordination with Canada and with the support of the Trans-Regional Airspace and Supporting ATM Systems Steering Group (TRASAS).

c)	Proposed amendment:

Editorial Note: Amendments are arranged to show deleted text using strikeout (text to be deleted), and added text with grey shading (text to be inserted).

1.  Amend the SUPPS applicable in the NAM Region as shown below.

On page NAM 2-1:

CHAPTER 2.   FLIGHT PLANS

2.1.2   Area navigation (RNAV) specifications

2.1.2.1	The letter R shall be inserted in Item 10 (Equipment) of the flight plan to indicate the aircraft meets the RNAV specification prescribed, has been appropriately approved and can comply with all conditions of that approval.  Additionally, the letter Z shall be inserted in Item 10 and NAV/RNP10 or NAV/RNP4, as appropriate, inserted in Item 18.

2.1.3   Required navigation performance (RNP) specifications

2.1.3.1	The letter R shall be inserted in Item 10 (Equipment) of the flight plan to indicate the aircraft meets the RNP specification prescribed, has been appropriately approved and can comply with all conditions of that approval. Additionally, the letter Z shall be inserted in Item 10 and NAV/RNP10 or NAV/RNP4, as appropriate, inserted in Item 18.

On page NAM 4-1:

CHAPTER 4.   NAVIGATION

4.1   PERFORMANCE-BASED NAVIGATION (PBN)

4.1.1   Area navigation (RNAV) specifications

4.1.1.1	RNAV 10 (RNP 10)

Note. – RNAV 10 retains the RNP 10 designation, as specified in the Performance-Based Navigation (PBN) Manual (Doc 9613), Volume I, 1.2.5.5.

Area of Applicability

4.1.1.1.1	A lateral separation minimum of 93 km (50 NM) may be applied between flights operating on oceanic routes or areas within the control area(s) of the Anchorage Arctic FIR/CTA and Edmonton FIR/CTA.

Note. – The NAT MNPS are set forth in NAT SUPPS, 4.1.1.5. NAT MNPS airspace is identified in NAT SUPPS, 4.1.1.5.1.

Means of Compliance

4.1.1.1.2	For application of 4.1.1.1.1, operators and civil aviation authorities must follow the provisions listed below.

4.1.1.1.3	The aircraft and operator must be approved for RNP 10 or RNP 4 by the State of the Operator or the State of Registry, as appropriate. RNP 10 is the minimum navigation specification for the application of 93 km (50 NM) lateral separation.

- 4.1.1.1.4	States shall ensure, when granting approval for RNP 10 or RNP 4, that operators establish programmes to mitigate the occurrence of large lateral track errors due to equipment malfunction or operational error.

Note. – The PBN Manual (Doc 9613) provides guidance on aircraft, operations and maintenance programmes for the initial achievement and continued compliance with the authorized navigation specification.

On page NAM 6-1:

CHAPTER 6.   AIR TRAFFIC SERVICES

6.2   SEPARATION

6.2.1   Lateral

(P-ATM – Chapters 5 and 15)

6.2.1.1	Minimum lateral separation in the Anchorage Arctic CTA shall be 167 km (90 NM) except the lower minima in 5.4.1.2 of the PANS-ATM may be applied, or further reduced in accordance with 5.11 of the PANS-ATM, when the conditions specified in the relevant PANS-ATM provisions are met.

Minimum lateral separation shall be:

a)	93 km (50 NM) between aircraft meeting the provisions in 4.1.1.1 except, minimum lateral separation between aircraft transitioning from Canadian MNPS airspace (CMNPS) to other MNPS airspace shall be 110 km (60 NM);

b)  110 km (60 NM) between aircraft in Canadian Domestic Airspace which are MNPS approved but not approved RNP 10 or RNP 4;

c)	167 km (90 NM) between aircraft in the Anchorage Arctic FIR/CTA not approved RNP 10 or RNP 4 or meeting MNPS;

Except that lower minima as detailed in 5.4.1.2 of the PANS-ATM may be applied, or further reduced in accordance with 5.11, where the conditions specified in the relevant PANS-ATM provisions are met (see 5.4).

On page NAM 7-1:

CHAPTER 7.   SAFETY MONITORING

7.2   AIRSPACE MONITORING

7.2.2   RNAV

7.2.2.1	RNAV 10 (RNP 10)

7.2.2.1.1	A target level of safety (TLS) of 5 x 10–9 fatal accidents per flight hour per dimension shall be established for route systems operating a 93 km (50 NM) lateral separation minimum. The safety level of such airspace shall be determined by an appropriate safety assessment.

Note. – Detailed guidance material on conducting safety assessments is contained in the Manual on Airspace Planning Methodology for the Determination of Separation Minima (Doc 9689) and the Safety Management Manual (Doc 9859).

7.2.2.1.2	Adequate monitoring of flight operations shall be conducted to provide data to assist in the assessment of the achieved lateral navigation performance of the aircraft population in relation to the lateral separation minimum. A safety assessment shall be carried out periodically, based on the data collected, to confirm that the safety level continues to be met. Data shall include operational errors due to all causes.

Note. – Monitoring will be conducted in accordance with the appropriate guidance material issued by ICAO. Detailed guidance is contained in the Manual on Airspace Planning Methodology for the Determination of Separation Minima (Doc 9689) and the Safety Management Manual (Doc 9859).

d)	Date when proposal received:

e)	Proposer’s reason for amendment:

1.	On 18 November 2010, the United States and Canada, in coordination with the Trans-Regional Airspace and Supporting ATM Systems Steering Group (TRASAS), plan to implement a reduction of lateral separation within the control area(s) of Anchorage Arctic FIR/CTA and the Edmonton FIR/CTA.

2.	93 km (50 NM) lateral separation will be applied between aircraft authorized Required Navigation Performance 10 (RNP 10) or RNP 4 by the State of Registry or State of Operator, as appropriate, operating on oceanic routes or areas.

Note. – Guidance and direction for RNP authorization is provided in ICAO Annex 6, Parts I and II, paragraph 7.2 (Navigation equipment). Criteria for the application of 93 km (50 NM) lateral separation between aircraft authorized RNP 10 or RNP 4 is currently published in Regional Supplementary Procedures for various regions including the CAR, NAT and PAC.

3.	Implementation of 93 km (50 NM) lateral separation in the Anchorage Arctic FIR/CTA and the Edmonton FIR/CTA will harmonize the application of 93 km (50 NM) lateral separation across the Pacific CTA/FIRs, the Anchorage Arctic FIR and the Edmonton FIR/CTA.

4.	Reduction of lateral separation from 167 km (90 NM) to 93 km (50 NM) in the Anchorage Arctic FIR/CTA and from 110 km (60 NM) to 93 km (50 NM) in the Edmonton FIR/CTA will increase the availability of routes and flight levels in that airspace and enable more aircraft to operate on time and fuel efficient routes and flight levels.  This, in turn, will reduce fuel burn and engine emissions. In addition, en route capacity and air traffic management (ATM) flexibility will be enhanced.

5.	Analysis of aircraft types operating in the oceanic areas affected by this initiative indicates that almost all of the flights conducted in the airspace are now flown by aircraft meeting RNP 10 or RNP 4 standards without modification.

6.	Aircraft that are not authorized for RNP 10 or RNP 4 will be allowed to continue to file any route at any flight level in areas listed above. They will be cleared to operate on their preferred routes and flight level as traffic permits; however, 93 km (50 NM) lateral separation will not be applied to them.

7.	93 km (50 NM) lateral separation has been applied between operators/aircraft authorized for RNP 10 or RNP 4 since 1998 in Pacific oceanic areas. It is also currently applied in the West Atlantic Route System (WATRS), in the European-South American Corridor; on routes between Santiago, Chile and Lima, Peru; on routes that pass south of the Himalayas connecting Australia, Asia, the Middle East, Europe and trans-Africa routes. Project planners will currently apply the experience gained in these other areas to implementation planning for the Arctic FIR’s.

8.	A Safety Risk Management (SRM) Panel composed of aviation experts from the United States and Canada convened to ensure that hazards are identified and unacceptable risk mitigated and accepted prior to any changes to the United States National Airspace System.  Applying the principles of the SRM process, the panel identified no high or medium risk hazards and only two low risk hazards. It was determined by the SRM Panel that the reduction of lateral separation in the Anchorage Arctic FIR can be safely implemented.

f)	Proposed implementation date of the amendment: 18 November 2010

g)	Proposal circulated to the following States and international organizations:

h)	Secretariat comments:

— END —