Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4:	Provide Status on CPWG/7 Actions

- PILUN AND LISKI CROSSING RESTRICTIONS

- (Action Item #CP06-14 and CP06-15)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information concerning options and restrictions for aircraft crossing the Canadian/Alaskan boundary on course for PILUN and LISKI.

- Introduction

  - The evolution of a set of options and restrictions can be a lengthy process.  This has definitely been the case for routes that transition from Edmonton Area Control Center (ACC) through Anchorage Air Route Traffic Control Center (ZAN) to Magadan ACC at fixes PILUN and LISKI. Over the past 18 months there have been three sets of NOTAMs that specified the requirements for passage from 141W to the two fixes on the MAG FIR.  None of these sets have proved satisfactory to both users and providers.

  - The Polar fixes of DEVID, RAMEL, NIKIN and ORVIT lie in an area of inconsistent communication and no surveillance.  This fact, and the relatively short time transiting aircraft are in the Anchorage Flight Information Region (FIR) make a feet-to-meters transition in Anchorage airspace unfeasible so it is completed in Russian airspace.  In contrast, the common fixes of FRENK, KUTAL, VALDA, and ERNIK are in the Anchorage Domestic FIR and are within areas of good frequency and surveillance coverage.  The aircraft are within Anchorage airspace for sufficient time to allow an easy feet-to-meters transition. Unfortunately PILUN and LISKI lie between these two groups of fixes and require ZAN to change the altitude to meters while lying in areas of marginal radar and radio coverage.

- Discussion

- Prior to the CPWG/6 meeting of 3-5 November 2008, aircraft overflying PILUN were required by Notice to Airmen (NOTAM) to (1) cross the Anchorage/Edmonton Flight Information Region (FIR) boundary over TAYTA (141W), and also flight plan over 72N157W, or to (2) cross the Anchorage/Edmonton FIR boundary over or north of point 7330N141W, and also flight plan over 7330N157W.  For LISKI, flights were required by NOTAM to cross the Anchorage/Edmonton FIR boundary over or south of position VOLOB (141W), and then flight plan over or south of 7030N157W.  This ensured procedural lateral separation outside of radar coverage and beyond very high frequency (VHF) pilot/controller frequency range.  During CPWG/6, the International Air Transport Association (IATA) requested that ZAN consider deleting the 141W restrictions in favor of a 150W restriction in order to gain efficiencies.

- In November 2008 it was decided that the 141W restriction would be rescinded and replaced with a “cross 157W at or north of 72N” for aircraft crossing PILUN.  The corresponding restriction for LISKI was ‘cross 157W at or south of 7030N.'  This ensured the aircraft would be 90 miles laterally separated and, it was thought, safety would not be compromised.  It was soon realized that aircraft don’t always fly as controllers expect and there were instances of aircraft crossing paths after 141W but prior to 157W.  In the interest of safety these restrictions were changed again.

- In May 2009 the NOTAMs were changed to read PILUN traffic must file ‘over or North of TAYTA’ and ‘over or north of 72N157W’ while those at LISKI must file ‘over or south of VOLOB’ and ‘over or south of 7030N157w.’  While this satisfied our safety concerns it did nothing for our Canadian friends.  The issue then became how to move these aircraft to their metric altitudes in areas of no VHF coverage.  It was believed that through CPDLC, ADS and HF we would be able to make adequate contact to get messages through.

- In practice this proved to be a flawed assumption.  Though aircraft transitioning to LISKI remained largely in an area of good very high frequency (VHF) communications and radar coverage, many of the aircraft routing to PILUN were problematic.  Very often aircraft proceeded past 141W without any communication with Anchorage ARTCC.  Failure of pilots to make the proper logon of controller pilot data link communication (CPDLC), lack of high frequency (HF) reception, and aircraft being out of range of their issued VHF frequency were some of the issues causing difficulties.  Controllers pointed out these problems.  One example from September 2009 illustrates this point. Four aircraft came across 141W in succession without a 141W progress report.  Two had failed to log onto CPDLC properly, and two had been issued a VHF frequency for which their northerly routing left them out of range.  Communications were eventually achieved with each aircraft and their transitions to meters accomplished, but it had become apparent progressive steps had to be taken.

- A conference call was arranged on 27 October 2009 to discuss what procedures needed to be put in place to ensure safety of aircraft at these “middle fixes” as planes moved from feet to meters.  Participants included Anchorage ARTCC controllers, National Air Traffic Controllers Association (NATCA) representatives, Anchorage ARTCC traffic management personnel, Anchorage ARTCC Airspace and Procedures staff, and representatives of the FAA Headquarters International Airspace and Procedures Office.  After a lengthy discussion, a consensus was reached and the following NOTAMs were strongly suggested.  While they reopen NCA32 and allow for greater variety in accessing LISKI, they do place a northern restriction for aircraft crossing 141W proceeding to PILUN.  This change in restrictions is seen as a good balance between meeting the needs of customers for efficiency, while ensuring the safety of aircraft in this region.

REFERENCE ATS ROUTE A218 AND RUSSIAN ENTRY/EXIT POINT LISKI: OPERATORS FLIGHT PLANNING VIA LISKI WESTBOUND MUST FLIGHT PLAN OVER TAYTA-NCA32-LISKI OR SOUTH .  OPERATORS FLIGHT PLANNING VIA LISKI EASTBOUND MUST FLIGHT PLAN VIA LISKI THENCE OVER OR SOUTH OF 7030N15700W THENCE VIA FLIGHT PLAN ROUTE.  REFER QUESTIONS TO ANCHORAGE INTERNATIONAL PROCEDURES AT (907) 269-1801.  FL260-FL430, WIE UFN.

REFERENCE ATS ROUTE B970 AND RUSSIAN ENTRY/EXIT POINT PILUN:  OPERATORS FLIGHT PLANNING VIA PILUN WESTBOUND MUST FLIGHT PLAN AT OR BETWEEN 7230N14100W AND TAYTA THENCE DIRECT PILUN. OPERATORS FLIGHT PLANNING VIA PILUN EASTBOUND MUST FLIGHT PLAN VIA PILUN THENCE OVER OR NORTH OF 7200N15700W THENCE VIA FLIGHT PLAN ROUTE.  ADDITIONALLY ALL OPERATORS MUST FILE  POINTS ALONG 157W AND 141W IN THE ROUTE OF FLIGHT.  REFER QUESTIONS TO ANCHORAGE INTERNATIONAL PROCEDURES AT (907) 269-1801.  FL   260-FL430, WIE UFN.

- Conclusion

- While the FAA realizes this is less than optimal for the operators, safety must be ensured.  The FAA continues to explore other alternatives to increase efficiencies in the Arctic and welcomes continued feedback and recommendations from the airlines and other air navigation service providers.

- Recommendation

  - The meeting is invited to:

  - note the information provided in this paper; and

  - suggest alternative solutions to the communication issues described.