Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4:	Provide Status on CPWG/7 Actions

- ROUTE RESTRICTIONS IN THE

- ANCHORAGE FLIGHT INFORMATION REGION (FIR)

- (Action Item #06-15 and 07-04)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents an overview of the existing restrictions within Anchorage FIR and provides justification for those which are needed, as well as changes that can be implemented.

- Introduction

- During the Seventh Meeting of the CPWG (CPWG/7), there were several discussions relating to various route restrictions within the Anchorage Flight Information Region (FIR).  It was agreed that Anchorage Air Route Traffic Control Center (ZAN) would review these restrictions with a view to eliminating or modifying where possible.

- This paper provides an overview of the existing restrictions and justification for those which are needed.

- Current restrictions are shown in normal font.  ZAN comments and possible changes are shown in italics.

- Discussion

- Restrictions in the High Area

- RUSOR – Eastbound 9100 meters (M) or 10100M, Westbound 9600M and 10600M. Westbound must join B241 no further west than ENEGU.  Eastbound must file B241 to ENM VOR.

ZAN is willing to revise ENEGU to ROCET.  Additional altitudes may be tactically approved but will not be track loaded.

- BESAT -- Eastbound 9100M or 10100M, Westbound 9600M and 10600M.  Eastbound must file G583 ENM.  Westbound must join G583 no further west than MUNRI.

Additional altitudes may be tactically approved but will not be track loaded.

- BAMOK* – Eastbound 10100M and Westbound 10600M. Must file B932 to MCG Eastbound. This restriction is necessary to avoid military airspace in the event of no communications.  Westbound must file PALIN G349 KIVAK B932 or LESAD B932.

Additional altitudes may be tactically approved but will not be track loaded.

- KOKES – Eastbound and Westbound is closed.

ZAN is willing to consider opening KOKES westbound at 8600M, 11600M and 13100M.  A time restriction may be needed.  These altitude restrictions also provide separation between BAMOK and KOKES traffic.

- B327 Closed between NULUK and BAMOK.

ZAN is considering leaving B327 solely in Russian airspace and renaming B327 between KATCH and NULUK in U.S. airspace.  There is already an airway between BAMOK and MORLY.  Internal discussions will continue.

- R338 – Flights utilizing R338 NATES R220, between 2200Z and 0300Z shall be able FL370 by 100nm southwest of OME.

This restriction cannot be changed due to non-radar crossing traffic going to RUSOR, BESAT, and BAMOK.

- East of KATCH – flights must remain in ZAN airspace not to enter Edmonton.

For aircraft northbound from Vancouver, this restriction is needed for automation reasons.

- Between KATCH and 152 30W – aircraft flying into Anchorage FIR from Oakland FIR must overfly KATCH, HMPTN, GRIZZ, MARLO, or ODK VORTAC.

This restriction is needed to provide Anchorage controllers with position estimates so non-radar separation can be established.  These restrictions can be revisited at such time as Ocean 21 is available in these sectors.

- KUTAL – Eastbound must file B233 to GAL VORTAC.

This restriction is required for ZAN to provide non-radar separation.  Aircraft may request direct upon radar identification.

- VALDA – Eastbound must file H222 to MCG VORTAC.

This restriction is required for ZAN to provide non-radar separation.  Aircraft may request direct upon radar identification.

- ERNIK – Eastbound must file B240 to ENM VOR.

This restriction is required for ZAN to provide non-radar separation.  Aircraft may request direct upon radar identification.

- Restrictions in the North Area

- PILUN – over or North of TAYTA and over or North of 72N157W.

ZAN is considering a new restriction to read Cross 141W North of TAYTA and south of 7230N due to communication limitations.  The restriction will be removed north of 72N157W.

- LISKI – over or South of VOLOB and over or South 7030N157W.

ZAN is changing the restriction to be “TAYTA-NCA32-LISKI or south”.  VOLOB will no longer be a factor.

- ORVIT – Must file 141W over or south of OMEKA.

This restriction is required for ZAN to provide non-radar separation.

- NIKIN -- Must file 141W over or North of COALL.

This restriction is required for ZAN to provide non-radar separation.

- Recommendation

- The Meeting is invited to note the information provided in this paper.

These restrictions were originally implemented with the new airway structure on May 6, 2009.  At that time it was predicted that the traffic would increase substantially.  At this point that increase has not occurred.  An analysis needs to be provided by IATA to indicate if the traffic is still planned to increase substantially or if the restrictions are curtailing the increase in traffic.  Traffic data indicates that these restrictions may only be required for about a five hour period, but history indicates a lack of ability of operators to adhere to the time restrictions, and the language barrier between adjacent air traffic control facilities limits the ability to negotiate options.

These restrictions were originally implemented with the new airway structure on May 6, 2009.  At that time it was predicted that the traffic would increase substantially.  At this point that increase has not occurred.  An analysis needs to be provided by IATA to indicate if the traffic is still planned to increase substantially or if the restrictions are curtailing the increase in traffic.  Traffic data indicates that these restrictions may only be required for about a five hour period, but history indicates a lack of ability of operators to adhere to the time restrictions, and the language barrier between adjacent air traffic control facilities limits the ability to negotiate options.