Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, Georgia, USA, 2-4 December 2009)

Agenda Item 3: Report on Relevant Outcomes from Other Pertinent Meetings

OPERATIONAL IMPROVEMENT IN HANDLING LARGE SCALE ROUTE CHANGES REQUIRED FOR EVENTS SUCH AS VOLCANIC ERUPTION

- (Action Item #CP02-27)

(Presented by the Japan Civil Aviation Bureau and the Federal Aviation Administration)

SUMMARY This paper presents a working paper submitted during the 31st Meeting of the Informal Pacific ATC Coordinating Group (IPACG/31) meeting regarding the volcanic eruption of Mt. Sarychev in Russia on 12 June 2009.

- Introduction

- During the 31st Meeting of the Informal Pacific ATC Coordinating Group (IPACG/31), the Japan Civil Aviation Bureau (JCAB) reported that due to the volcanic eruption of Mt. Sarychev in Russia on 12 June 2009, aircraft flying in North Pacific (NOPAC) airspace had to avoid the resulting volcanic ash area.

- JCAB stated that the volcanic activities lasted about 10 days, with volcanic ash blocking all five ATS routes between Anchorage and Japan in NOPAC airspace.  JCAB noted that several aircraft operators had limited information relating to this volcanic activity and thus some aircraft had to return to the departing airports due to insufficient fuel onboard.  JCAB also reported that some aircraft diverted south of the NOPAC routes, creating ATC operational issues.

- As a result, JCAB indicated its desire for the establishment of procedures to minimize the impact on ATC and aircraft operation in the event that large scale route changes are required for several days, such as occurred during the volcanic eruption. Appendix A contains the working paper presented by JCAB to IPACG/31.

- Discussion

2.1	JCAB mentioned three main challenges during the discussion at IPACG/31:  (1) operational deviations of aircraft into Russian airspace, (2) sharing of ATC information (latest forecasts and weather data), and (3) development of a process to respond to operator requests on lifting of the restrictions on Tracks H/I in the Pacific Organized Track System (PACOTS).

2.2	Based on discussions, the FAA and JCAB agreed on the following conclusions:

- To improve coordination, Fukuoka Air Traffic Management Center (ATMC) will begin to study the installation of a voice line with Russia to support emergency and operational deviations.

- In the near-term, Anchorage Air Route Traffic Control Center (ARTCC) agreed to assist with relying information from the Fukuoka ATMC to Russia via the US-Russia communication methods; and

- To coordinate with Russia bilaterally and discuss the need to update Letters of Agreement (LOAs) with Russia to include emergency and contingency procedures for aircraft to divert into Russian airspace.

- Recommendation

3.1	The meeting is invited to:

- Note the discussion by the JCAB on the flight deviations that resulted from the volcanic activity and ash;

- Discuss recommendations by the JCAB to mitigate the effect and their applicability within the Cross Polar and Russian Trans East Routes; and

- Discuss and implement procedures and process as needed in response to mitigate effects for the Cross Polar and Russian Trans East routes.

THE THIRTY-FIRST MEETING OF

THE INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/31)

(Tokyo, Japan, 5-9 Oct 2009)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

OPERATIONAL IMPROVEMENT IN CASE THAT LARGE SCALE ROUTE CHANGES ARE REQUIRED FOR SEVERAL DAYS SUCH AS VOLCANIC ERUPTION

(Presented by JCAB)

SUMMARY

With a volcanic eruption of Mt. Sarychev in Russia on June 12, 2009, undesirable impacts were on ATC/aircraft operations. Five aircraft made air turn back and many aircraft made in-flight route change, and ATC was very busy with complicated duty. This paper discusses development of some procedures to minimize the impact on ATC/aircraft operation in case that large scale route changes are required for several days such as volcanic eruption.

- Introduction

  - With a volcanic eruption of Mt. Sarychev in Russia on June 12, 2009, aircraft flying in NOPAC airspace had to avoid volcanic ash area. The volcanic activities extended for about 10days, and volcanic ash area blocked all 5 ATS routes between Anchorage and Japan in NOPAC airspace.

  - Some of aircraft operators had only limited information of volcanic activities, 5 aircraft made air turn back to divert airport in Japan because they were unable to change route for avoidance of volcanic ash area due to fuel on board.ATC operational issues also have raised while Mt. Sarychev volcanic activities.

  - Learning from the Sarychev volcanic eruption, JCAB propose to discuss development of some procedures to minimize the impact on ATC/aircraft operation in case that large scale route changes are required for several days such as volcanic eruption.

- A series of events with the Sarychev volcanic eruption at middle of June, 2009

  - The Sarychev Peak Volcano erupted on June 12, 2009 around 0600 UTC. Several flights bound for North America changed their routes or deviated around the area impacted by the volcanic ash on the day of eruption.

  - One of the above flights ultimately diverted its destination to the Kansai airport after requesting route change to Russian airspace from NOPAC.

  - Most of traffic usually using NOPAC flew Russian airways between June 14 and 17. PACOTS Tracks were generated around the impact area of the volcanic ash. The information of the impact area was acquired through Tokyo or Washington VAAC. (See attached diagram1.)

  - Three EVA flights flew a detour around the impact area of the volcanic ash and they were forced to divert their destination to RJCC due to fuel lack after the request of further detour by Oakland ARTCC according to a SIGMET of Aviation Weather Center on June 16.

  - One SHQ flight flew NOPAC without information of the volcanic ash and returned to the departure airport after an advisory from ATC to avoid the impact area of the volcanic ash on June 17.

  - UAL flew their seven flights with destination of Japan or Korea on Tracks H/I regardless of the restriction of the Tracks that the Tracks were available only to the flights with destination of VHHH/RCTP or south on June 19. UAL indicated that operators should have one appropriate point of contact of Air Traffic Service providers for such a large scale irregularity.

  - Seven flights of various companies changed their routes simultaneously on June 22. (See attached diagram2.)

  - Tokyo VAAC declared the dispersion of the volcanic ash on June 23.

- Deviation or route change into Russian airspace from the NOPAC

  - On June 12th, aircraft flying between ADNIP and ADGOR on R591 bound for Anchorage requested route change into Russian airspace. ATC responded that unable to approve route change into Russian airspace, and suggested route change towards east. The aircraft finally requested to make air turn back for divert to Kansai airport, maybe due to fuel

  - After the Korean aircraft accident on 1983, deviation towards Russian airspace from NOPAC area is strictly prohibited. And there is no ATC coordination line (communication circuit) and no coordination procedures between ATMC and Petropavlovsk-Kamchatsky/ Yuzhno-Sakhalinsk FIR.

  - In order to avoid similar events, it should be considered to establish ATC coordination line and coordination procedures between ATMC and Petropavlovsk-Kamchatsky/ Yuzhno-Sakhalinsk FIR.

  - And as provisional procedures until establishment of communication line, JCAB would like to discuss any possibility that Anchorage relays coordination messages between ATMC and Petropavlovsk-Kamchatsky　FIR.

- Information sharing not only ATCs but also aircraft operators

  - On June 16th, 3 aircraft (Taipei-Anchorage) were flying on detour route to avoid volcanic ash. The detour route was planned by aircraft operators. The Oakland ARTCC asked ATMC to change route of 3 aircraft towards further east when received ATC transfer information.

  - These 3 aircraft were unable to make route change due to fuel, and made air turn back to the New Chitose airport. On the same day, another aircraft (Shanghai-Anchorage) also made air turn back to the Shanghai airport.

  - This air turn back was caused by that aircraft operators were not aware of requested/recommended routes for avoid incidents before airborne. It is unavoidable if aircraft were airborne before requested/recommended routes were issued. But, the aircraft operators might flight plan of requested/recommended routes if they had noticed well time prior to airborne.

  - It should be discussed how to share critical information between not only ATC facilities but also aircraft operators on a timely basis in order to avoid similar event.

- Coordination scheme for remove or relax of PACOTS restriction

  - On June 19th, an airline operator asked ATMC to temporally delete of city pair restriction of PACOTS due to avoid volcanic ash area. The aircraft operator said he wondered how to coordinate, who is responsible. Actually the operator coordinated with ATMC, ATCSCC and Oakland ARTCC.

  - The Oakland ATRCC is responsible to establish westbound PACOTS, and ATMC is responsible to establish eastbound PACOTS. Restrictions for each PACOTS route are determined through the discussion at IPACG meeting.

  - Presently there is no scheme and no procedures of temporal delete/relax PACOTS restriction. In order to respond correctly and properly to the requests, it should be discussed to establish scheme or procedures to approve temporal delete/relaxation of the PACOTS restrictions.

- Other issues

  - Any other issues may be raised by aircraft operators learning from the Sarychev volcanic eruption. If the raised issue is suitable, it should be discussed at the IPACG meeting.

- Action by the meeting

  - The meeting is invited to:

- Discuss to improve ATC/aircraft operation in case that large scale route changes are required for several days such as volcanic eruption., and;

- Raise any issues learning from the Sarychev volcanic eruption.

- END -

Attachment

Diagram 1

Diagram 2