Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4:	Provide Status on CPWG/7 Actions

- STATUS OF THE RAMEL TRIAL

- (Action Item # CP01-03A)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents updated information regarding the flight level test at fix RAMEL for aircraft passing from Edmonton Flight Information Region (FIR) through Anchorage FIR into Magadan FIR.

- Introduction

1.1	Aircraft transiting the Anchorage Flight Information Region (FIR) are required to be at specific flight levels when transferred from Edmonton Area Control Center (CYEG) to Anchorage Air Route Traffic Control Center (ZAN) and subsequently transferred to Magadan Area Control Center (MAG).  In Spring 2009 the altitudes available at each of the polar fixes on the Anchorage/Magadan boundary were flight level (FL) 300 and FL320 (both transition to 9600 meters (M)), FL340 (for transition to 10,600M) and FL380 (for transition to FL11,600M.)

1.2	As the need for greater traffic capacity became apparent, the concept of allowing more altitudes to be track loaded was explored. Due to the inconsistent communication in parts of the Arctic Region, aircraft must remain at the same altitude for entry into MAG airspace as they are when transferred to ZAN leaving CYEG airspace.  Since most aircraft are unable to maintain FL380 at the point when they are transferred from CYEG to ZAN, the proposal of track loading FL360 instead of FL380 (for transition to 11,600M) was adopted, and went into effect 30 March 2009.  The increased usage of FL360 was noted, but customers then requested a second phase of the trial which would allow the use of FL360 or FL380 for this transition.  This trial began 19 May 2009.  At CPWG/7 users requested the use of FL370 eastbound which had been blocked by the use of FL360 and FL380 converging with 11, 600M.

- Discussion

2.1	The track loaded altitudes for RAMEL remain FL320 for aircraft transiting ZAN at FL300 and FL320 (both transition to 9600M), FL340 (for transition to 10,600M) and FL360 for aircraft at FL360 and FL380 (both transition to 11,600M).  Though FL340 remains the most used altitude, for the month of July over 20 percent of aircraft requested FL360 with very few flying at FL380.

  - With FL360 and FL380 track loaded as one altitude and MAG agreeing to accept these aircraft at International Civil Aviation Organization (ICAO) Flight Levels, the responsibility for transitioning aircraft to metric altitudes resided with them.  Air carrier representatives stated that FL370 eastbound was their optimum altitude for fuel economy.  In an effort to be responsive to their needs, MAG and ZAN agreed to allow FL370 eastbound provided ZAN was able to provide one hour advance notification of aircraft crossing RAMEL at FL360.  CYEG approved this use of FL370 in Canadian as well as the request to continue transfer of aircraft to ZAN one hour prior to the CYEG/ZAN FIR boundary.  On

1 August 2009 this third phase of the RAMEL trial was implemented with FL370 being available to eastbound users.

  - No significant logistical problems have resulted from the increased varieties of altitudes available to customers at RAMEL.  No significant safety issues have been noted due to this trial, which is still ongoing.

- Recommendation

- The Meeting is invited to note the information presented in this paper.