Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 6:	Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) issues

- REROUTE OF AIRCRAFT DUE TO COMMUNICATION FAILURE

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents information concerning events leading to a reroute of aircraft due to communication failure between Anchorage Air Route Traffic Control Center (ZAN) and Magadan Area Control Center (MAG) on 19 September 2009.

- Introduction

- On 19 September 2009 a total of 75 aircraft were track loaded to cross the Russian Far East and the Polar fixes.  This was a very heavy day of traffic with 22 flights over ERNIK, 9 flights over KUTAL and 16 crossing VALDA.  During the period of heaviest traffic, Anchorage Air Route Traffic Control Center (ZAN) was unable to coordinate transfer of control for several aircraft with Magadan Area Control Center (MAG), resulting in five aircraft being rerouted to North Pacific (NOPAC) Air Traffic Services (ATS) Route R220.

- Coordination between ZAN and the Russian ACCs is accomplished through the use of commercial phone lines as the primary method, aeronautical fixed telecommunication network (AFTN) teletype messages as the secondary alternative, and dedicated voice circuits as the tertiary method.  The dedicated voice circuits have not been formally approved for interfacility communications.

- Discussion

- The Letter of Agreement between ZAN and MAG requires transfer of control information to be sent to the receiving facility a minimum of 30 minutes prior to the aircraft crossing the common flight information region (FIR) boundary.

- The procedure to follow if coordination has not been accomplished as outlined in ZAN internal directives is that aircraft will be rerouted to avoid entering another facility’s FIR.  The previous procedure for aircraft to be advised that coordination had not been completed and provide the aircraft with the option to accept a reroute or to proceed on course at his/her own risk and announce the flight on frequency is no longer available for safety of flight reasons.

- The last communication with MAG on 19 September 2009 prior to the interruption was at 2320 UTC.  At 2347 UTC ZAN began rerouting aircraft south to R220, three aircraft from ERNIK and two from KUTAL.

- The next communication with MAG was to the controller working the radar position at 2356 UTC when MAG called ZAN regarding an aircraft at an incorrectly coordinated altitude. The next contact was by commercial phone at 0019 UTC.

- When contact from the ZAN sector to MAG was unsuccessful the controllers sent transfer messages via AFTN.  The responses, which are routed to the ZAN Flight Data position, were not received until after the reroutes were in effect.  Controllers also attempted contact on the new dedicated ground to ground voice circuits, but there was no response from MAG. The Supervisor for the area tried three different phone numbers for MAG and was unable to reach the appropriate facility (two numbers said they were not MAG and the third spoke only Russian.)  The Traffic Management Unit attempted to call a contact in Moscow but due to the time difference was unable to reach anyone.

- Recommendation

- The Meeting is invited to note the information provided in this paper.