Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009,)

Agenda Item 4:  Provide Status on CPWG/6 Actions

- HIGH FREQUENCY DATA LINK

- (Action Items CP03-29 and CP06-06)

(Presented by the Federal Aviation Administration)

SUMMARY This paper provides updates on Anchorage Air Route Traffic Control Center (ARTCC)’s observation of high frequency data link (HFDL) possibilities and an ongoing HFDL trial in the Oakland Flight Information Region (FIR).

- Introduction

  - Communication between aircraft conducting polar operations and Anchorage Air Route Traffic Control Center (ARTCC) are frequently inadequate.  This has resulted in incomplete and erroneous information transfer between air traffic service (ATS) providers and between ATS providers and airlines.  This also creates inefficiencies and potential safety concerns when controllers are unable to respond to requests or issue control instructions.

  - In June 2008 Oakland ARTCC, Hawaiian Airlines, and ARINC commenced an operational trial of High Frequency (HF) Data Link (HFDL).  The data gathered during the trial was evaluated against the criteria in RTCA DO-306, SAFETY AND PERFORMANCE STANDARD FOR AIR TRAFFIC DATA LINK SERVICES IN OCEANIC AND REMOTE AIRSPACE, to determine the suitability of the HF media for air traffic control (ATC) data link communications.

- Anchorage Communications Problems

- Aircraft north of 72 degrees north latitude have little chance of communicating with Anchorage ARTCC via very high frequency (VHF) radio.  Aircraft using HF must contact Gander Radio and request them to relay requests, position reports or other information.  Delays in receiving information sometimes force controllers to deny pilot requests.  In some cases, the uncertainty whether aircraft have received and complied with instructions affects the accuracy of coordinated information between ATS providers.

- Controller pilot data link communication (CPDLC) is unreliable north of 80 degrees north latitude based on the positioning of the INMARSAT geosynchronous satellites.  An unsuccessful transmission by an aircraft based on coverage is not identified to the pilot so communication is broken but not recognized.

- Anchorage ARTCC is encouraged by the HF testing currently being conducted by Oakland ARTCC and Hawaiian Airlines.  If data link can effectively use HF for exchanging information, then efficiencies may be gained in polar operations.

- Status of the Oakland ARTCC HFDL Trial

  - Hawaiian Airlines (HAL) operates a fleet of B767 that are equipped with FANS but not INMARSAT satellite communication (SATCOM).  This configuration (VHF and HF only) presented an opportunity to evaluate HF media message transit delay times that were not “burdened” with initial attempts via SATCOM.

  - The trial incorporates a three-phase approach in the implementation of HFDL operations. The phased approach was taken to allow for baseline measurements, HF data radio upgrade, early data collection, and aircrew training.  While the design of the trial defined three distinct operating environments, each of the phases was not exclusive of previous phases (e.g. Phase 2 operations could continue while Phase 3 operations were also being conducted).

  - Beginning on 1 April 2009, HAL crews using FANS began terminating the automatic dependent surveillance (ADS) contracts after initial logon and operating CPDLC only using the CPDLC position report rather than ADS to meet reporting requirements.  This change to the operating environment of the trial was made for two reasons.  The first was to generate more CPDLC downlinks so that Oakland ARTCC could have greater confidence in the earlier results.  The second reason was to evaluate what may very well be the initial post-trial HFDL operating environment: CPDLC only.

  - Overall performance since December 2008 when the last aircraft received upgraded radios indicates that HFDL CPDLC is meeting the requirements for RCP400.  However, ADS is not meeting the requirements in RTCA DO-306 for position report delivery times associated with reduced time-based or distance-based separations.

  - The following table shows a sample of 161 HF CPDLC downlinks and their one way transit times.  The asterisked time approximates the delay standard in RCP-400D (370-60/2).

| Total | 161 |  |
| --- | --- | --- |
| <= 60 seconds | 116 | 72.05% |
| <= 120 seconds | 140 | 86.96% |
| <= 155 seconds* | 146 | 90.68% |
| <= 180 seconds | 149 | 92.55% |
| > 180 seconds | 12 | 7.45% |

  - The following table shows a sample of 73 HF CPDLC uplinks.  The uplink transit times include the uplink time and the downlink time of the MAS message.  The asterisked time is the delay standard for RCP-400D.

| Total | 73 |  |
| --- | --- | --- |
| <= 120 seconds | 55 | 75.34% |
| <= 240 seconds | 70 | 95.89% |
| <= 310 seconds* | 72 | 98.63% |
| <= 360 seconds | 72 | 98.63% |
| > 360 seconds | 1 | 1.37% |

  - The use of HF CPDLC messages in the trial is working well.  The FAA will inform the CPWG of future progress and provide a detailed report of the final results of the trial.

- Recommendation

  - The Meeting is invited to note the information provided in this paper.