Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4: Provide Status on CPWG/7 Actions

Report on the Status of the Arctic 50 NM

Lateral Separation Reduction Project

- (Action Item #CP03-30)

(Presented by the Federal Aviation Administration)

SUMMARY This paper presents a status of the Arctic RNP 10 project to implement 50 NM lateral separation between operators/aircraft authorized Required Navigation Performance (RNP) 10 or RNP 4 in the Anchorage Arctic Flight Information Region (FIR).

1.	Introduction

  - 1.1     As part of the discussions and updates from the TRASAS/2 meeting, the Fifth Meeting of the CPWG (CPWG/5) agreed to include the TRASAS 2/2 conclusion as part of its new CPWG Work Program. The Cross Polar Working Group (CPWG) identified a need to harmonize separation standards and improve efficiencies across the polar region.

- 1.2	The FAA, NAV CANADA and ISAVIA reported to the Seventh Meeting of the CPWG (CPWG/7) that they had formed a team to explore implementing 50 NM lateral separation between operators and aircraft authorized RNP 10 or RNP 4 in Anchorage Arctic FIR and Canadian Minimum Navigation Performance Specifications (CMNPS) airspace.

- 1.3	This information paper presents an update on the work conducted by the team.

- Discussion

  - The United States and Canada in coordination with the Trans-Regional Airspace and Supporting ATM Systems Steering (TRASAS) group are planning to implement a reduction of the lateral separation standard within the Control Areas (CTAs) of Anchorage Arctic FIR and Edmonton FIR/CTA. The lateral separation applied in the Anchorage Arctic FIR will be reduced from the existing 90 NM standard to 50 NM between aircraft authorized for RNP 10 or RNP 4 operations and will harmonize the application of 50 NM lateral separation across the Pacific CTA/FIRs, the Anchorage Arctic FIR and Edmonton FIR/CTA.

  - A Safety Risk Management (SRM) Panel composed of representatives from Anchorage Air Route Traffic Control Center, Edmonton Area Control Centre (ACC), NAV CANADA, FAA Western Service Center, FAA Flight Standards, and FAA Oceanic and Offshore Operations (AJE-32), and facilitated by CSSI Inc., was convened to ensure that hazards are identified and unacceptable risk is mitigated and accepted prior to any changes to the United States (US) National Airspace System (NAS). Applying the principles of the SRM process, the panel identified no high or medium risk hazards and only two low risk hazards. It was determined by the SRM Panel that the reduction of lateral separation in the Anchorage Arctic FIR can be safely implemented.

  - The FAA’s next step is to complete the SRMD once we receive the collision risk model from the William J Hughes Technical Center. NAV CANADA’s next step is to submit to their Regulator (Transport Canada) a project Concept of Operations and a Local Hazard Identification and Risk Assessment (HIRA) for approval.

  - The planned implementation date is 18 November 2010.

- Recommendation

3.1        The meeting is invited to note the information provided in this paper.