Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009)

Agenda Item 4:  Provide Status on CPWG/6 Actions

PROPOSAL FOR AMENDMENT OF

- NAM REGIONAL SUPPLEMENTARY PROCEDURES (Doc 7030)

- LOSS OF COMMUNICATIONS

(Action Item # CP06-13)

(Presented by the Federal Aviation Administration)

SUMMARY This working paper provides information on comments made to the proposal for the amendment to the ICAO North American (NAM) Regional Supplementary Procedures (Doc 7030) regarding loss of communications. The meeting is requested to consider the issues raised and come to agreement on modified language for the proposed amendment prior to submitting it to ICAO.

- Introduction

  - During the Third Meeting of the CPWG (CPWG/3) held in Washington on 24-26 April 2007, it was agreed that action be taken to harmonize Polar region standards.  The action was further discussed during the CPWG/6 meeting in Hong Kong, 3-5 November 2008, as part of the group’s work program.

  - CPWG/7 agreed to establish a standardized procedure for lost communications in the Polar region for flights entering Russian airspace as well as to submit a proposal for the amendment to the ICAO North American (NAM) Regional Supplementary Procedures (Doc 7030).

  - A proposed amendment to the NAM Regional Supplementary Procedures for aircraft communications failure was presented to the group and minor changes to the wording were suggested during the discussions.  The proposed amendment is at Appendix A.

- Discussion

- Following the meeting, the FAA conducted further internal coordination of the proposed amendment.  The following concerns were raised during that coordination:

- Paragraph 9.3.2, subparagraph 6) states the aircraft shall “… maintain the last assigned speed and level, without regard for changes filed in the flight plan until exiting the airspace defined…”  The reviewer suggested that typical flights going over polar routes are widebody aircraft that step climb throughout the flight.  The aircraft start low and climb every few hours.  If an aircraft is assigned a speed at a lower altitude and loses communication, this procedure would require the aircraft to maintain that specific speed and altitude which may compromise the flight due to excessive fuel burn.  The recommendation was that the speed should be left to the discretion of the pilot so that he may not compromise a flight due to fuel issues.

- These procedures may be creating an increase in situations where an aircraft will have to conduct a contingency procedure in accordance with ICAO PANS-ATM (Doc 4444), i.e., offset 15 NM and climb/descend 500 ft.

- These issues must be addressed prior to submitting the proposed amendment to ICAO for review and processing.

- Recommendation

- The meeting is requested to:

- Note the information in this working paper;

- Consider the issues raised during the FAA review; and

- Come to agreement on modified language for Appendix A prior to submitting this proposed amendment to ICAO.

CROSS POLAR WORKING GROUP PROPOSAL FOR AMENDMENT OF

NAM REGIONAL SUPPLEMENTARY PROCEDURES (SUPPS) (Doc 7030)

(Serial no. NAT xxxx:)

a)	Proposed by:	The United States of America

b)	Proposed amendment:  Add Section 9.3 as shown below to the SUPPs in the NAM Region:

CHAPTER 9.  SPECIAL PROCEDURES

9.3  Air-Ground Communication Failure

(A2 – Chapter 3; P-ATM – Chapter 15; P-OPS, Vol. 1)

Note:  The following procedures are intended to provide general guidance for aircraft operating within, to or from the NAM region experiencing a communications failure. These procedures are intended to complement and not supersede Annex 2, the PANS-ATM and State procedures/regulations. It is not possible to provide guidance for all situations associated with a communications failure

9.3.1 	The following procedures apply to aircraft operating in the airspace of Anchorage Arctic FIR, Anchorage Continental FIR, and the Edmonton FIR CTA. These procedures are intended to complement and not supersede State procedures/regulations.

    - In the event of total loss of communication, an aircraft shall:

- try to re-establish communication by all other means;

- if all attempts to re-establish communication with ATC are unsuccessful:

    - squawk 7600;

    - if able, broadcast in the blind at suitable intervals: flight identification, flight level, aircraft position (including the ATS route designator or the track code) and intentions on the frequency in use, as well as on frequency 121.5 MHz (or as a back-up, the VHF inter-pilot air-to-air frequency 123.45 MHz);

    - attempt to contact either another aircraft or any ATC facility and inform it of the difficulty and request that information be relayed to the ATC facility with whom communications are intended.

    - watch for conflicting traffic both visually and by reference to airborne collision avoidance systems or traffic displays (if required);

    - turn on all aircraft exterior lights (commensurate with appropriate operating limitations);

    - maintain the last assigned speed and level, without regard for changes filed in the flight plan until exiting the airspace defined in paragraph 9.3.1 above;

    - upon exiting the airspace as defined in paragraph 9.3.1 above, conform to the relevant State procedures and regulations.

9.3.3    In the event of lost communication, ATC shall maintain separation between the aircraft having the communication failure and other aircraft, based on the assumption that the aircraft having the communication failure will operate in accordance with procedures in 9.3.2.

c)	Intended date of implementation: 04 June 2010

d)	Proposal circulated to the following States and international organizations:

Algeria

Argentina

Angola

Australia

Austria

Bahamas

Bangladesh

Barbados

Belgium

Bolivia

Brazil

Brunei Darussalam

Bulgaria

Cambodia

Cameroon

Canada

Cape Verde

Central African Republic

Chile

China

Colombia

Congo

Cote d´Ivoire

Cuba

Czech Republic

Democratic People´s Republic of Korea

Democratic Republic of the

Congo

Denmark

Dominican Republic

Ecuador

Egypt

Ethiopia

Finland

France

Gabon

Gambia

Germany

Ghana

Greece

Guinea Bissau

Guyana

Haiti

Hungary

Iceland

India

Indonesia

Ireland

Israel

Italy

Jamaica

Japan

Jordan

Kazakhstan

Kenya

Lebanon

Luxembourg

Madagascar

Malaysia

Mexico

Monaco

Mongolia

Morocco

Mozambique

Namibia

Nepal

Netherlands

New Zealand

Niger

Nigeria

Norway

Pakistan

Panama

Paraguay

Peru

Philippines

Poland

Portugal

Republic of Korea

Romania

Russian Federation

Saudi Arabia

Senegal

Sierra Leone

Singapore

Somalia

South Africa

Spain

Suriname

Sweden

Switzerland

Tajikistan

Thailand

Trinidad and Tobago

Tunisia

Turkey

Uganda

Ukraine

United Arab Emirates

United Kingdom

United Republic of Tanzania

United States*

Uruguay

Uzbekistan

Venezuela

Viet Nam

ASECNA*

COCESNA*

EUROCONTROL*

IACA*

IAOPA*

IATA*

IBAC*

IFALPA*

IFATCA*