Eighth Meeting of the Cross Polar Trans East Air Traffic Management Providers’ Work Group (CPWG/8)

(Atlanta, USA, 2-4 December 2009

Agenda Item 6  Communications, Navigation, Surveillance (CNS) and Air Traffic Management (ATM) Issues

- REVIEW OF TRAFFIC FLOWS OVER WAYPOINTS ORVIT AND NIKIN

(Presented by Continental Airlines for International Air Transport Association (IATA)

SUMMARY This working paper presents information for the Group’s consideration about forced separation and traffic flows over Waypoints ORVIT and NIKIN in the Polar Area.

1.	Introduction

- Anchorage Air Route Traffic Control Center (ZAN) NOTAMS A0307/07 and A0304/07(below) refer to “no-fly zone” from ORVIT to NIKIN to COALL to OMEKA.

A0307/07 NOTAMR A0305/07

A) PAZA B) WIE C) UFN

E) QXXXX REFERENCE: ORVIT AND ATS ROUTE G494. OPERATORS FLIGHT

PLANNING ORVIT (7900.0N 16858.4W), REGARDLESS OF DIRECTION OF FLIGHT,

MUST ALSO FLIGHT PLAN A POINT ALONG 141 WEST LONGITUDE OVER OR SOUTH

OF OMEKA (7810.6N 14100.0W). DIRECT QUESTIONS TO ANCHORAGE ARTCC

INTL PROCEDURES AT 907-269-1801. CREATED: 06 Jun 2007 17:26:00

A0304/07 NOTAMR A0291/07 A) PAZA B) 0706070000 C) UFN

E) REFERENCE: NIKIN AND ATS ROUTE G226. OPERATORS FLIGHT PLANNING NIKIN (8049.6N 16858.4W), REGARDLESS OF DIRECTION OF FLIGHT, MUST

ALSO FLIGHT PLAN A POINT ALONG 141 WEST LONGITUDE OVER OR NORTH OF

COALL (8000.0N 14100.0W). DIRECT QUESTIONS TO ANCHORAGE ARTCC INTL

PROCEDURES AT 907-269- 1801. CREATED: 06 Jun 2007 15:56:00

- Discussion

2.1	ZAN has repeatedly told the group that by their rules, they are required to have 90NM lateral separation between traffic in this region.  Waypoints NIKIN to ORVIT and COALL to OMEKA are both approximately 110NM apart.

2.2	However with such a broad swath NOTAM like this, even with one aircraft  (NO TRAFFIC), that one lone operator is forced to fly a less than optimal wind route between almost any city pairs.

2.3	The Great Circle Routes between Chicago and New York are of course very different.  The present FIXED NO-FLY ZONE favours the Midwest over the busy East coast cities such as EWR, IAD, PHL, JFK.

2.4	Please refer to figures 1, 2, and 3 for graphical representation of inefficiencies based on possible traffic versus actual.

2.5	The following questions are posed for consideration and discussion:

- Can ZAN work on a more equitable location of the fixes if they need to remain on their Eastern FIR boundary?

- Can ZAN narrow the parallelogram (points COALL and OMEKA) to better reflect their required lateral separation?   Unless the narrowing longitudinal lines and northern latitudes prevent this?

- Can ZAN work with the Russian Federation/airlines to better anticipate the ACTUAL TRAFFIC flow each day before instituting such harsh measures?

- Are there any other short term measures the Air Traffic Control experts can think of to lessen these penalties that are based on perceived rather than ACTUAL DAILY FLOWS.

- Can individual flights request their most optimal route from ZAN before they get to the fixes of NIKIN or ORVIT for east bound flows.

- Recommendation

3.1	The meeting is invited to:

  - Review the information contained in this Working Paper;

  - Discuss options and alternatives to remove or lessen the penalties provided in this Working paper; and

  - Consider whether use be made of REAL TIME On Line Track Advisory / Gateway Reservation List to better anticipate actual conflicts hours before they arrive for westbound traffic.

Figure 1 - Great Circle Route To Chicago from NIKIN and ORVIT

    - If via NIKIN, then forced to fly to COALL before UPR to Chicago

    - If via ORVIT, then forced to fly via OMEKA to UPR to Chicago

COALL

OMEKA

Figure 2 - Great Circle route To YUL (arrival fix for New York) from NIKIN and OVRVIT.

- If via NIKIN – then no penalty.

- If via ORVIT, forced to fly to OMEKA before UPR to East coast.

Figure 3 - Great Circle To SYR (arrival fix for New York) from NIKIN and ORVIT

- If via NIKIN – then no penalty.

- If via ORVIT, forced to fly to OMEKA before UPR to East coast.